| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | An Open-loop MIMO Scheme based on Phase Shift Diversity |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Moon-il Lee Wookbong Lee Bin-Chul Ihm LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-7903 E-mail: emoon1@lge.com wbong@lge.com bcihm@lge.com |
| Re: | IEEE 802.16m-08/016r1 Call for Contributions on Project 802.16m System Description Document (SDD). Specific topic : Downlink MIMO schemes |  |
| Abstract | This contribution proposes an open-loop MIMO scheme based on phase shift diversity (PSD) which can provide considerable diversity gain with lower receiver complexity. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

An Open-loop MIMO Scheme based on Phase Shift Diversity

Moon-il Lee, Wookbong Lee and Bin-Chul Ihm

LG Electronics

Introduction

For better support of a high-mobility MS, it is necessary to employ an open-loop MIMO (OL-MIMO) scheme that can provide enough diversity gain so that reliable high-data rate transmission can be supported in IEEE 802.16m system. In reference system, STBC and rank-2 SM are employed as OL-MIMO schemes. In order to reduce number of options, which is one of IEEE 802.16m requirement, we propose to extend these schemes for different number of antennas.

To compare with existing IEEE 802.16e transmit diversity scheme, we focus on 4Tx with rank-2 transmission in this contribution. But proposed scheme can be extended to any number of antennas.

To employ proper open-loop MIMO scheme considering performance and complexity trade-off, the scheme should be evaluated with sophisticated detector such as maximum likelihood (ML) receiver or low complexity ML-like receiver which can provide reliable detection performance with reasonable decoding complexity.

In [1], the double space-time transmit diversity (D-STTD) with cycling (i.e., 4Tx B matrix) was specified for 4Tx with rank-2 transmission in order to provide full diversity gain. However, it seems hard to employ ML-like receiver for 4Tx B matrix since four data symbols should be jointly detected at the same time even though the 4Tx B matrix is rank-2 transmission scheme, thereby requiring excessive decoding complexity. Keeping that in mind, it is preferable to employ rank-2 based transmission scheme which spreads only two data symbols to spatial domain at a time so that only two data symbols need to be jointly detected at the same time. As a candidate of an open-loop transmission scheme that spreads only two data symbols, we introduce an open-loop precoding scheme based on phase-shift-diversity (PSD) which can be employed with ML-like receiver with reasonable performance gain as compared with 4Tx B matrix.

In this contribution, we compare the performance of proposed scheme and 4Tx B matrix under high mobility scenario. In addition, we also investigate the decoder complexity between 4Tx B matrix with MMSE receiver and proposed open-loop scheme with ML-like receiver.

Proposed Scheme

Following figure 1 shows the open-loop transmission scheme based on phase-shift-diversity (PSD) which can be defined irrespective of the number of transmit antennas and rank.

Figure 1. the structure of proposed scheme.

The proposed scheme can be defined by combining phase-shift diagonal matrix (i.e., phase shift diversity matrix) and fixed unitary matrix and MIMO Encoding as shown in the figure 1. Here MIMO encoder makes data streams based on basic transmission schemes, which are STBC/SFBC for rank-1 transmission and SM for higher rank transmission. The proposed scheme can be easily defined by adapting unitary matrix according to the number of transmit antennas and spatial multiplexing rate. Please note that STBC or SFBC can share MIMO encoder and decoder. It only requires different resource mapping.

- Detailed on proposed scheme

Table 1 shows an exemplary of the proposed schemes for 4Tx according to the rank. A large-delay sample is preferable for phase-shift diagonal matrix for high-mobility MS to obtain high frequency diversity gain when 4Tx common pilots are used. In addition, the unitary matrix can be a Fourier matrix or a Walsh Hadamard (WH) matrix and so on. In the table 1, we use 4x4 WH matrix as an example of the proposed scheme. But the choice of the matrix is FFS.

Table 1. Example of the proposed scheme for 4 Tx according to the rank.

| 4 Transmit Antennas |  |  |  |
| --- | --- | --- | --- |
| Rank 1 | Rank 2 | Rank 3 | Rank 4 |

In table 1,  and  denote resource index or subcarrier index and phase angles for each transmit antenna, respectively. The phase angle  can be defined according to the delay sample  as . The choice of k and is FFS. For rank-1 transmission, x1 and x2 are STBC/SFBC modulated.

Performance Evaluation of the Proposed Scheme

In this section, we evaluate the performance of proposed scheme under ITU Vehicular-A channel with 30Km/h MS speed and compare with 4Tx B matrix. In addition, we also compare the computational complexity between proposed scheme with ML-like receiver and 4Tx B matrix with MMSE receiver. Note that 4Tx B matrix needs to calculate 4x4 MMSE weight including an inversion of 4x4 complex equivalent channel matrix, thereby requiring higher decoding complexity over ML-like receiver for proposed scheme.

- Performance comparison

In this subsection, the performance of proposed scheme under high-mobility scenario will be shown. The simulation assumption is shown in Table 2. In the simulation, we compare the performance between proposed scheme and 4Tx B matrix defined in [1]. In addition, a packet error rate (PER) was used as a proper performance measure. For the proposed scheme, we use large delay value with same pilot overhead to compare only transmit diversity gain.

Table 2. Simulation assumption

| Parameter | Assumption |
| --- | --- |
| System Bandwidth | 10 MHz |
| Subframe length | 0.617 ms |
| Resource unit size | 18 subcarriers * 6 OFDM symbol |
| Resource allocation | 2RU with distributed mode |
| Channel Models | ITU Vehicular-A |
| Mobile Speed (kmph) | 30 |
| Modulation schemes and channel coding rates | QPSK (R= 1/2, 3/4) 16-QAM (R=1/2, 3/4) |
| Channel Code | Turbo code, Component decoder : max-log-MAP |
| MIMO structure | Vertical Encoding (SCW) |
| Delay samples for proposed scheme | 64 |
| Antenna configuration | 4 transmitter, 2 receiver => [4Tx, 2Rx] |
| Spatial correlation (Tx, Rx) | (0%, 0%) |
| MIMO receiver | Linear Minimum Mean Squared Error (MMSE) for B matrix [1] QRM-MLD [2] for proposed scheme |
| Channel Estimation | Perfect channel estimation |

In figure 3, it is seen that the proposed scheme outperforms the 4Tx B matrix irrespective of channel coding rate and modulation. Furthermore, the gain seems to be more significant as the coding rate becomes higher since the diversity gain of proposed scheme can be fully exploited by ML-like receiver.

Figure 3. Performance comparison between proposed scheme and B matrix under ITU Veh-A channel.

- Complexity comparison

In table 3, we show the computational complexity of full ML receiver and ML-like receiver for proposed scheme and MMSE receiver for B matrix. In addition, number of real multiplications assuming rank-2 and one RU case for instance. As shown in the table, ML-like receiver (i.e., QRM-MLD [2]) has much less computational complexity as compared with MMSE receiver for 4Tx B matrix since the 4Tx B matrix needs to decouple four data symbols with one MMSE filtering so that higher order MMSE weight calculation is required in rank-2 transmission. In addition, the complexity reduction seems more considerable as the modulation order gets lower.

Table 3. Number of real multiplications per resource block [2]. (4Tx, 2Rx, Rank(R)=2)

| Decoding Method | Operation | Required number of multiplications per frame | Examples |
| --- | --- | --- | --- |
| Full MLD for proposed scheme | Generation of symbol replica candidates |  | - 4QAM (C=4) : 13824 - 16QAM (C=16) : 138240 - 64QAM (C=64) : 1880064 |
|  | Calculation of squared Euclidian distance |  |  |
| MMSE for B Matrix | MMSE weight generation |  | - 4QAM (C=4) : 74304 - 16QAM (C=16) : 84672 - 64QAM (C=64) : 126144 |
|  | Signal separation using MMSE weight |  |  |
|  | Calculation of squared Euclidian distances |  |  |
| QRM-MLD for proposed scheme | QR decomposition of channel matrix |  | - 4QAM (C=4) : 12960 - 16QAM (C=16) : 43200 - 64QAM (C=64) : 129600 |
|  | Multiplication of to received signal vector |  |  |
|  | Generation of symbol replica candidates |  |  |
|  | Calculation of squared Euclidian distances |  |  |

Pilots for proposed scheme

Usually OL-MIMO scheme are used with distributed resource allocation. In this case, different MS preferred different number of independent data stream. To avoid extremely large pilot overhead, we propose followings for larger number of transmit antenna case, e.g., 4 Tx and 8 Tx or else. The pilot can be precoded with the phase shift diagonal matrix multiplied by first two columns of U matrix. For rank-1 and rank-2 transmission, MS can estimate channel exactly same way as 2Tx case. For rank-3 and rank-4 transmission, MS estimates 4Tx channel from multiple pilots with given parameter  and U.

Conclusions

In this contribution, we introduced the PSD based scheme as a candidate of an open-loop MIMO transmission scheme in order for better support of high mobility MS with high geometry so that the system can provide proper high quality services irrespective of the mobile speed. From the above investigation, we can conclude as follows:

- Proposed scheme can provide significant performance gain over 4Tx B matrix with less decoding complexity

  - ML-like receiver can provide full performance gain with less complexity when two data symbol joint detection is capable.

- The performance difference between two schemes including proposed scheme with ML-type receiver and 4Tx B matrix with MMSE receiver could be more significant if we take channel estimation loss into account since the MMSE receiver is more sensitive to channel estimation incorrectness due to inter-layer interference increment.

- Moreover, the performance difference between two schemes including proposed scheme with ML-type receiver and 4Tx B matrix with MMSE receiver could be more significant if we use small delay value with smaller pilot overhead.

Therefore, we strongly recommend the use of proposed scheme for IEEE 802.16m downlink open-loop MIMO scheme since it can obtain full frequency diversity gain in combination with ML-type receiver with lower decoding complexity.

Text Proposal

---------------------------------------------------  Text Start  ---------------------------------------------------

11.x Multiple antenna transmission

11.x.1 MIMO structure

11.x.2 Downlink

11.x.2.1 Open-loop MIMO

Open loop MIMO (OL-MIMO) scheme requires only CQI. OL-MIMO is suitable for high speed user, control channel or limited uplink feedback situation.

To reduce options or complexity, a single or limited number of transmit diversity (TxD) scheme which can be extended to any number of transmit antennas and any number of streams is preferred. To implement simpler receiver, the number of simultaneously transmit symbols is same as transmit rank except for rank-1 transmission.

Figure x. the structure of open-loop MIMO scheme.

Where  and  denote resource index or subcarrier index and phase angles for each transmit antenna, respectively. The phase angle  can be defined according to the delay sample  as . The value of k and  is FFS. Here MIMO encoder makes data streams based on basic transmission schemes, which are STBC/SFBC for rank-1 transmission and SM for higher rank transmission. R is the number of simultaneously transmitted symbols.

For distributed resource allocation, pilots are precoded with rank-2 precoder which is phase shift diagonal matrix multiplied by first two columns of U matrix. For rank-1 and rank-2 transmission, MS can estimate channel exactly same way as 2Tx antenna case. For rank-3 and rank-4 transmission, MS estimates 4 virtual channels from multiple pilots with given parameter  and U.

11.x.2.2 Closed-loop MIMO

11.x.2.3 Collaborative MIMO

---------------------------------------------------  Text End  ---------------------------------------------------

References

[1] IEEE 802.16Rev2/D4, April 2008

[2] Kenichi Higuchi et. al. ‘Adaptive selection of surviving symbol replica candidates based on maximum reliability in QRM-MLD for OFCDM MIMO multiplexing,’ Globecom 2004