| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Supporting long CP size in the frame structure |  |
| Date Submitted | 2008-5-05 |  |
| Source(s) | Hongwei YANG, Dong LI, Liyu CAI, Keying WU, Xiaolong ZHU, Liu Jianhua Alcatel Shanghai Bell Co., Ltd. Xiaolu Dong, Ying Du CATR Fang Xie CMCC Yinggang Du, Yingming Wang Huawei Hongyun Qu, Huiying Fang ZTE Hangfeng Zhang, Zhuo Gao Datang Mobile | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 mailto: hongwei.yang@alcatel-sbell.com.cn mailto: dongxiaolu, duying@mail.ritt.com.cn mailto: xiefang@chinamobile.com mailto: duyinggang, wangyingming@huawei.com mailto: hongyun.qu, huiying.fang@zte.com zhanghanfeng, gaozhuo@datangmobile.com |
| Re: | IEEE 802.16m-08/016r1 Call for Contributions on Project 802.16m System Description Document (SDD) Target topic: Comments to IEEE 802.16m-08/003r1 |  |
| Abstract | This contribution proposes to support long CP size in the baseline frame structure so that it can support multicast-broadcast services effectively. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Supporting long CP size in the frame Structure

Hongwei YANG, Dong LI, Liyu CAI, Keying WU, Xiaolong ZHU

Alcatel Shanghai Bell Co., Ltd.

Xiaolu Dong, Ying Du

CATR

Fang Xie

CMCC

Yinggang Du, Yingming Wang

Huawei

Hongyun Qu, Huiying Fang

ZTE

Hangfeng Zhang, Zhuo Gaos

Datang Mobile

# 1. Introduction

In the frame structure baseline content in the latest SDD (802.16m-08/003r1), two different CP sizes of 1/8 and 1/16 Tu are supported, where Tu is the OFDMA symbol duration without CP. However, they are not sufficient for some services like multicast-broadcast. This contribution proposes to support a long CP size in the baseline frame structure so that it can provide better support for multicast-broadcast services (MBS).

# 2. Requirement for long CP size

Single frequency network (SFN) is very important for MBS to achieve high spectral efficiency and good coverage. In SFN, all BSs transmit the same signal with the same physical layer processing at the same radio resource. This results in increased delay spread confronted by terminal. In order to quantize the delay spread of equivalent channel in SFN, we simulate the CDF of mean delay spread, RMS delay spread and max delay spread of SFN equivalent channels based on typical outdoor channel models for unicast services in 16m EVM document with different inter-BS distances, and then derive those mean values as shown in Table 1. Effective power threshold is assumed as -30dB, and the other simulation parameters are appended in this contribution. From the table we can see that the max delay spread for MBS is often much larger than that for unicast service. To avoid inter-symbol-interference, CP size for MBS should be longer than unicast.

Table 1 Parameters of SFN equivalent channel

| Channel Model for unicast | Inter-BS Distance (m) | Mean Delay Spread (us) | RMS Delay Spread(us) | Max. Delay Spread (us) |
| --- | --- | --- | --- | --- |
| ITU Pedestrian B | 500 | 0.89 | 0.78 | 4.52 |
|  | 1500 | 1.54 | 1.56 | 9.50 |
|  | 3000 | 2.51 | 2.91 | 17.66 |
|  | 5000 | 3.81 | 4.76 | 18.67 |
| ITU Vehicular A | 500 | 0.72 | 0.62 | 3.76 |
|  | 1500 | 1.40 | 1.49 | 9.02 |
|  | 3000 | 2.41 | 2.90 | 17.31 |
|  | 5000 | 3.76 | 4.80 | 28.39 |
| Urban Macro Cell | 500 | 0.71 | 0.58 | 3.65 |
|  | 1500 | 1.32 | 1.43 | 9.01 |
|  | 3000 | 2.22 | 2.77 | 17.23 |
|  | 5000 | 3.43 | 4.56 | 28.25 |
| Bad Urban Macro Cell | 500 | 1.30 | 1.65 | 8.81 |
|  | 1500 | 1.92 | 2.16 | 12.75 |
|  | 3000 | 2.85 | 3.28 | 20.0 |
|  | 5000 | 4.07 | 4.96 | 30.67 |
| Suburban Macro Cell | 500 | 0.49 | 0.48 | 3.01 |
|  | 1500 | 1.12 | 1.40 | 8.41 |
|  | 3000 | 2.08 | 2.79 | 16.57 |
|  | 5000 | 3.35 | 4.65 | 27.46 |
| Urban Micro Cell-NLOS | 500 | 0.53 | 0.52 | 3.04 |
|  | 1500 | 1.29 | 1.51 | 8.63 |
|  | 3000 | 2.41 | 3.01 | 17.03 |
|  | 5000 | 3.94 | 5.01 | 28.23 |
| Bad Urban Micro Cell | 500 | 1.04 | 1.00 | 5.04 |
|  | 1500 | 1.66 | 1.69 | 10.04 |
|  | 3000 | 2.59 | 2.96 | 18.07 |
|  | 5000 | 3.81 | 4.74 | 28.90 |
| Rural Macro Cell | 500 | 1.10 | 0.65 | 3.14 |
|  | 1500 | 2.97 | 1.88 | 8.85 |
|  | 3000 | 6.04 | 3.85 | 17.75 |
|  | 5000 | 10.25 | 6.50 | 30.01 |

4. Tex proposal

# -----------------------------------------Start of Text Proposal for SDD------------------------------------------

[Insert a sentence into the end of Section 11.3]

To support multicast-broadcast services, longer CP size than the normal one shall be supported. Numerologies for the longer CP size are FFS.

---------------------------------------------- End of Text Proposal for SDD---------------------------------------------

# 5. Appendix

The simulation parameters used in this contribution is listed in the following table:

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| Carrier frequency | 2.5 GHz | Path Loss Model | Loss (dB) = 130.62+37.6log10(R) (R in km) ** |
| Total bandwidth | 10 MHz | Lognormal Shadowing Standard Deviation | 8 dB |
| Number of points in full FFT | 1024 | Correlation Distance for Shadowing | 50m |
| Sampling frequency | 11.2 MHz | Mobility | 0-120 km/hr |
| OFDMA symbol duration without CP | 91.43 us | Transmit antenna Gain | 17 dBi |
| Frame length | 5 ms | Number of sectors per cell | 3 |
| Site-to-Site Distance | 500/1500/3000/5000m | Number of cells | 19 |
| Carrier Frequency | 2.5 GHz | Front-to-back power ratio | 20 dB |
| BS Tx Power per sector | 46 dBm | Noise figure | 5 dB |
| Penetration Loss | 10 dB | Cable loss | 2 dB |