| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame structure supporting multiple CP sizes |  |
| Date Submitted | 2008-5-05 |  |
| Source(s) | Hongwei YANG, Dong LI, Liyu CAI, Keying WU, Xiaolong ZHU Alcatel Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 mail to: Hongwei.Yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/016r1 Call for Contributions on Project 802.16m System Description Document (SDD) Target topic: Comments to IEEE 802.16m-08/003r1 |  |
| Abstract | This contribution proposes to support multiple CP sizes in the baseline frame structure so that it can adapt to the environments as well as multicast-broadcast services effectively. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frame structure supporting multiple CP sizes

Hongwei YANG, Dong LI, Liyu CAI, Keying WU, Xiaolong ZHU

Alcatel Shanghai Bell Co., Ltd.

# 1. Introduction

In the frame structure baseline content in the latest SDD (802.16m-08/003r1), two different CP sizes of 1/8 and 1/16 Tu are supported, where Tu is the OFDMA symbol duration without CP. However, this frame structure has several open issues:

- These two CP sizes are not sufficient for some environments with large delay spread and especially for some services like multicast-broadcast.

- The CP size can only change frame by frame rather than sub-frame by sub-frame, which highly restricts the flexibility of radio resource scheduling.

- The OFDM parameter definition for CP size of 1/16 Tu is not complete. Given the overall OFDM symbols per frame, how to allocate the defined OFDM symbols per sub-frames needs further definition.

This contribution proposes to support another long CP size in the baseline frame structure so that it can adapt to the environments efficiently as well as provide better support for multicast-broadcast services (MBS). Correspondingly, OFDM parameters are defined for long CP, and completed for short CP.

# 2. Requirements for long CP size

Single frequency network (SFN) is very important for MBS to achieve high spectral efficiency and good coverage. In SFN, all BSs transmit the same signal with the same physical layer processing at the same radio resource. This results in increased delay spread confronted by terminal. In order to model the delay spread of equivalent channel in SFN, we simulate the CDF of mean delay spread, RMS delay spread and max delay spread of equivalent channels for typical outdoor channel models in 16m EVM document with different inter-BS distances, and then derive those mean values as shown in Table 1. Effective power threshold is assumed as -30dB, and the other simulation parameters are appended in this contribution. From the table we can see that the delay spread for MBS is much larger than that for unicast service. To avoid inter-symbol-interference, CP size for MBS should be longer than unicast.

Table 1 Parameters of equivalent channel in SFN

| Channel Model | Inter-BS Distance (m) | Mean Delay Spread (us) | RMS Delay Spread(us) | Max. Delay Spread (us) |
| --- | --- | --- | --- | --- |
| ITU Pedestrian B | 500 | 0.89 | 0.78 | 4.52 |
|  | 1500 | 1.54 | 1.56 | 9.50 |
|  | 3000 | 2.51 | 2.91 | 17.66 |
|  | 5000 | 3.81 | 4.76 | 18.67 |
| ITU Vehicular A | 500 | 0.72 | 0.62 | 3.76 |
|  | 1500 | 1.40 | 1.49 | 9.02 |
|  | 3000 | 2.41 | 2.90 | 17.31 |
|  | 5000 | 3.76 | 4.80 | 28.39 |
| Urban Macro Cell | 500 | 0.71 | 0.58 | 3.65 |
|  | 1500 | 1.32 | 1.43 | 9.01 |
|  | 3000 | 2.22 | 2.77 | 17.23 |
|  | 5000 | 3.43 | 4.56 | 28.25 |
| Bad Urban Macro Cell | 500 | 1.30 | 1.65 | 8.81 |
|  | 1500 | 1.92 | 2.16 | 12.75 |
|  | 3000 | 2.85 | 3.28 | 20.0 |
|  | 5000 | 4.07 | 4.96 | 30.67 |
| Suburban Macro Cell | 500 | 0.49 | 0.48 | 3.01 |
|  | 1500 | 1.12 | 1.40 | 8.41 |
|  | 3000 | 2.08 | 2.79 | 16.57 |
|  | 5000 | 3.35 | 4.65 | 27.46 |
| Urban Micro Cell-NLOS | 500 | 0.53 | 0.52 | 3.04 |
|  | 1500 | 1.29 | 1.51 | 8.63 |
|  | 3000 | 2.41 | 3.01 | 17.03 |
|  | 5000 | 3.94 | 5.01 | 28.23 |
| Bad Urban Micro Cell | 500 | 1.04 | 1.00 | 5.04 |
|  | 1500 | 1.66 | 1.69 | 10.04 |
|  | 3000 | 2.59 | 2.96 | 18.07 |
|  | 5000 | 3.81 | 4.74 | 28.90 |
| Rural Macro Cell | 500 | 1.10 | 0.65 | 3.14 |
|  | 1500 | 2.97 | 1.88 | 8.85 |
|  | 3000 | 6.04 | 3.85 | 17.75 |
|  | 5000 | 10.25 | 6.50 | 30.01 |

Multi-BS MIMO technique has been proposed during last several 16m sessions to perform joint MIMO transmission and reception between multiple coordinated BSs and multiple MSs over the same radio resources. This also results in similar increased delay spread at terminal to SFN, and demands longer CP size than 1/8 Tu. In this contribution, we suggest a long CP to be adopted in 16m system to adapt environments and services. That is, three types of CP modes are defined: long CP, normal CP and short CP.

# 3. OFDM parameter design for IEEE802.16m

# Given a CP length, there are two different ways to design a frame. One is based on an unfixed-length sub-frame, and the other on a fixed-length sub-frame. In the former case, the frame duration is fixed and thus different CP size leads to a different number of OFDM symbols per frame. In the latter one, both the frame duration and sub-frame duration is fixed and thus different CP size results in a different number of OFDM symbols per sub-frame. Which one is better needs careful consideration.

The major drawback of unfixed-length sub-frame based design is that the CP size can only change from frame to frame. When simplifying the control signaling overhead, it significantly reduces the flexibility to schedule users with different CP sizes in a frame. Moreover, since the number of OFDM symbols in a sub-frame may be different, pilot pattern, sub-carrier permutation, and HARQ timing should be specifically designed. Comparatively, fixed-length sub-frame based design supports CP size to change from sub-frame to sub-frame. At the same time, half an OFDM symbol which is typically located at the end of the subframe is often used to align the time duration of sub-frame for different CP size. Modulation of half an OFDM symbol can be implemented easily by only using the even (or odd) numbered sub-carriers and removing one half of the time waveform after IDFT modulation at the transmitter, and therefore not cost additional complexity for both transmitter and receiver.

# In this contribution, OFDM parameters are proposed based on fixed-length sub-frame. The subframe structure for different CP sizes is shown in Figure 1.

# The normal CP mode is the default one. In this CP configuration, there are totally 6 OFDM symbols in each subframe, each with a CP length of 1/8-Tu. For the long CP configuration, a subframe contains five and a half OFDM symbols. For the short CP mode, each subframe contains six and a half OFDM symbols.

Table 2 suggests the OFDM parameters for various CP sizes. Note that the CP length in Table 2 is in unit of samples and a sampling frequency of 11.2MHz for bandwidth of 10MHz is assumed. These values can be easily extended to the other bandwidths such as 5MHz and 20MHz.

Figure 1: Proposed subframe structure for different CP sizes

The following principles may be used as guidelines for the application of the sub-frame structure:

- The CP length configuration can be performed sub-frame by sub-frame. That is, the sub-frames within one frame or super-frame can be configured differently.

- The configuration information can be conveyed in the frame header or superframe header.

- The first sub-frame in each super-frame is configured to normal CP mode.

- The subframe structure can be applied to both the FDD and TDD modes.

- In TDD mode, for the subframe immediately before the DL/UL or UL/DL switching point, some resource will be reserved and used as RTG/TTG as follows

  - If the subframe is in normal CP mode, the last OFDM is reserved;

  - If the subframe is in short CP mode or long CP mode, the last symbol (i.e., the half OFDM symbol) and half of the second last OFDM symbol will be reserved. For example, the samples after the first half of S5 will be used as the gap in short CP case.

Table 2: OFDM parameter definitions for the proposed CP sizes

| Nominal Channel Bandwidth (MHz) | 5 |  | 7 |  |  | 8.75 |  | 10 |  |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor | 28/25 |  | 8/7 |  |  | 8/7 |  | 28/25 |  |  | 28/25 |  |
| Sampling Frequency (MHz) | 5.6 |  | 8 |  |  | 10 |  | 11.2 |  |  | 22.4 |  |
| FFT Size | 512 |  | 1024 |  |  | 1024 |  | 1024 |  |  | 2048 |  |
| Sub-Carrier Spacing(kHz) | 10.94 |  | 7.81 |  |  | 9.77 |  | 10.94 |  |  | 10.94 |  |
| Tu ((s) | 91.4 |  | 128 |  |  | 102.4 |  | 91.4 |  |  | 91.4 |  |
| Cyclic Prefix (CP) | Number of samples per each OFDM symbol per sub-frame[1] |  |  |  |  |  |  |  |  |  |  | Idle Time(s) |
|  | 0 | 1 |  | 2 | 3 |  | 4 |  | 5 | 6 |  |  |
| Normal CP | 128/1024[2] | 128/1024 |  | 128/1024 | 128/1024 |  | 128/1024 |  | 128/1024 | N/A |  | 62.86 |
| Short CP | 36/1024 | 36/1024 |  | 36/1024 | 36/1024 |  | 36/1024 |  | 36/1024 | 40/512 |  | 62.86 |
| Long CP | 214/1024 | 214/1024 |  | 214/1024 | 214/1024 |  | 214/1024 |  | 210/512 | N/A |  | 62.86 |

Note:

- These values are for sampling frequency of 11.2MHz for 10MHz BW and can be extended straightforwardly to BWs of 5MHz and 20MHz.

- A/B: A denotes the CP length and B denotes the useful OFDM symbol length, both in samples.

4. Tex proposal

# -----------------------------------------Start of Text Proposal for SDD------------------------------------------

[Replace Table 11.3-1 with the following table and corresponding notes]

| Nominal Channel Bandwidth (MHz) | 5 |  | 7 |  |  | 8.75 |  | 10 |  |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor | 28/25 |  | 8/7 |  |  | 8/7 |  | 28/25 |  |  | 28/25 |  |
| Sampling Frequency (MHz) | 5.6 |  | 8 |  |  | 10 |  | 11.2 |  |  | 22.4 |  |
| FFT Size | 512 |  | 1024 |  |  | 1024 |  | 1024 |  |  | 2048 |  |
| Sub-Carrier Spacing(kHz) | 10.94 |  | 7.81 |  |  | 9.77 |  | 10.94 |  |  | 10.94 |  |
| Tu ((s) | 91.4 |  | 128 |  |  | 102.4 |  | 91.4 |  |  | 91.4 |  |
| Cyclic Prefix (CP) | Number of samples per each OFDM symbol per sub-frame[1] |  |  |  |  |  |  |  |  |  |  | Idle Time(s) |
|  | 0 | 1 |  | 2 | 3 |  | 4 |  | 5 | 6 |  |  |
| Normal CP | 128/1024[2] | 128/1024 |  | 128/1024 | 128/1024 |  | 128/1024 |  | 128/1024 | N/A |  | 62.86 |
| Short CP | 36/1024 | 36/1024 |  | 36/1024 | 36/1024 |  | 36/1024 |  | 36/1024 | 40/512 |  | 62.86 |
| Long CP | 214/1024 | 214/1024 |  | 214/1024 | 214/1024 |  | 214/1024 |  | 210/512 | N/A |  | 62.86 |

Note:

- These values are for sampling frequency of 11.2MHz for 10MHz BW and can be extended straightforwardly to BWs of 5MHz and 20MHz.

- A/B: A denotes the CP length and B denotes the useful OFDM symbol length, both in samples.

[Replace Figure 11.4-2 with the following figure and corresponding text]

Figure 11.4-2: Regular and irregular sub-frames in TDD duplex scheme

The normal CP mode is the default one. In this CP configuration, there are totally 6 OFDM symbols in each subframe, each with a CP length of 1/8-Tu. For the long CP configuration, a subframe contains five and a half OFDM symbols. For the short CP mode, each subframe contains six and a half OFDM symbols.

---------------------------------------------- End of Text Proposal for SDD---------------------------------------------

# 5. Appendix

The simulation parameters used in this contribution is listed in the following table:

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| Carrier frequency | 2.5 GHz | Path Loss Model | Loss (dB) = 130.62+37.6log10(R) (R in km) ** |
| Total bandwidth | 10 MHz | Lognormal Shadowing Standard Deviation | 8 dB |
| Number of points in full FFT | 1024 | Correlation Distance for Shadowing | 50m |
| Sampling frequency | 11.2 MHz | Mobility | 0-120 km/hr |
| OFDMA symbol duration without CP | 91.43 us | Transmit antenna Gain | 17 dBi |
| Frame length | 5 ms | Number of sectors per cell | 3 |
| Site-to-Site Distance | 500/1500/3000/5000m | Number of cells | 19 |
| Carrier Frequency | 2.5 GHz | Front-to-back power ratio | 20 dB |
| BS Tx Power per sector | 46 dBm | Noise figure | 5 dB |
| Penetration Loss | 10 dB | Cable loss | 2 dB |