| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of legacy terminals in a 16m TDD system with four switching points |  |
| Date Submitted | 2008-04-05 |  |
| Source(s) | Per Ernström, Per-Erik Östling Ericsson AB SE-164 80 Stockholm, Sweden | E-mail: Per.Ernstrom@ericsson.com, Per-Erik.Ostling@ericsson.com |
| Re: | The call for contributions on “The entire content of IEEE 802.16m-08/003r1” (Frame structure) |  |
| Abstract | Multiple sections of the documents contain text proposals in support of comment submitted separately on the proposed frame structure baseline content in document C802.16m-08/118r1. |  |
| Purpose | Propose text changes to the SDD in support of a comment to IEEE 802.16m-08/003r1 (SDD) submitted separately |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support of legacy terminals in a 16m TDD system with four switching points

Per Ernstrom, Per-Erik Östling

Ericsson AB

If a 16m system with four switching points per radio frame is deployed on the same carrier frequency and in the same geographical area as a legacy system, this would create severe interference problems (unless some special arrangement is done such as blanking out parts of the legacy frames). A new 16m system with four switching points per radio frame should therefore be deployed with sufficient separation either geographically or in frequency from legacy systems. Similarly, when a legacy system is upgraded to a 16m system with four switching points per radio frame, all base stations need to be upgraded at the same time.

A 16m system with four switching points can, however, support legacy terminals. This has been noted in a number of contributions, e.g. in C802.16m-07/242r1, C80216m-07_263, C80216m-07/215, C80216m-07_235, and C80216m-08_095r1.

This was also noted in the Orlando meeting in comment 62 in the commentary database C80216m-08_247 and in the accompanying contribution C802.16m-08/245. The comment was approved and the sentence "There shall only be two switching points in each TDD radio frame when supporting legacy systems" was modified to "In the case of coexistence with legacy systems, two switching points may be selected in each TDD radio frame." as proposed in the comment.

In the current SDD it is, however, not illustrated how a legacy terminal is supported in a 16m system with four switching points. Here we propose to include a figure and associated text to do that.

----------TEXT PROPOSAL for section 11.4.4.1  in IEEE 802.16m-08/003r1-----------

11.4.4.1 Time Zones in TDD

As was mentioned earlier, the concept of time zones applies to TDD mode (see Figure 13 and Figure X), as well. The following constraints apply:

In a mixed deployment, the allocation of time zones in the TDD duplex mode shall be as shown in Figure 13 and Figure X for the two and four switching point case respectively.

The duration of the zones may vary. Every frame shall start with a preamble and the MAP followed by legacy

DL zone since legacy terminals/relays expect 802.16e zones in this region. In the case of coexistence, the UL portion

shall start with legacy UL zone since legacy BS/terminals/relays expect 802.16e UL control information be sent in this region. In a green-field deployment, the legacy zones can be removed.

In a deployment where a legacy BS and a new BS co-exist on the same frequency band and on the same or neighboring geographical areas the four switching point case should not be used.

Switching points should be synchronized across network to reduce inter-cell interference. The switching points would require use of idle symbols to accommodate the gaps. In case of TDD operation with the generic frame structure, the last symbol in the slot immediately preceding a downlink-to-uplink/uplink-to-downlink switching point may be reserved for guard time and consequently not transmitted.

SF

#0

DL

SF

#1

DL

SF

#2

DL

SF

#3

UL

SF

#4

DL

SF

#5

DL

SF

#6

UL

SF

#7

UL

SF

#0

DL

SF

#1

DL

SF

#2

DL

SF

#3

UL

SF

#4

DL

SF

#5

DL

SF

#6

UL

SF

#7

UL

5ms Radio Frame

Legacy DL Zone

New DL Zone

Legacy UL Zone

New UL Zone

Switching Point

Figure X  Time zones in a TDD system with four switching points per radio frame.

-----------------------------END OF TEXT PROPOSAL------------------