| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CQI Feedback Scheme for MCW MIMO |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Yinggang Du, Hongjie Si, Zhigang Rong, Jianmin Lu Huawei Jianhua Liu Alcatel Shanghai Bell | E-mail: duyinggang@huawei.com jianhua.liu@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-08/016 “Call for Contributions on Project 802.16m System Description Document (SDD)” Target topic: “Downlink MIMO Scheme” |  |
| Abstract | This contribution provides an enhanced CQI feedback scheme for MCW MIMO in order to reducing signaling overhead. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CQI Feedback Scheme for MCW MIMO

Yinggang Du, Hongjie Si, Zhigang Rong, Jianmin Lu

Huawei

Jianhua Liu

Alcatel Shanghai Bell

# Introduction

In current system, a five-bit CQI for SISO transmission is signalled to the Base Station for MCS selection. If multiple data streams are transmitted simultaneously in a MIMO system, the required CQI bits increase exponentially as the number of the data streams. If full CQI feedback is used, it will lead to large feedback overhead.

Since CQI feedback is strongly correlated with the performance of MIMO, particularly for the case of four Tx antenna, it is necessary that the feedback procedure and overhead should be clearly described.

In this contribution we propose an efficient CQI feedback scheme with rather low overhead where effective CQI table size is not reduced.

# Description

In 802.16 system, CQI feedback can be updated every 5ms which is short enough compared to the channel variation. This especially meets the MIMO scenarios.

In [1], a CQI-SNR mapping scheme was proposed and its corresponding SNR in an AWGN SISO channel satisfies SNR= -3.5+CQI (dB), where the modulation and code rate of all streams are fed back from UE with the CQI index in the first column of Table 1. Hence each stream is fed back with 5 bits CQI. It should be notified that Table 1 is just an example of CQI mapping. The feedback principle in this contribution can be extended to any CQI mapping rules.

| CQI | TBS+CRC | Modulation | Code Rate |
| --- | --- | --- | --- |
| 0 | N/A |  |  |
| 1 | 200 | QPSK | 0.21 |
| 2 | 260 | QPSK | 0.27 |
| 3 | 340 | QPSK | 0.35 |
| 4 | 400 | QPSK | 0.42 |
| 5 | 480 | QPSK | 0.50 |
| 6 | 680 | QPSK | 0.35 |
| 7 | 820 | QPSK | 0.43 |
| 8 | 960 | QPSK | 0.50 |
| 9 | 1290 | QPSK | 0.45 |
| 10 | 1520 | QPSK | 0.53 |
| 11 | 1780 | QPSK | 0.62 |
| 12 | 2300 | QPSK | 0.60 |
| 13 | 2610 | QPSK | 0.68 |
| 14 | 3330 | QPSK | 0.69 |
| 15 | 3590 | 16-QAM | 0.37 |
| 16 | 4200 | 16-QAM | 0.44 |
| 17 | 4700 | 16-QAM | 0.49 |
| 18 | 5300 | 16-QAM | 0.55 |
| 19 | 5910 | 16-QAM | 0.62 |
| 20 | 6600 | 16-QAM | 0.69 |
| 21 | 7200 | 16-QAM | 0.75 |
| 22 | 9750 | 16-QAM | 0.73 |
| 23 | 11500 | 16-QAM | 0.75 |
| 24 | 14400 | 16-QAM | 0.75 |
| 25 | 17300 | 16-QAM | 0.75 |
| 26 | 21600 | 16-QAM | 0.75 |
| 27~31 | RSVD |  |  |

Table 1: Index of CQI mapping scheme

Then in a MIMO scenario, there are totally 5*Nstream CQI bits if each stream is individually rate controlled, which is a high overhead burden.

In this proposal, each CQI feedback is composed of 5+(Nstream-1) bits: 1) The 5 bits (0~31) in the first part represent the possible transport block size, modulation and code rate for a certain stream as defined in the second, third, forth and fifth column in Table 1 respectively. A CQI_index(i, j) at ith stream and jth TTI comes from the mapping scheme CQI_index(i,j) = floor(SINR(i,j) + 3.5) where SINR(i,j) is measured at the detector output for ith stream and jth TTI  and floor(X) gives an integer no larger than X. It is easy to understand that CQI_index(i,j) satisfies 0 ≤ CQI_index(i,j) ≤ 26. 2) The (Nstream-1) bits in the second part represent Up/Down signaling of other streams. Table 2 shows an example for the scenario of 4 transmit streams (Nstream=4) from TTI j to TTI (j+3) where j satisfy mod(j,4)＝0. The first part and second part are transmitted via separated uplink channels, i.e. FCQICH (Full CQI Channel) and RCQICH (Relative CQI Channel) respectively. The ACQICH can easily re-use existing feedback.

| Bit | M bits | M+1 | M+2 | M+3 |
| --- | --- | --- | --- | --- |
| Part | First part (FCQICH): CQI index | Second Part (RCQICH): Up/Down signaling for CQI index |  |  |
| TTI j | 0th stream | 1st stream | 2nd stream | 3rd stream |
| TTI j+1 | 1st stream | 0th stream | 2nd stream | 3rd stream |
| TTI j+2 | 2nd stream | 0th stream | 1st stream | 3rd stream |
| TTI j+3 | 3rd stream | 0th stream | 1st stream | 2nd stream |

Table 2: Structure of feedback for consecutive four TTI

The Up/Down signaling transmitted in RCQICH for ith transmit stream at jth TTI is defined as follows:

If CQI_index(i, j-1) > CQI_index (i, j)

Up/Down Signalling = -1 (Down)

Elseif CQI_index(i, j-1) < CQI_index (i, j)

Up/Down Signalling = 1  (Up)

Else

Up/Down Signalling = 0  (Keep same)

For a single-bit representation, the Up/Down Signalling can be transmitted with DTX transmission to represent the three state: Up, Down or Keep same. Up/Down Signalling with larger step may be considered depending on the tradeoff between tracking channel variation and reducing feedback overhead.

In the transmitter, we propose a simple modulation and rate control scheme that adjust the stream throughput solely based on the above feedback sequence. However, more information, such as uplink channel estimation and system load, can be employed in the modulation and code rate decision in real implementation. For ith stream, the transmitter receives the CQI_index(i,j) at jth TTI in ACQICH, where j satisfies mod(j, Nstream)=i. In the next TTI (j+1) to TTI (j+Nstream-1), an Up/Down signaling for ith stream is received in RCQICH and the transmitter can correspondingly adjust the modulation and code rate of ith  stream as CQI_index(i, j) = CQI_index(i, j-1)+Up/Down Signalling. This process repeats until an updated CQI_index(i, j+Nstream) is received in ACQICH at TTI (j+ Nstream).

# Simulation

The simulations are run for the scenarios with ideal full feedback without delay (5 bits each stream), delayed full feedback (5 bits each stream) and the proposed differential feedback with 5+(Nstream-1) bits. The simulation assumptions are given in Table 3. In the simulation, no HARQ is considered, so if the feedback CQI index of a stream is larger than the present real CQI index corresponding to present SINR, the throughput of this stream for this TTI will be directly calculated as 0.

| Carrier Frequency | 2 GHz |
| --- | --- |
| TTI length | 10 ms |
| Modulation | QPSK, 16-QAM |
| DPCH/Ior | 0 dB |
| Channel Model | Single path Rayleigh channel |
| Channel correlation | i.i.d. |
| Channel Estimation | Perfect |
| MIMO scheme | 2×2 MIMO and 4×4 MIMO |
| UE Speed | 3 km/h and 30 km/h |
| CQI Feedback Delay | Two TTI |
| CQI Feedback error | None |
| Target FER | 10% |
| Output metric | Single user throughput based on the TBS |

Table 3: Simulation assumptions

Figure 1 and Figure 2 give the throughput with the 2×2 MIMO scheme. It shows that the proposed differential feedback (6 bits feedback) obtains the almost same throughput with the full feedback (10 bits feedback) scheme at any mobile speed. The throughput gap is negligible. It is easy to understand that both the differential feedback scheme and the delayed full feedback scheme suffer some throughput loss from the ideal full feedback without delay. For UE with very low speed (3km/h), the throughput gap from the ideal feedback scheme is smaller compared with the one with high speed (30km/h).

Figure 1: Throughput of a 2×2 scenario at 3km/h    Figure 2: Throughput of a 2×2 scenario at 30km/h

Figure 3 and Figure 4 give the capacity with the 4×4 MIMO scheme, which gives the similar conclusion as the 2×2 scenario. The proposed differential feedback (8 bits feedback) obtains the almost same throughput with the full feedback (20 bits feedback) scheme at any mobile speed. The throughput gap is negligible.

Figure 3: Throughput of a 4×4 scenario at 3km/h     Figure 4: Throughput of a 4×4 scenario at 30km/h

# Conclusion

The simulation results show that the proposed differential feedback scheme performs almost same as the full feedback scheme in low and media mobile speeds. We recommend WG to adopt this scheme.

# References

[1]   R4-020612, Revised HSDPA CQI Proposal, Motorola and Nokia, 2002-04

# Text proposal to SDD

------------------------------------Start text proposal------------------------------------

[Adopt the following text in the ToC of P802.16m System Description Document (SDD)]

11. x.x.x MCW

In MCW mode, the differential CQI feedback for each layer should be implemented. All layers feedback the full CQI in a round-robin manner. When one layer feedback the full CQI, other layers transmit the differential CQI from its last feedback to BS. The differential CQI information can be presented using one or more bits.

------------------------------------End text proposal------------------------------------