| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Coexistence Supports with Other TDD Systems in Frame Structure |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Dongcheol Kim, Sungho Moon, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1852 E-mail: dongcheol.kim@lge.com msungho@lge.com samji@lge.com |
| Re: | IEEE 802.16m-08/016r1 “Call for Contributions on P802.16m SDD” - IEEE 802.16m-08/003r1 |  |
| Abstract | The contribution describes many consideration points and proposes some candidates for IEEE 802.16m frame structure based on the document C802.16m-08/003r1. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Coexistence Supports with Other TDD Systems in Frame Structure

DongCheol Kim, Sungho Moon, and Jin Sam Kwak ,

LG Electronics

# Introduction

The IEEE 802.16m system shall support co-deployment scenarios with other TDD systems, e.g., E-UTRA TDD and UTRA LCR-TDD systems in the same or adjacent frequency band as mentioned in the IEEE 802.16m System Requirement Document (SRD) [1]. Therefore, the inter-system DL and UL alignment should be considered to prevent sever interference at the cell edge. However, E-UTRA TDD and UTRA LCR-TDD system has 10ms frame structure and in addition, E-UTRA TDD system has configurations with a switch-point periodicity of 10 ms, which can cause large number of symbol or sub-frame puncturing for IEEE 802.16m system to avoid the interference from DL/UL mis-alignment. In this contribution, we consider coexistence scenarios between the IEEE802.16m TDD frames and other TDD frames, and suggest asymmetric configuration and UL-alignment scheme for IEEE 802.16m to reduce the number of punctured symbols in all possible DL and UL configurations of E-UTRA TDD and UTRA LCR-TDD systems. As a text proposal for the 802.16m System Description Document (SDD), we suggest text and illustrations on document C802.16m-08/003r1, “Draft IEEE 802.16m System Description Document” [2].

# Coexistence with Other TDD Systems

The 802.16m System Requirement Document (SRD) defines co-deployment scenarios with other TDD systems as follows [1]:

- IEEE 802.16m and non 802.16m systems may be deployed in the same licensed band. Adjacent channels may be used for deployment of 802.16m and non-802.16m systems.

- IEEE 802.16m may be deployed in a licensed band adjacent to an unlicensed band in which non 802.16m systems are deployed. Hence additional coexistence mechanisms may be required to reduce interference.

- IEEE 802.16m may be required to share bands:

- on a co-channel basis with other 802.16m systems.

- on a co-channel basis with non 802.16 systems where 802.16m is either the primary or non-primary system

- on a non co-channel basis with non 802.16 systems where 802.16m is either the primary or non-primary system

According to the 16m SRD, we can consider the IEEE802.16m TDD system co-deployed/coexisted in the same or adjacent frequency band of other TDD systems. The inter DL-UL interference can be defined as a interference caused by mis-alignment of DL and UL regions between TDD systems. Therefore, the DL/UL regions of IEEE 802.16m should be aligned with existing TDD systems in order to avoid the inter DL-UL interferences in the coexistence scenario. In order to coexist with a target TDD system which has different DL and UL configurations, the IEEE 802.16m TDD frame should be delayed or offset by a proper amount of time in transmission, and these offset values can be varied according to DL to UL configurations of the target TDD system.

In general, we can consider two kinds of coexistence methods categorized by aligned points between two TDD systems. Fig. 1 shows an example of these two methods for coexisting with LTE TDD [3]. The first scheme can be called DL-aligned scheme in which the IEEE 802.16m TDD frame is aligned to the start point of consecutive DL sub-frames in LTE. In the second scheme, the IEEE 802.16m UL frame is aligned to the start of UpPTS or GP in LTE (or the start of consecutive UL region = UpPTS). If we assume that the reference time of IEEE 802.16m frame is the start point of consecutive DL sub-frames, the DL-aligned or UL-aligned offset can be defined as a delay of the IEEE 802.16m reference time from the LTE reference time. In both schemes, some DL and UL symbols can be punctured due to mis-alignment between DL and UL regions because sub-frame sizes and DL/UL configuration periods are different between the IEEE 802.16m and LTE TDD systems. The punctured symbols should be minimized to maintain spectral efficiency in IEEE 802.16m. Thus, the DL and UL-aligned offset can be additionally shifted within RTG and/or TTG in the IEEE 802.16m system.

Fig. 1 Alignment examples of IEEE 802.16m and LTE TDD frames

# Frame Alignment with E-UTRA TDD (LTE)

The LTE specification defines a 10 ms radio frame as two half-frames with a length of 5 ms, and each half-frame includes 5 sub-frames with a length of 1 ms. Within a frame, there are one or two special sub-frames consisting of DwPTS, GP and UpPTS. In the view point of IEEE 802.16m TDD frame, the UL/DL configuration of LTE TDD can be categorized into two cases by switch-point periodicity. Table 1 shows the downlink-uplink allocations in LTE TDD where “S” consists of DwPTS, GP, and UpPTS [3].

Table 1. Downlink-Uplink Allocations in LTE TDD

| Configuration | Switch-point periodicity | Sub-frame number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st half-frame |  |  |  |  | 2nd half-frame |  |  |  |  |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 10 ms | D | S | U | U | U | D | S | U | U | D |

# Coexistence with 5 ms Periodicity Frame Configurations

The configurations 0, 1, and 2 have two identical 5 ms half-frames within a 10 ms LTE TDD frame, and thus, the optimal offset values to minimize punctured symbols can be derived from one of the previously described DL- or UL-alignment schemes between an IEEE 802.16m frame and a half-frame in LTE TDD.

The LTE specification defines the length of DwPTS, GP, and UpPTS as 9 cases for the normal CP and 7 cases for the extended CP, and has totally 48 cases with configuration 0, 1, and 2. From the best choice between DL-aligned and UL-aligned schemes, the number of punctured symbols can be limited to two symbols among 48 cases, while the maximum number of punctured symbols can be four if either DL-aligned or UL-aligned schemes is applied for all cases. The detailed frame alignment methods, offset, DL to UL ratios to minimize punctured symbols are summarized at Table 4 in Appendix.

# Coexistence with 10 ms Periodicity Frame Configurations

The configurations 3, 4, 5, and 6, which have 10 ms switch-point periodicity, have two different 5 ms frames within a 10 ms LTE TDD frame, and thus, it’s obvious that there will be a large number of punctured symbols when IEEE 802.16m TDD frame have the same DL to UL ratio for every frame.

Fig. 2 shows a coexistence scenario where the IEEE 802.16m frame is delayed by DL-aligned offset with the frame configuration 4 in LTE. The whole sub-frames mis-aligned with LTE should be punctured, and it can cause 25% loss in the spectral efficiency of IEEE 802.16m. In order to prevent this kind of sub-frame puncturing with LTE configuration 3, 4, and 5, asymmetric frame configurations, in which each frame can have a different DL to UL ratio from others, should be considered for IEEE 802.16m TDD.

In the configurations 3, 4, and 5, since the 1st half-frames are the same as the frames in the configurations 0, 1, and 2, respectively, and the 2nd half-frames consist of all DL sub-frames, asymmetric frame configuration with the same offset values derived from the configurations 0, 1, and 2 can be used as well. Because the all-DL part cause no symbol puncturing in the asymmetric frame configuration, the numbers of punctured symbols are the exactly same as those in the configuration 0, 1, and 2. The optimized DL to UL ratio can be defined as the combinations of all-DL ratio, i.e., 8:0 and the DL to UL ratio used in configuration 0, 1, and 2. Therefore, possible DL to UL ratio of IEEE 802.16 to coexist with 10 ms switch-point periodicity LTE frames can be 10:6, 11:5, 12:4, 13:3, 14:2, and 15:1.

Fig. 2 Asymmetric frame configuration for 10 ms switch-point periodicity in LTE

In the configuration 6, the 2nd half-frame does not consist of all-DL sub-frames, and thus, an asymmetric frame configuration can minimize punctured symbols. Since three DL sub-frames are spread across one LTE frame, it is not possible to use a DL-aligned asymmetric configuration. One sub-frame of LTE is 1 ms which is covered by 8 symbols in IEEE 802.16. Therefore, both DL-aligned asymmetric and DL-aligned symmetric configurations must yield more than 8 punctured symbols every 10 ms. However, we can find an available UL-aligned asymmetric configuration in which the number of punctured symbols is smaller than 8 for all DwPTS, GP, and UpPTS cases. Therefore, the UL-alignment is necessary to minimize punctured symbols in case of configuration 6 in LTE.

Fig. 3 shows an UL-aligned asymmetric frame configuration with frame configuration 6. The UL-aligned offset is the delay time of IEEE 802.16m 10 ms frame, which is newly defined as shown in Fig. 3, in the viewpoint of LTE TDD frame. The reference time of IEEE 802.16m is the start point of the DL region which has more consecutive DL sub-frames than the other.

Fig. 3. Asymmetric frame configuration for LTE configuration 6 (the 8th case of normal CP)

The UL-aligned asymmetric frame configuration can reduce the number of punctured symbols with LTE configuration 6. However, there exist multiple switching points between DL and UL in the odd frame, and thus, this configuration can be only used for legacy-support disabled mode. If the IEEE 802.16m operates as legacy-support mode, DL-aligned or UL-aligned symmetric frame configurations should be applied to coexist with LTE configuration 6. The detailed frame alignment methods, offset, DL to UL ratios to minimize punctured symbols are summarized at Table 5 in Appendix.

# Frame Alignment with UTRA LCR-TDD (TD-SCDMA)

The TD-SCDMA specification defines a 10ms radio frame which is divided into two 5ms sub-frames and each sub-frame consists of seven traffic time slots. The duration of one traffic time slot is 0.675ms. There is a synchronization and guard period with a duration of 0.275ms including DwPTS, GP and UpPTS. Table 2 shows the downlink-uplink allocations in TD-SCDMA.

The LCR-TDD frame has two identical 5 ms half-frames within a 10 ms frame like the configuration 0, 1, and 2 in LTE, and has only one set for DwPTS, GP, and UpPTS. Therefore, the optimal offset values to minimize punctured symbols can be derived from one of the previously described DL- or UL-alignment schemes for 5 ms periodicity frames.

Table 2. Downlink-Uplink Allocations in TD-SCDMA

| Configuration | Switch-point periodicity | Sub-frame number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st half-frame |  |  |  |  |  |  |  | 2nd half-frame |  |  |  |  |  |  |  |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 5ms | D | S | U | U | U | D | D | D | D | S | U | U | U | D | D | D |
| 1 | 5ms | D | S | U | U | D | D | D | D | D | S | U | U | D | D | D | D |
| 2 | 5ms | D | S | U | D | D | D | D | D | D | S | U | D | D | D | D | D |

Table 3 shows the comparison of the number of punctured symbols to coexist with LCR-TDD system. DL-aligned scheme shows smaller number of punctured symbols than UL-aligned scheme. However, the UL-aligned scheme still can be useful and be another option due to different DL to UL ratios of IEEE 802.16m regardless of more punctured symbols.

Table 3. IEEE 802.16m DL/UL ratios and punctured symbols to coexist with LCR-TDD

| DwPTS = 75 us, GP = 75 us, and UpPTS = 125 us |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LCR-TDD DL/UL Ratio | 4:3 |  | 5:2 |  | 6:1 |  |
| Comparison btw. DL- and UL-Aligned Schemes | DL-Aligned | UL-Aligned | DL-Aligned | UL-Aligned | DL-Aligned | UL-Aligned |
| DL/UL ratio for 802.16m TDD in 5ms frame, Punctured symbols in 5ms frame | 5:3, two | 4:4, three | 6:2, two | 5:3, three | 7:1, one | 6:2, four |
| Offset | 2975 | 3279.43 | 2300 | 2662.51 | 1625 | 2045.59 |

# Conclusion

We can summarize main results in this contribution as follows:

- The optimal coexistence scheme, offset, and DL to UL ratio for each configuration are summarized when the IEEE802.16m system coexists with E-UTRA TDD and UTRA LCR-TDD systems.

- To minimize the number of punctured symbols, one of DL-aligned and UL-aligned scheme can be selectively used for each configuration.

- To coexist with a frame configuration of 10 ms switch-point periodicity, asymmetric frame configurations should be considered for the IEEE802.16m TDD.

- The UL-alignment is necessary to minimize punctured symbols in configuration 6 of E-UTRA TDD.

# References

[1] IEEE 802.16m-07/002r4, “IEEE 802.16m System Requirements”.

[2] IEEE 802.16m-08/003r1, “Draft IEEE 802.16m System Description Document”.

[3] 3GPP LTE, TS26.211

# Appendix

Table 4. Alignment scheme (DL or UL-aligned), offset(), and DL/UL ratio of IEEE 802.16m with configurations of 5 ms switch-point periodicity

*PS : punctured symbol

| Configuration | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE TDD DL/UL Ratio | 1:3 |  |  |  | 2:2 |  |  |  | 3:1 |  |  |  |
| Normal CP | Align | offset | DL:UL | PS | Align | offset | DL:UL | PS | Align | offset | DL:UL | PS |
| 0 | DL | 0 | 2:6 or 3:5 | none | DL, | 4000 | 4:4, | none | DL | 3000 | 6:2, | None |
| 1 | DL | 0 | 3:5, | none | DL, | 4000 | 5:3, | one | UL | 3044.53 | 6:2, | None |
| 2 | DL | 0 | 3:5, | none | DL, | 4000 | 5:3, | one | UL | 3010.18 | 6:2, | None |
| 3 | DL | 0 | 3:5, | none | DL, | 4000 | 5:3, | one | UL | 3081.53 | 6:2, | One |
| 4 | DL | 0 | 3:5, | none | DL, | 4000 | 5:3, | one | UL | 3512.89 | 6:2, | One |
| 5 | DL or UL | 0 or 5003.64 | 2:6 or 3:5, | none | DL, | 4000 | 4:4, | none | DL | 3000 | 6:2, | None |
| 6 | UL | 5003.64 | 3:5, | none | DL, or UL, | 4000 or 4172.63 | 5:3, or 4:4, | two | UL | 3044.53 | 6:2, | None |
| 7 | UL | 5003.64 | 3:5, | none | DL or UL, | 4000 or 4244.01 | 5:3, or 4:4, | two | UL | 3010.17 | 6:2, | None |
| 8 | UL | 5003.64 | 3:5, | none | DL | 4000 | 5:3, | two | UL | 3081.53 | 6:2, | One |
| Extended CP |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | DL | 0 | 3:5, | none | DL | 4000 | 4:4, | none | DL | 3000 | 6:2, | None |
| 1 | DL | 0 | 3:5, | none | DL | 4000 | 5:3, | one | DL or UL | 3000 or 3067.97 | 7:1, or 6:2, | None |
| 2 | DL | 0 | 3:5, | none | DL | 4000 | 5:3, | one | DL or UL | 3000 or 3045.59 | 7:1, or 6:2, | None |
| 3 | DL | 0 | 3:5, | none | DL | 4000 | 5:3, | one | DL | 3000 | 7:1, | None |
| 4 | UL | 5085.39 | 3:5, | none | DL | 4000 | 4:4, | none | DL | 3000 | 7:1, | None |
| 5 | UL | 5085.39 | 3:5, | none | DL or UL | 4000 or 4196.1 | 5:3, or 4:4, | two | UL | 3067.97 | 6:2, | None |
| 6 | UL | 5085.39 | 3:5, | none | DL | 4000 | 5:3, | two | UL | 3045.59 | 6:2, | None |

Table 5. Alignment scheme (DL or UL-aligned), offset(), and DL/UL ratio of IEEE 802.16m with configurations of 10 ms switch-point periodicity

*PS : punctured symbol

| Configuration | 3 |  | 4 |  | 5 |  | 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE TDD DL/UL Ratio | 6:3 |  | 7:2 |  | 8:1 |  | 3:5 |  |  |  |
| Normal CP | Align, offset, and PS | Asymmetric 10ms DL:UL | Align, offset, and PS | Asymmetric 10ms DL:UL | Align, offset, and PS | Asymmetric 10ms DL:UL | Align | offset | Asymmetric 10ms DL:UL | PS |
| 0 | The same as configuration 0 | 10:6 or 11:5 | The same as configuration 1 | 12:4 | The same as configuration 2 | 14:2 | UL | 9458.07 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 1 |  | 11:5 |  | 13:3 |  | 14:2 | UL | 9458.07 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 2 |  | 11:5 |  | 13:3 |  | 14:2 | UL | 9458.07 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 3 |  | 11:5 |  | 13:3 |  | 14:2 | UL | 9458.07 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 4 |  | 11:5 |  | 13:3 |  | 14:2 | UL | 9386.72 | 4:5:3:4 (DL:UL:DL:UL) | 4 |
| 5 |  | 10:6 or 11:5 |  | 12:4 |  | 14:2 | UL | 9386.72 | 4:5:3:4 (DL:UL:DL:UL) | 4 |
| 6 |  | 11:5 |  | 13:3 or 12:4 |  | 14:2 | UL | 9386.72 | 4:5:3:4 (DL:UL:DL:UL) | 4 |
| 7 |  | 11:5 |  | 13:3 or 12:4 |  | 14:2 | UL | 9386.72 | 4:5:3:4 (DL:UL:DL:UL) | 4 |
| 8 |  | 11:5 |  | 13:3 |  | 14:2 | UL | 9386.72 | 4:5:3:4 (DL:UL:DL:UL) | 4 |
| Extended CP |  |  |  |  |  |  |  |  |  |  |
| 0 | The same as configuration 0 | 11:5 | The same as configuration 1 | 12:4 | The same as configuration 2 | 14:2 | UL | 9396.35 | 3:6:2:5 (DL:UL:DL:UL) | 4 |
| 1 |  | 11:5 |  | 13:3 |  | 15:1 or 14:2 | UL | 9446.1 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 2 |  | 11:5 |  | 13:3 |  | 15:1 or 14:2 | UL | 9446.1 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 3 |  | 11:5 |  | 13:3 |  | 15:1 | UL | 9446.1 | 4:5:3:4 (DL:UL:DL:UL) | 5 |
| 4 |  | 11:5 |  | 12:4 |  | 15:1 | UL | 9396.35 | 3:6:2:5 (DL:UL:DL:UL) | 4 |
| 5 |  | 11:5 |  | 13:3 or 12:4 |  | 14:2 | UL | 9196.097 | 4:6:2:4 (DL:UL:DL:UL) | 6 |
| 6 |  | 11:5 |  | 13:3 |  | 14:2 | UL | 9279.43 | 4:5:3:4 (DL:UL:DL:UL) | 8 |

Text Proposal for the 802.16m SDD

============================== Start of Proposed Text =================================

11.4.6 Coexistence Supports in Frame Structure

11.4.6.1 Coexistence with E-UTRA (LTE TDD)

The IEEE802.16m TDD system should be coexisted with the E-UTRA TDD system in the same or adjacent frequency band. The DL- or UL-aligned offset may be applied between the IEEE 802.16m frame and the E-UTRA TDD frame. Figure X shows how coexistence using two types of offsets can be achieved in order to minimize the number of punctured symbols within the IEEE 802.16m frame. As coexisting with E-UTRA TDD operating in a configuration of 10ms switch-point periodicity, the IEEE 802.16m shall support asymmetric DL/UL configurations within a 10 ms period to minimize the number of punctured symbols.

Figure X Alignment examples of IEEE 802.16m and LTE TDD frames

11.4.6.2 Coexistence with UTRA LCR-TDD (TD-SCDMA)

The IEEE802.16m TDD system can be coexisted with the UTRA LCR-TDD system in the same or adjacent frequency band. The DL- or UL-aligned offset may be applied between the IEEE 802.16m frame and the UTRA LCR-TDD frame.

============================== End of Proposed Text =================================