| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Grid-of-Beams (GoB) Based Downlink Multi-User MIMO |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Stephan SAUR, Hardy HALBAUER, Andreas RUEEGG, Frank SCHAICH Alcatel-Lucent Bell Labs Germany | Mail to: {Stephan.Saur, Hardy.Halbauer, Andreas.Rueegg, Frank.Schaich}@alcatel-lucent.de |
| Re: | IEEE 802.16m-08/016: Call for Contributions on Project 802.16m System Description Document (SDD): DL MIMO schemes |  |
| Abstract | MU-MIMO allows sharing one radio resource in time and frequency direction among several users exploiting Space Division Multiple Access (SDMA). Aim is to increase the mean cell throughput. This contribution focuses on a DL MU-MIMO scheme with low complexity based on a fixed grid of beams (GoB). The proposed concept can be applied in both open loop and closed loop mode. Moreover, it can be designed easily to support inter-cell interference coordination at the same time. |  |
| Purpose | Discuss and adopt this proposal into the 16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Grid-of-Beams (GoB) Based Downlink Multi-User MIMO

Stephan SAUR, Hardy HALBAUER, Andreas RUEEGG, Frank SCHAICH

Alcatel-Lucent Bell Labs, Germany

# Background

One aim of 16m is to boost the mean cell throughput. This can be achieved by establishing the principle of Space Division Multiple Access (SDMA) on top of OFDMA. One radio resource in time and frequency direction is shared among several users. Downlink Multi-User (MU)-MIMO is an important enabling building block for that. The fact that several users need to be grouped raises the question about MU-scheduling, i.e. which users will share the time and frequency allocation. The MU signal can be designed such that the mean intra-cell interference is minimized. However, the realization of an effective inter-cell interference coordination scheme, which is prerequisite for the achievement of the required performance figures in particular in systems with low frequency re-use factor, provides additional requirements and will become very difficult or even impossible without appropriate system design .

There is the clear need of a downlink MU-MIMO scheme that maintains the possibility for inter-cell interference coordination as it can be applied today for single-user beamforming in 16e. The idea focused in this contribution is based on a fixed grid of beams. It can be designed to support inter-cell interference coordination at the same time.

# Concept Description

The basic concept is that all transmission streams, represented by QAM symbols dn, n=1...N, that have to be sent on the same time-frequency allocation are weighted at the BS with precoding vectors pn. pn are elements of a limited set of predefined vectors, the so called fixed beams. The number of transmission streams assigned to one user is either one or two assuming that the mobile station is equipped with two receive antennas. In the case of two streams assigned to one user, either two different precoding vectors may be used for the two streams, or the same precoding vector may e used, if cross-polarized tx-antenna elements are available. In the latter case, each stream is sent on one polarization direction. This might be the preferable solution. Figure 1 shows the basic principle of this transmission scheme. The precoding matrix in Figure 1 is P=[p1...pN].

GoB

Precoding

P

user signal d1

user signal dN

…

Base Station

Mobile Station #n

SM

Detector

W

user signal dn

Channel

H

Feedback of desired beam index

Figure 1: Transmission scheme

The vectors pn shall be designed such that the corresponding radiation patterns build up a so called grid of beams as illustrated in Figure 2.

#8

#7

#6

#5

#4

#3

#2

#1

Figure 2: Schematic illustration of the beam grid

The example shows a set of eight different beams which exhibit spatial separation to a certain degree. At the same time, a subset of beams (e.g. two) is used to serve users in an SDMA manner. The assignment of beams to users has to be made according to the location of the respective user within the cell. The subsets (e.g. pairs) of beams shall be designed such that intra-cell interference is minimized. As an example, beams #1 and #5 could build up a suitable combination. That is, two users preferring these beams are candidates to be scheduled together on the same OFDMA allocation.

In contrast to alternative approaches of MU-MIMO modes, the proposed scheme allows for the application of advanced inter-cell interference coordination techniques, that is however a prerequisite for cellular networks with low frequency reuse factor.

The assignment of the beam index shall be carried out at the base station. In closed loop mode, the mobile terminal reports its preferred beam(s). The mobile station may determine its preferred beams by measuring and evaluating DL test signals, which are sent for each precoding vector. An additional standard procedure for these test signals has to be introduced. In open loop mode, the base station has to derive appropriate beams from the uplink signals (e.g. by AoA estimation).

# Requirements

- In closed loop mode, the mobile station has to report preferred indices. At least three uncoded feedback bits are necessary assuming one index out of eight possible values is reported.

- The availability of downlink test signals for all possible beams can ease the MS decision which preferred beam(s) it will report to the BS in closed loop mode.

# Text Proposal to SDD

------------------------------------Start text proposal------------------------------------

MIMO modes

In 16m, three MIMO modes shall be supported:

- Single-BS single-user MIMO (SU-MIMO)

- Single-BS multi-user MIMO (MU-MIMO)

- In downlink, a MU-MIMO scheme based on a limited number of possible precoding vectors building up a grid of beams with spatial separation shall be supported. This scheme aims in increasing the mean cell throughput even in cellular systems with low frequency reuse factor. Main advantage is the capability to support advanced inter-cell interference coordination techniques at the same time. The proposed scheme exhibits very low complexity and can be applied in both closed and open loop MU-MIMO. Either one or two data streams can be assigned to one user. The use of cross-polarized antennas at the base station is recommended.

- Multi-BS MIMO:

------------------------------------End text proposal------------------------------------