| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Classification of DL MIMO Modes |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Joerg Schaepperle Juergen Otterbach Andreas Rüegg Alcatel-Lucent Bell Labs Germany | Voice: +4971182132266 E-mail: {Joerg.Schaepperle, Juergen.Otterbach, Andreas.Rueegg}@alcatel-lucent.de *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) 802.16-08/016r1 Subject: DL MIMO schemes |  |
| Abstract | The antenna configurations in 802.16m (up to 4x4) and the different operation scenarios allow a big variety of multi-antenna transmission schemes. This document proposes means of classificaton for any DL MIMO mode to be supported by 16m. |  |
| Purpose | TG should review, adopt and provide means in SDD to classify the variety of DL MIMO modes foreseen in 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Classification of DL MIMO Modes

Joerg Schaepperle, Juergen Otterbach, Andreas Rüegg

Alcatel-Lucent, Bell Labs Germany

# Introduction

According to [1] §5.7  a minimum requirement of 2 TX antennas at the BS and 2 RX antennas at the MS is foreseen. Target antenna configuration is 4x4. IEEE 802.16m shall further support single-user and multi-user MIMO techniques. This allows a big variety of possible antenna configurations and transmission schemes to be defined in the 16m TG.. This contribution proposes means for classification of  DL MIMO modes in IEEE 802.16m.  Some examples of DL MIMO modes for 802.16m are given but the definition of the totality of DL MIMO modes is for further study.

# DL MIMO Modes

The modes are characterized by

- Number of Tx antennas

- Number of encoded data streams

- Number of subscriber stations (SSs), each SS being identified by the CID of its connection.

- Number of used orthogonal pilot patterns

- Matrix that maps information symbols to time slots and antennas

- Mapping of encoded data streams to matrix elements

Examples are shown in Table 1.

Matrix definitions A and B are according to [2] § 8.4.8  and the number refers to the number of TX antennas.

| Number of Tx antennas | Number of encoded data streams | Number of orthogonal pilot patterns | Matrix | Mapping of encoded streams to matrix elements | Number of subscriber stations | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | A2 | Encoded stream #0: S1, S2 | 1 | STC |
| 2 | 1 | 2 | B2 | Encoded stream #0: S1, S2 | 1 | SM vertical encoded |
| 2 | 2 | 2 | B2 | Encoded stream #0: S1 Encoded stream #1: S2 | 1 | SM horizontal encoded |
| 2 | 2 | 2 | B2 | Encoded stream #0: S1 Encoded stream #1: S2 | 2 | SM horizontal encoded |
| 4 | 1 | 4 | A4 | Encoded stream #0: S1, S2, S3, S4 | 1 | STC |
| 4 | 1 | 4 | B4 | Encoded stream #0: S1, S2, S3, S4 | 1 | SM |
| 4 | 2 | 4 | B4 | Encoded stream #0: S1, S2, S5, S7 Encoded stream #1: S3, S4, S6, S8 | 2 | SM |
| 4 | 4 | 4 | [TBD] | [TBD] | 4 | MU-MIMO, 4 users |
| 1 | 2 | 1 | [TBD] | [TBD] | 2 | Superposition |
| 2 | 4 | 2 | [TBD] | [TBD] | 4 | Superposition |

Table 1: Examples for DL MIMO modes

To achieve capacity in broadcast channels, modes with number of streams larger than the number of antennas at transmitter are included. For these, joint detection is required at the receiver.

# Proposed Text for SDD

IEEE 802.16m supports different DL MIMO modes for different channel conditions. The modes are characterized by

- Number of Tx antennas

- Number of encoded data streams

- Number of subscriber stations (SSs), each SS being identified by the CID of its connection.

- Number of used orthogonal pilot patterns

- Matrix that maps information symbols to time slots and antennas

- Mapping of encoded data streams to matrix elements

Definition of MIMO modes and required control structures are for further study. Examples for MIMO modes are given in Table 1.

Abbreviations

SU-MIMO	single user MIMO

MU-MIMO	multi user MIMO

STC		Space Time Coding

SM		Spatial Multiplexing

CID		Connection Identifier

References

- IEEE 802.16m-07/002r4, “802.16m System Requirements”

- Part 16: Air Interface for Broadband Wireless Access Systems, P802.16Rev2/D4

Parts of this contribution are based on work supported by the FP7 project WiMAGIC