| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed MSCCH Structure for Supporting E-MBS on dedicated carrier in IEEE 802.16m System |  |
| Date Submitted | 2008-5-8 |  |
| Source(s) | Chun-Yen Wang, ITRI Chun-Yuan Chiu, ITRI Chie-Ming Chou, ITRI Fang-Ching (Frank) Ren, ITRI Richard Li, ITRI Wern-Ho sheen, NCTU/ITRI | Voice: E-mail: ChunYen@itri.org.tw richard929@itri.org.tw |
| Re: | Call for comments on contribution IEEE C802.16m-08/297 (Proposed Baseline Content on the Downlink Control Structure for the 802.16m SDD). |  |
| Abstract | This contribution proposes a MSCCH control structure from supporting E-MBS point of view. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed MSCCH Structure for Supporting E-MBS on dedicated carrier in IEEE 802.16m System

Chun-Yen Wang, Chun-Yuan Chiu, Chie-Ming Chou, Fang-Ching (Frank) Ren, Richard Li

ITRI

Wern-Ho sheen

ITRI/NCTU

# 1.	Introduction

In the IEEE 802.16m system requirement document (SRD) [1], it has been agreed that IEEE 802.16m shall support enhanced multicast-broadcast service (E-MBS) in an efficient manner, including achievement of higher spectrum efficiency in multi-cell multicast-broadcast single frequency networks (MBSFN) on both mixed and dedicated frequencies. In addition, the design for multicast service control channel (MSCCH) structure on mixed carrier has been presented in the contribution C802.16m-08/376 [4]. Thus, based on the proposal in [4], this contribution further proposes a MSCCH structure on dedicated carrier so as to meet the E-MBS requirements specified in the IEEE 802.16m SDD.

# 2.	Proposed MSCCH Structure on dedicated carrier

According to the proposal in [4], MSCCH is separated into primary MSCCH (P-MSCCH) and secondary MSCCH (S-MSCCH). And BCH would not have any service-specific information, but provide an indication for E-MBS interested users to retrieve the P-MSCCH, which would contain the essential control information for all E-MBS services. Therefore, in order to support E-MBS transmission on dedicated carrier, it is proposed that the P-MSCCH in the primary carrier shall maintain a service list, which classifies E-MBS services according to their transmission carriers. In addition, the P-MSCCH in the primary carrier shall also contain some dedicated carrier description (e.g., CP size, carrier frequency, bandwidth, etc.) to fasten E-MBS users to access the dedicated carrier. The proposed MSCCH structure is illustrated in Fig. 1.

Fig. 1. The proposed MSCCH structure on dedicated carrier

The MSCCH structure of the dedicated carrier is similar to that of the mixed carrier as described in [4]. The P-MSCCH of the dedicated carrier only contains the control information of the E-MBS services, which are transmitted on this dedicated carrier. Similarly, the P-MSCCH of the dedicated carrier contains those slow changing or the cell-specific control information. And for each MBSFN service, it would also provide the schedule information of its corresponding S-MSCCH.

Likewise, the S-MSCCH of the dedicated carrier mainly contains fast changing control information (e.g., the schedule information of the incoming data traffic, etc.). Moreover, the S-MSCCH still contains the schedule information of the next S-MSCCH. Also, a “change indicator” bit to indicate that some P-MSCCH control information has been updated is provided as well.

As for a MS who would like to access a E-MBS service which is transmitted on dedicated carrier, it can follow the indication from the BCH of the primary carrier to retrieve the P-MSCCH of the primary carrier. And then, the MS would find that the desired E-MBS is served on the dedicated carrier. Thus, according to the dedicated carrier description in the P-MSCCH of the primary carrier, the MS can switch to the E-MBS dedicated carrier and get some control information from the P-MSCCH of the dedicated carrier. In the meanwhile, the MS also get the schedule information of the corresponding S-MSCCH. After receiving the S-MSCCH, the MS obtain all the necessary control parameters and the schedule information of incoming data traffic. Afterwards, since the S-MSCCH also contains the resource information of the next S-MSCCH, the MS can continue the process S-MSCCH  data  S-MSCCH  data … to enjoy the incoming E-MBS service. Once the “change indicator” is set or some error happens in decoding the S-MSCCH, the MS can re-read the P-MSCCH of the dedicated carrier to get the update control parameters.

# 3.	Conclusions

In order to support E-MBS transmission, this contribution presents a MSCCH control structure on dedicated carrier for IEEE 802.16m system. Based on the proposed control structure, E-MBS subscribers are able to retrieve their desired E-MBS services in an efficient way. Therefore, it is suggested to capture the proposed text into the IEEE 802.16m system description document (SDD) [5].

# References

[1]	IEEE 802.16m-07/002r4, “802.16m System Requirements”

[2]	C802.16m-08/297, “Proposed Baseline Content on the Downlink Control Structure for the 80.16m SDD”

[3]	IEEE C802.16mDL_Ctrl-08/033, “Proposed Text for Draft 2 of DL Control Rapporteur Group Contribution”

[4]	C802.16m-08/376, “Proposed MSCCH Structure for Supporting E-MBS on mixed carrier frequency in IEEE 802.16m System”

[5]	802.16m-08/003r1, “The Draft IEEE 802.16m System Description Document”

# Proposed Text

The following text is proposed to be captured in the IEEE 802.16m system description document (SDD).

------------------------------------------------------- Start of the proposed text ------------------------------------------------

[Insert the following text into this section]

# 11.x.7.	Multi-carrier Control Structure

# 11.x.7.x	   MSCCH structure on E-MBS dedicated carrier

Fig. X.	MSCCH control structure on E-MBS dedicated carrier

The MSCCH control structure on E-MBS dedicated carrier is depicted in Fig. X.

The P-MSCCH of the primary carrier shall maintain an E-MBS service list, which classifies all of the E-MBS services served in this cell according to their transmission carrier.

The P-MSCCH of the primary carrier shall contain some dedicated carrier description (e.g., CP size, carrier frequency, bandwidth, etc.), and the resource information about the P-MSCCH of the dedicated carrier.

The P-MSCCH of the dedicated carrier only contains the control information of the E-MBS services, which are transmitted on this dedicated carrier.

The control information content carried in the P-MSCCH and S-MSCCH of the dedicated carrier is similar to that of the mixed carrier as described in section 11.x.6.

-------------------------------------------------------- End of the text ------------------------------------------------------