| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Alternate SDD Text on Downlink Control Structure Section 11.x.1 |  |
| Date Submitted | 2008-05-12 |  |
| Source(s) | Dominic Tolli , Mark Cudak, Fan Wang | fanw@motorola.com Mark.Cudak@motorola.com tolli@motorola.com |
| Re: | DL Control Rapporteur Group Discussions Call for comments on DL_CTRL |  |
| Abstract | This text proposes alternative text to the C802.16mDL_ctrl-08/297. |  |
| Purpose | For review and discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Alternate SDD Text on Downlink Control Structure Section 11.x.1

Dominic Tolli , Mark Cudak, Fan Wang

Motorola, Inc.

----------------   Proposed Text for DL Control Information Classification ---------------

# DL Control Information Classification

Information carried in the control channels has been classified as follows.

# Synchronization information

This type of control information is necessary for system acquisition and synchronization.

# Essential system parameters and system configuration information

This control information includes a minimal set of time-critical system configuration information and parameters needed for the mobile station (MS) to complete initial access in a power-efficient manner. Examples of such information are cyclic prefix (CP) size, system bandwidth, etc. This group of information also includes the superframe configuration information related to the configuration of sub-frames within the superframe. There are multiple types of essential information described in the following sections.

# Network-wide essential information

This group includes PHY layer parameters that are common among all sectors in the network and essential for decoding the DL. Examples of this type of information include CP length, system bandwidth (at least DL bandwidth, UL bandwidth may be classified as “Remaining essential” if asymmetric bandwidth is supported), TDD ratio, and superframe index.

# Sector-specific DL essential information

This group includes sector-specific parameters and information that are needed for the MS to decode the DL-broadcasted system message that are carried on the traffic channels. Examples of this type of information include antenna configuration, DL resource allocation configuration (for potential flexible frequency reuse), pilot configuration, 16e/16m co-existence configuration, as well as indicators for either boosting the common reference signal power or existence of a HARQ acknowledgement.

# Sector-specific UL access information

This group includes sector-specific parameters and information that are needed for the MS to initiate access on the uplink (UL). Examples of this type of information include the initial ranging channel type, UL bandwidth if asymmetric bandwidth is supported, and UL power control parameters.

# Remaining system parameters and configuration information

This group includes the remainder of parameters and information not sent using the primary broadcast resource, instead relying on secondary broadcast resources or unicast resources. Examples of this type of information include UL resource allocation configuration (if different from the DL one), UL pilot configuration, UL bandwidth split between data and control regions, and MB-SFN subframe information.