| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Changes to Chapter 6 of SDD |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Sean McBeath, Huawei Technologies Yair Bourlas, Nextwave Mo-Han Fong, Nortel Networks | Voice: E-mail: smcbeath@huawei.com YBourlas@nextwave.com mhfong@nortel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/024: Call for Contributions on Project 802.16m System Description Document (SDD), Target topic: “SDD Text”. |  |
| Abstract | This contribution suggests the removal of IEEE 802.16e-2005 message names from chapter 6 of the SDD. |  |
| Purpose | Discussion and approval by TGm for the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Changes to Chapter 6 of SDD

Sean McBeath, Huawei Technologies

Yair Bourlas, Nextwave

Mo-Han Fong, Nortel Networks

# Introduction

Chapter 6 of the SDD consistently refers to messages defined in IEEE 802.16e-2005.  It is premature to specify the specific messages used for the functions outlined in Chapter 6.  In this contribution, we propose changes to remove reference to these specific messages (Change Set 2) as well as several editorial changes (Change Set 1).

# Text Changes

Change Set 1

Blue = editorial insertions

Red = editorial deletions

Change Set 2

Pink = insertions in support of removing IEEE 802.16e message names

Gray = deletions in support of removing IEEE 802.16e message names

# 6.  IEEE 802.16m Mobile Station State Diagrams

<Editor’s Note: To capture only the top level states of the mobile stations, base stations. Detailed feature specific state diagrams will be captured elsewhere in the respective sections.>The following Figure  illustrates the system state transition diagram for IEEE802.16m BS and MS. System state diagram for IEEE802.16m systems consists of 4 states,  Initialization state, Access state, Connected state and Idle state. The Connected state consists of 3 separated modes which are sleep mode, active mode and scanning mode based on its operation and MAC message generation. The Idle state consists of 2 separated modes, paging listening mode and paging unavailable mode based on its operation and MAC message generation.

Figure 5 System State Transition Diagram of IEEE802.16m

## 6.1 Initialization State

Initialization State is where an MS performs cell selection by scanning and synchronizing to a BS preamble, and acquiring the system configuration information through BCH before it is ready to perform a ranging process to entering Access State. The MS can return back to the scanning step in case that it fails to perform the actions required to for each step.

Figure 6 Initialization State Transition Diagram

During this state, if the MS could can not properly perform the BCH information decoding and cell selection, it should be back return to perform the scanning and DL synchronization. The red dashed line represents stands the abnormal case. If the MS successfully decodes BCH information, it goes to the Access Sstate. If the abnormal case occurs in the Access sState, the states transition should be achieved from the Initialization state again.

## 6.2 Access State

Access State is where the MS performs network entry to the selected BS by going through several processes. A The MS performs the ranging process using RNG-REQ/RSP MAC message in order to get obtain UL synchronization. A Next, the MS performs the basic capability negotiation with the BS using SBC-REQ/RSP MAC message. A The MS then performs the authentication and authorization process through key exchanging. And finally a Next, the MS performs the registration process using REG-RES/RSP MAC message. followed by a Finally, the MS gets receives the MAC CID and IP address.

Figure 7 Access State Transition Diagram

[Note to Editor: Remove message names in parenthesis from Figure 7]

Upon successfully performing the aAccess sState operation, a the MS goes to the cConnected sState in order to exchange the user data between the BS and MS. Otherwise, in case of abnormal operation, a the MS goes back to the Initialization ssState in case of abnormal operation.

## 6.3 Connected State

The connected Connected state State consists of 3 modes;: sleep Sleep mode Mode, active Active mode Mode and scanning Scanning mode Mode. During Connected State, the MS maintains at least one connection as established during Access State, while MS and BS may establish additional transport connections. In addition, to save reduce power consumption of the MS during exchanging the user datae exchange, a the MS or BS can request a transition to sleep mode. And also, MS can scan neighbor cell’s signal to reselect a cell which provides robust and reliable services.

Figure 8 Connected State Transition Diagram

### 6.3.1 Active mode

During Active Mode, the MS and the BS perform normal operations to exchange the DL/UL traffic transaction between the MS and BS. And MS can perform the Fast re-entry procedures after handover:; while in handover, MS maintains its IP address and any 802.16m specific IDs which are local to the MSCID and IP address are remained. Without going through aAccess sState, MS can remain keep in cConnected sState with target BS.

### 6.3.2 Sleep mode

During Sleep mode, MS can do may enable power saving techniques during traffic interval. MS in Active mode transitions to sleep mode through sleep mode MAC signaling management messages such as MOB_SLP-REQ/RSP. MS does not transmit and receive any traffic to/from its BS in during the sleep interval. A MS can receive a MOB_TRF-IND message during listening interval and then whether a MS can transit to active mode or be stayed in sleep interval according to a indication bit in MOB_TRF-IND message.

### 6.3.3 Scanning mode

During scanning mode, the MS performs scanning operation and may be temporarily unavailable to the BS, and performs scanning operation. While in active mode, MS transitions to scanning mode through explicit MAC signaling scanning transaction through MOB_SCN-REQ/RSP. In this mode, MS is unavailable to BS. In addition, a the MS can may perform the implicit scanning procedures, meaning that the MS performs a scansning other BSs without MAC signaling scanning management messages generation.

## 6.4 Idle State

During Idle sState, the MS may performs power saving by switching between Paging listening mode and Paging Unavailable mode

Figure 9 Idle State Transition Diagram

### 6.4.1 Paging Listening Mode

The MS may be paged by the BS while it is in During the paging listening mode, MOB_PAG-ADV is received. Iif a the MS is paged, it MS shall transitions to the access Access state State for its network re-entry. Location update procedure is can also be achieved.

### 6.4.2 Paging Unavailable Mode

During paging unavailable mode, MS does not need to monitor the down link channel in order to reduce save its power consumptions. While in this mode, MS can also transition to access state Access State if required.