| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A two-stream Alamouti scheme for MIMO DL in 802.16m |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Ramesh Annavajjala, Andreas F. Molisch, Philip V. Orlik, Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Lab Toshiyuki Kuze Mitsubishi Electric Corp | Voice: 617-621-{7518,7558,7570, 7557, 7595} Fax: 617-621-7550 { annavajjala, molisch, porlik, tao, jzhang}@ merl.com Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | MIMO DL Rapporteur Group Discussions |  |
| Abstract | Contribution to the TGm SDD on MIMO DL. |  |
| Purpose | Discuss and accept the proposal into the Draft of MIMO DL SDD document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A two-stream Alamouti scheme for MIMO DL in 802.16m

Ramesh Annavajjala, Andreas. F. Molisch, Philip V. Orlik, Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze

Mitsubishi Electric Corp

Introduction

The mapping of the transmission layers to the different antennas is an important part of the DL-MIMO specifications currently under development in the 16m group. In this contribution, we concentrate in particular on the case of 4 transmit and 2 receive antennas. This case is relevant for practical deployment because many BSs are anticipated to use 4 antenna elements, since this can provide high diversity while keeping dimensions of the array (and thus windload) reasonably small. Furthermore, we are concentrating on the case of two data streams that are to be transmitted to the receiver.

The current Draft 3 of the DL-MIMO rapporteur group suggests the following specifications for the 4 TX, 2 RX, 2 stream case: the input to the MIMO encoder is represented as a 2  1 vector

x=s1s2.

The output of the MIMO encoder is a 2  1 vector

z=x.

The output of the precoder is a 4  1 vector

y=D×W×z,

where W is a 4  2 unitary precoder and D is a 4  4 diagonal delay matrix. Note that W and D may be frequency dependant as described in section 11.x.2.1.1.

In this contribution we suggest an alternative scheme that is simpler to implement and gives better results. Namely, we suggest the use of a two-stream Alamouti scheme. The MCSs (modulation and coding schemes) for the two streams can be adjusted individually (based on the CQI fed back from the receiver). Such schemes, also known as DSTTD, have been proposed and analyzed in the scientific literature (see, e.g., [1-3]) and are also implemented in the IEEE 802.11n standard.

Proposed transmitter structure

Figure 1 shows the structure of the proposed transmitter. The 2 data streams (layers) are independently encoded by two standard Alamouti encoders, resulting in 4 outputs. These outputs are then mapped onto the TX antennas in such a way that outputs from Alamouti encoder A are mapped onto antenna elements 1 and 3, while outputs from Alamouti encoder B are mapped onto antenna elements 2 and 4, giving good decorrelation.

Figure 1: Description of the proposed system transmitter.

Simulation results

In Figures 2-4, the average symbol error rate performance of three schemes are compared for QPSK, 16-QAM and 64-QAM constellations, respectively. The following are the schemes under comparison:

- Proposed two-stream Alamouti scheme with 4-transmit anteannas and two-receive antenna MMSE receiver

- Proposed scheme with an additional per-tone cyclic delay diversity

- Two-stream spatial multiplexing scheme with unitary precoding followed by cyclic delay diversity.

An IEEE Veh-A channel is assumed with 60 Hz Doppler, and perfect channel estimation at the receiver. As shown in Figs. 2-4, the proposed system performs significantly better than the two-stream spatial multiplexing without Alamouti enoding (i.e., scheme c). On the other hand, with an additional per-tone cyclic delay diversity over the proposed scheme marginally improves the performance at higher signal-to-noise ratios.

Figure 2: Symbol error rate performance for QPSK modulation.

Figure 3: Symbol error rate performance for 16-QAM modulation.

Figure 4: Symbol error rate performance for 64-QAM modulation.

Text proposal

for the 4 TX, 2 RX, 2 stream case: the input to the MIMO encoder is represented as a 2  2 vector

where the two rows represent the two layers, and the columns two (subsequent) time instants.

The output of the Alamouti encoders is a 4  2 matrix

The output of the encoder is mapped onto the antennas after multiplication by multiplication

,

where W is a 4  4 permutation matrix

The MCSs of the two layers can be adapted independently, based on CQI feedback from the receiver.

# References

[1] Onggosanusi E.N.; Dabak A.G.; Schmidl T.A., High rate space-time block coded scheme: performance and improvement in correlated fading channels, Wireless Communications and Networking Conference, 2002, 194-199

[2] Kyungchul Kwak; Jihyung Kim; Byungjoon Park; Daesik Hong, Performance analysis of DSTTD based on diversity-multiplexing trade-off,  Vehicular Technology Conference, 2005, 1106-1109.

[3] Giulii Capponi; Yahong R. Zheng; Gumaste, A.; Chengshan Xiao, Error Performance of Double Space Time Transmit Diversity System, 2006 IEEE International Conference on Communications, 4859-4864.