| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Abnormal Case for MS state transition |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Allan Xu Huawei Xiaolu Dong, Ying Du CATR | E-mail: zhier81@huawei.com dongxiaolu@mail.ritt.com.cn duying@mail.ritt.com.cn |
| Re: | IEEE 80216m-08_024: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) : Topic: “Call for Comments” |  |
| Abstract | MS in Connected State or Idle State also can transit to Initialization State in case of abnormal case. |  |
| Purpose | Discussion and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Abnormal Case for MS state transition

Allan Xu

Huawei

Xiaolu Dong, Ying Du

CATR

# 1. Introduction

In SDD document 003r3, Figure 5 depicts the mobile station state transition diagram of IEEE 802.16m, In this figure we can see MS in Access State MS goes back to Initialization State in case of abnormal operation, but MS in Connected State or Idle State can’t transit to Initialization State. We can image that when a mobile station goes into blind area, this MS will go back to Initialization State to perform cell selection. So MS in Connected State or Idle State will transit to Initialization State in case of abnormal operation.

# 2. Text Proposal

Remedy 1:

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Change Figure 5 of 003r3 as following:]

Power On/Off

Initialization State

Access State

Connected State

Idle State

Normal Network Re-entry

Fast Network Re-entry

Power Down

: Abnormal Case

: Normal Case

-------- End of proposed text ---------------------------------------------------------------------------------------------

Remedy 2:

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Change Figure 6 of 003r3 as following:]

Power On/Off

Initialization State

Scanning and DL synchronization

(Preamble Detection)

DL MAP (Common Control Channel) Acquisition

Broadcast Channel Acquisition

Cell Selection Detection

To Access State

From Access State

From Connected State

From Idle State

-------- End of proposed text ---------------------------------------------------------------------------------------------

Remedy 3:

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Change Figure 8 of 003r3 as following:]

Connected State

Sleep Mode

Active Mode

From Connected State

To Initialization State

Sleep interval

Listening interval

Scanning Mode

From Access State

To Connected State

To Idle State

-------- End of proposed text ---------------------------------------------------------------------------------------------

Remedy 4:

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Change Figure 9 of 003r3 as following:]

Idle State

Paging Listening Mode

Paging Unavailable Mode

To Access State

From Connected State

To Initialization State

-------- End of proposed text ---------------------------------------------------------------------------------------------