| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power Management in IEEE 802.16m System |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Lina Liu, Jin Lei, Ke Zeng, Juejun Liu, Zhengzheng Li, Yiming Gao Huawei Technologies | Voice: E-mail: liulina@huawei.com jinlei60020191@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/024: Call for Contributions on Project 802.16m System Description Document (SDD), Target topic: “Power Management”. |  |
| Abstract | This contribution suggests SDD text related to power management. |  |
| Purpose | Discussion and approval by TGm for the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power management Management in IEEE 802.16m Ssystem

Lina Liu, Jin Lei, Ke Zeng, Juejun Liu, Zhengzheng Li, Yiming Gao

Huawei Technologies

# Introduction

Chapter 6 of the SDD is intended to capture only the top level states of mobile stations and base stations. Detailed feature specific state diagrams should be captured elsewhere in the respective sections.

In contribution IEEE 802.16m_08/688, the definition of subframe based sleep mode and frame based sleep mode are proposed.   In addition, for paging mechanism, BCH should be received before decoding the paging message. In order to reduce power consumption, a superframe is proposed as the basic unit for paging listening/unavailable interval.

# Text Changes

[note to the editor: Make the following changes to the TGm SDD as shown below;

Blue = insertions

Red = deletions

]

------------------------------------------------------------Start of the Text------------------------------------------------------

[Modify the following text after line#12, page# 15 of the TGm SDD]

## 6.3.2 Sleep mode

During Sleep mode, MS can do power saving during traffic interval. MS in Active mode transits to sleep mode through sleep mode MAC management messages such as MOB_SLP-REQ/RSP. MS does not transmit and

receive any traffic to/from its BS in sleep interval (measured in subframes/frames). A MS can receive a MOB_TRF-IND message the signaling during listening interval and then whether a MS can transit to active mode or be stayed in sleep interval according to a indication bit in MOB_TRF-IND message.

-------------------------------------------------------------End of the Text----------------------------------------------------

------------------------------------------------------------Start of the Text------------------------------------------------------

[Modify the following text after line#8, page# 16 of TGm SDD]

### 6.4.1 Paging Listening Mode

During the paging listening mode (measured in superframes), MOB_PAG-ADV is received. If a MS is paged, MS transits to access state for its network re-entry. Location update procedure is also achieved.

### 6.4.2 Paging Unavailable Mode

During paging unavailable mode (measured in superframes), MS does not need to monitor down link channel in order to save its power consumptions. While in this mode, MS can also transit to access state if required.

-------------------------------------------------------------End of the Text----------------------------------------------------