| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MS Initial Entry Procedure for Multi-carrier Support in IEEE 802.16m |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Dora Chen, Stanley Hsu, Paul Cheng MediaTek Inc. No.1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan, R.O.C. | IK.Fu@mediatek.com |
| Re: | 802.16m-08/024, “Call for Comments and Contributions on Project 802.16m System Description Document (SDD)” - Comments on New Text in SDD |  |
| Abstract | Based on current definition on multi-carrier operations, this contribution aims to clarify the MS initial entry procedure for multi-carrier support in IEEE 802.16m |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MS Initial Entry Procedure for

Multi-carrier Support in IEEE 802.16m

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Dora Chen, Stanley Hsu, Paul Cheng

MediaTek Inc.

- Introduction

In P802.16m System Description Document (SDD), the primary carrier is defined as:

  - Primary carrier is the carrier where the BS and the MS exchange traffic and full PHY/MAC control information defined in 16m specification. Further, the primary carrier is in charge of delivering all control information for proper MS operation. Each MS shall have only one primary carrier.

Since MS shall have only one primary carrier and the MS need to acquire complete control information at initial network entry stage. The only feasible initial network entry procedure is that MS performing initial Ranging and network entry only in primary carrier first, and then determine to enable the secondary carrier or not for communication with serving BS after initial network entry. Therefore, some text input into P802.16m SDD will be necessary and the Fig.1 is an example to illustrate the basic idea.

Fig.1 MS initialization procedure for multi-carrier support

After initial network entry, the system may determine to enable the secondary carrier for communication between the MS and the serving BS. There are two possible Ranging mechanisms for secondary carrier. If the secondary carrier includes the control channel for Ranging response, the Ranging in secondary carrier can be the same as normal case for single carrier operation (as shown in Fig.2).

Fig.2 UL Ranging with multi-carrier support (1)

If the secondary carrier is partially configured and unable to reply Ranging response, BS can reply the Ranging response to MS via primary carrier as shown in Fig.3.

Fig.3 UL Ranging with multi-carrier support (2)

- Text Proposal

-------------------------------------------------------------Start of the Text-------------------------------------------------------

[Add the following text after line#25, page#48 of IEEE 802.16m-08/003r3]

When IEEE 802.16m BS perform services over multiple RF carriers, the MS shall perform initial network entry over one of the primary carriers and follow the initialization flow depicted in Fig.x.

Fig.x MS initialization flow for multi-carrier support in IEEE 802.16m

--------------------------------------------------------------End of the Text-------------------------------------------------------