| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Possible Bandwidth Aggregation for Multi-carrier MS in IEEE 802.16m |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | I-Kang Fu, Pei-Kai Liao, Paul Cheng MediaTek Inc. No.1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan, R.O.C. | IK.Fu@mediatek.com |
| Re: | IEEE 802.16m-08/024, “Call for Comments and Contributions on Project 802.16m System Description Document (SDD)” - Comments on P802.16m SDD |  |
| Abstract | This contribution provides several examples on possible MS transceiver architecture for multi-carrier support in IEEE 802.16m system. It shows that there are different ways to aggregate multi-carrier bandwidth, which corresponds to different receiver bandwidths. |  |
| Purpose | For TGm discussion and adoption on proposed text revision |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Possible Bandwidth Aggregation for

Multi-carrier MS in IEEE 802.16m

I-Kang Fu, Pei-Kai Liao, Paul Cheng

MediaTek Inc.

- Introduction

In order to support multi-carrier operations in IEEE 802.16m system, the MS need to be capable to receive and transmit the radio signals over different RF carriers at the same time. In addition, the MS may also need to transmit or receive signal over the guard sub-carriers when contiguous RF carriers are operated by the same system. In order to support multi-carrier operation and data transmission over guard sub-carriers, a virtual multi-carrier technique was proposed [1] to utilize larger size FFT (i.e. 2048) to receive the radio signals over adjacent RF carriers and the guard sub-carriers for post-processing.

The main objective of the design in [1] is expected to flexibly transmit/receive data over primary carrier, secondary carrier and guard sub-carriers, and it’s actually a reasonable design. The main potential concern on such transceiver architecture design is that it needs a symmetric frame structure layout as the example directed in [2], so that the receiver does not need to adjust its RF center frequency when utilizing additional secondary carriers and guard sub-carriers at both side. Unfortunately the legacy system does not follow such symmetric frame structure layout for system deployment and such requirement on symmetric frame structure layout may result in backward compatibility problem since 16e system is expected to be co-deployed with 16m system for a period of time. The potential concern on frame structure is explained by Fig.1.

Fig.1 Potential concern on the frame structure layout proposed in [2] due to inefficient backward compatibility

- Possible Transceiver Architecture for Multi-carrier Support

There are different kinds of transceiver architecture to realize multi-carrier operation for IEEE 802.16m, one of the examples is given in Fig.2.

Fig.2 Flexible MS transceiver architecture to support multi-carrier operation

Take Fig.2 for example, the transceiver may equip with multiple RF modules, multiple PHY and a common MAC entities. In response to different system operation, the transceiver can be flexibly configured for different transmission scheme. Fig.3 shows an example to support single carrier 4x4 MIMO transmission by multiplexing the MIMO signal from PHY1 to multiple RF modules as the most common case.

Fig.3 Configure the transceiver for single carrier 4x4 MIMO transmission

In addition, when multiple RF carriers are available in IEEE 802.16m system, the MS may also be configured to support 2x2 MIMO transmission scheme in two different RF carriers as shown in Fig.4. On the other hand, the MS may also be configured to support SISI transmission in four different RF carriers as shown in Fig.5.

Fig.4 Configure the MS transceiver for multi-carrier 2x2 MIMO transmission

Fig.5 Configure the MS transceiver for multi-carrier SISO transmission

For the examples from Fig.2 to Fig.5, this multi-carrier transceiver is actually an aggregation of multiple single-carrier transceivers. For this case, the bandwidth of each single carrier transceiver can be the same as legacy system for easy backward compatibility. When transmit/receive the radio signal over the guard sub-carriers, the RF center frequency can be adjusted to the guard sub-carrier location. Another example on the way to transmit/receive data over guard sub-carriers is given in C802.16m-08/561.

In summary, there may be many different kinds of bandwidth aggregation for the IEEE 802.16m multi-carrier MS. The current drawing in Fig.18 in P802.16m SDD is confusing the multi-carrier MS with multiple bandwidth aggregation with the single carrier MS. Therefore, the drawing shall be revised to prevent possible misleading.

- Text Proposal

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[Adopt the following figure to replace the Fig.18 in IEEE 802.16m-08/003r3]

Figure 18 Example of the proposed frame structure to support multi-carrier operation

------------------------------------------------------------End of the Text---------------------------------------------------------

Reference

[1] Hujin Yin et al, ”Virtual Multi-Carrier Operation for IEEE 802.16m,” S802.16m-08/364r1, May 2008.

[2] Sassan Ahmadi et al, “A Multi-Bandwidth Terminal Support Scheme for IEEE 802.16m Systems,” C802.16m-08/402r2, May 2008.