| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Multi-carrier Control Structure and Operation Principles with Handover Consideration |  |
| Date Submitted | 2008-7-7 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen and Paul Cheng MediaTek Inc. No.1, Dusing Rd. 1, HsinChu Science-Based Industrial Park, HsinChu, Taiwan 300, R.O.C. | Kelvin.Chou@mediatek.com IK.Fu@mediatek.com Yihshen.Chen@mediatek.com Paul.Cheng@mediatek.com |
| Re: | IEEE 802.16m-08/016, “Call for Contributions on Project 802.16m System Description Document (SDD)”. In response to the following topics: • Comments on the newly adopted and updated sections of the SDD as identified by the revision marks contained in IEEE 802.16m-08/003r2 |  |
| Abstract | This contribution presents several clarifications on the multi-carrier control structure and operation principles with handover consideration |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Multi-Carrier Control Structure and Operation Principles with Handover Consideration

Kelvin Chou, I-Kang Fu, Yih-Shen Chen and Paul Cheng

MediaTek Inc.

# Introduction

The current SDD has defined, mostly from the BS’s perspective, the concept of primary/secondary carrier and fully/partially configured carrier as the multi-carrier control structure, as well as several operation principles and scenarios corresponding to the control structure. However, from the MS’s perspective, these definitions and operation principles need to be further clarified when handover is being considered.

# Proposed Text for SDD

-------------------------------  Text Start  ------------------------------------------------------------------------------------

[Modify the definition of Primary carrier in Section 11.6.4 in IEEE 802.16m-08/003r3 as follows]

### 11.6.4 Multi-carrier Control Structure

The carriers involve in a multi-carrier system, from one MS point of view, can be divided into two types:

- Primary carrier is the carrier where the BS and the MS exchange traffic and full PHY/MAC control information defined in 16m specification. Further, the primary carrier is in charge of delivering all control information for proper MS operation. Each MS shall have only one primary carrier with respect to its serving (anchor) BS.

[Modify Section 19.1 in IEEE 802.16m-08/003r3 as follows]

## 19.1 Multi-carrier operation Principles

The following is common in all modes of multi-carrier operation:

- The system BS defines N standalone primary RF carriers as defined in section 11.x.6 11.6.4, each fully configured with all synchronization, broadcast, multicast and unicast control signaling channels. Each MS in the cell is connected to and its state being controlled through only one of the primary carriers.

- In the multicarrier operation a common MAC can utilize radio resources in one or more of the secondary carriers as defined in section 11.x.6 11.6.4, while maintaining full control of MS mobility, state and context through the primary carrier.

- Some information about the secondary carriers including their presence and location shall be made available to the user through the primary carriers. The primary carrier may also provide user the information about the configuration of the secondary carrier.

- The resource allocation can span across multiple RF carriers. Link adaptation feedback mechanisms would need to incorporate measurements relevant to both primary and secondary carriers.

- The multi-carrier may be used in the downlink and/or uplink asymmetrically based on system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

- A primary RF carrier may dynamically utilize resources across multiple secondary RF carriers. Multiple primary RF carriers may also share the same secondary carrier.

- The multiple carriers may be in different parts of the same spectrum block or in non-contiguous spectrum blocks.

- Each user will be connected to only one primary carrier with respect to its serving (anchor) BS. A secondary carrier for a MS, if fully configured, may serve as primary carrier for other MS’s.

There are two scenarios to multicarrier deployment.

Scenario 1: All carriers in the system are fully configured to operate standalone and may support some users as their primary carrier.

Scenario 2: In this case, in addition to fully configured and standalone RF carriers the system also utilizes additional supplementary radio carriers optimized as data pipes for certain services or traffic types using limited control signaling capability. Such supplementary carriers may be used only in conjunction with a primary carrier and cannot operate standalone to offer IEEE 802.16m services for a MS.

In multi-carrier operation, MS can access multiple carriers. The following multi-carrier operations are identified:

- Carrier aggregation

  - MS shall always maintain its physical layer connection and monitor the control information on the primary carrier.

- Carrier switching

  - MS can switch its physical layer connection from the primary to the secondary carrier per BS’ instruction. When the MS is connected to the secondary carrier, the MS doesn’t need to maintain its physical layer connection to the primary carrier.

  - This mode may be used for the cases of single radio MS or non-contiguous spectrum.

  - MS may request or be requested by its serving BS to relocate to another primary carrier of the same serving BS.

-------------------------------  Text End  ------------------------------------------------------------------------------------