| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power Control Based on Interference Management for Uplink |  |
| Date Submitted | 2008-07-10 |  |
| Source(s) | Zhaohua Lu, Ying Liu, Peng Hao，Kun Liu, Xiangyu Liu, Dengkui Zhu, Yanfeng Guan, Rui Kang ZTE Corporation | lu.zhaohua@zte.com.cn |
| Re: | IEEE 802.16m-08/024 Call for Contributions on Power Control |  |
| Abstract | In this contribution we propose consideration on power control based on interference management for uplink. |  |
| Purpose | For TGm discussion and adoption on the proposed text input for P802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power Control Based on Interference Management for Uplink

- Zhaohua Lu, Ying Liu, Peng Hao,Kun Liu, Xiangyu Liu, Dengkui Zhu, Yanfeng Guan, Rui Kang

- ZTE Corporation

- Introduction

Power control is a key technology for 802.16m to meet the performance requirement specified in SRD. Therefore, power control based interference management will be the key to increase the capacity of IEEE 802.16m OFDMA system.

802.16m OFDMA system can concurrently communicate with multiple MSs on the uplink. Multiple MSs may simultaneously transmit data on the uplink. This is often achieved by multiplexing the transmissions on each link to be orthogonal to one another in time, frequency domain.

On the uplink, the transmissions from MSs communicating with different BSs are typically not orthogonal to one another. Consequently, each MS may cause interference to other MSs communicating with nearby BSs and may also receive interference from these other MSs, as shown in Figure1. The performance of each MS is degraded by the interference from the other MSs communicating with other BSs.

- Fig.1 An example of inter-cell interference for uplink

Therefore there is a need for power control to mitigate interference for uplink in a wireless communication system.

- Uplink Power Control Based on Interference management

In order to manage uplink inter-cell and intra-cell interference, power control for uplink should consider other BSs’ uplink interference level except its serving BS.

The basic goal of uplink inter-cell power control is to control the bad impact of inter-cell interference on system performance to an acceptable level. Uplink interference indication message can be exchanged between BSs by MSs and/or RSs which are communicating with multiple BSs, then BS direct MSs to adjust transmit power according to other BSs’ uplink interference level.

- Proposed Text for SDD

---------------------------------------------------------Start of the Text-------------------------------------------------------

x.x.x Uplink Power Control

Uplink power control should consider BSs’ uplink interference level to manage inter-cell and intra-cell interference.

The basic goal of uplink inter-cell power control is to control the bad impact of inter-cell interference on system performance to an acceptable level. Uplink interference indication message can be exchanged between BSs by MSs and/or RSs which are communicating with multiple BSs, then BS direct MSs to adjust transmit power according to other BSs’ uplink interference level.

----------------------------------------------------------End of the Text-------------------------------------------------------