IEEE C802.16m-08/587

| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 2D-POD for MIMO with Interference Mitigation |  |
| Date Sub. | 2008-07-07 |  |
| Source(s) | Kiran Kuchi J Klutto Milleth Vinod Ramaswamy Baskaran Dhivagar K Giridhar Bhaskar Ramamurthi CEWiT, India | kkuchi@cewit.org.in klutto@cewit.org.in vinodr@cewit.org.in dhiv@cewit.org.in giri@tenet.res.in bhaskar@tenet.res.in |
| Re: | IEEE C802.16m-08/024 - Call for Comments and Contributions on Project 802.16m System Description Document (SDD), on the topic Uplink MIMO schemes and interference mitigation. |  |
| Abstract | Open-loop 2D-POD single stream and multiple stream MIMO schemes for 2 and 4 transmit antennas are proposed for IEEE 802.16m system. This scheme when used in a single stream mode can mitigate interference of low SINR cell edge users in an efficient manner using the existing receivers in a FFR zone. For 4 transmit antennas, this scheme combined with STBC/SFBC/SM can be used for high SINR cell centre users in a different FFR zone. |  |
| Purpose | To be discussed by TGm for incorporation into the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

2D-POD for MIMO with Interference Mitigation

Kiran Kuchi, J Klutto Milleth, Vinod Ramaswamy, Baskaran Dhivagar, K Giridhar, Bhaskar Ramamurthi

CEWiT, Chennai, India

Introduction

It’s a well known fact that spectrum is a scarce resource, and has to be used in an efficient manner. Therefore, it is inevitable to reuse the available spectrum in every sector commonly known as reuse 1 deployment. Also, it is not uncommon to see deployments with a typical cell of radius as small as 200/300 meters. In this scenario, interference is the major concern, and therefore, the MIMO schemes proposed for 16m should not target only on diversity. It should consider schemes that provide interference mitigation gain along with diversity gain without increasing the receiver complexity or otherwise using the existing receivers available in the legacy 16e systems.

Proposed Diversity Techniques

Two dimensional phase offset diversity (2D-POD) schemes are proposed for single and multiple streams that give significant interference mitigation gain along diversity gain without making any modification to the existing receivers.

- In IEEE 802.16m data is allocated in groups of resource blocks/slots. For example, a slot is defined as 18 subcarriers and 6-OFDM symbols

- The following matrix-A with 2D-POD is proposed.

- Data in each slot is multiplied with a distinct phase rotation and transmitted through second antenna

A=skskejΦ(p,q)Ant1Ant2Φ(p,q)=Φ(p)+Φ(q)

(p,q)

is the slot index in Frequency-Time Plane

- Φ(q)             Time dependent phase offset

- Φ(p)              Frequency dependent phase offset

- Slot wise phase rotations are optimized to maximize the diversity advantage

  - The phase rotations maximally de-correlates channel

  - 2D-POD outperforms both CDD and STBC/SFBC

  - The matrix A can be generalized to arbitrary number of transmit antennas

MIMO Support for Interference Mitigation

The following table provides the interference suppression feasibility with OL/CL MIMO

| MIMO Mode used by the Desired Signal | MIMO Mode used by CCI | Whitened MLD Receiver Support | MMSE Receiver Support |
| --- | --- | --- | --- |
| CL/POD | CL/POD | Yes | Yes |
| STBC | STBC | No | Yes, but inferior performance |
| CL/POD | STBC | No | No |
| STBC | CL/POD | No | Yes |

- Interference suppression not feasible when CL and STBC OL users cause mutual interference

- Users can switch between 2D –POD and single stream CL-MIMO with full IC support

Other Advantages

The proposed 2D-POD scheme has the following advantages

- No feedback overhead, because of being an open-loop (OL) scheme

- Sub-frames can be regular or irregular

- Achieves the diversity gain available in the system

- Significant interference mitigation gains

  - Cancels at least two interferers using MLD receivers available in the 16e systems

- Good trade-off between diversity and interference mitigation gains

- Reduced pilot overhead or better channel estimation using same pilot overhead

- Can coexist with single stream open-loop and closed-loop schemes

  - Switching between 2D–POD and single stream CL-MIMO with full IC support possible

- More suitable for low SINR cell edge users

- When combined with STBC/SFBC/SM using 4 transmit antennas can be used for high SINR users as well.

# ------------------------------------------- Proposed SDD Text --------------------------------

The following text is proposed for SDD

- 2D-POD scheme is the baseline transmit diversity scheme in a single stream MIMO mode, which can mitigate interference of low SINR cell edge users in an efficient manner using the existing receivers in a FFR zone. For 4 transmit antennas, this scheme combined with STBC/SFBC/SM will be used for high SINR cell centre users in a different FFR zone.