| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Expedited Handover Cancel Procedure |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Yuji Kojima, Keiichi Nakatsugawa, Masato Okuda Fujitsu | E-mail: {kojima.yuuji, nakatsugawa, okuda}@jp.fujitsu.com |
| Re: | IEEE 802.16m-08/024,” Call for Comments and Contributions on Project 802.16m System Description Document (SDD)”. Target topic: Mobility Management |  |
| Abstract | This contribution provides the method of expediting a handover cancel procedure. |  |
| Purpose | This contribution provides the method of expediting a handover cancel procedure. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Expedited Handover Cancel Procedure

Yuji Kojima, Keiichi Nakatsugawa, Masato Okuda

Fujitsu

# 1. Introduction

In IEEE 802.16m System Requirements Document [1], the reduction of the handover latency is required. Also, in System State Transition Diagram of Draft IEEE 802.16m SDD [2], Fast Network Re-entry procedure after handover is defined.

This contribution provides the method of expediting a handover cancel procedure.

# 2. Methods of expediting Handover Procedure

In general, the method of the reduction of the handover latency is categorized as the following.

- Omit the messages exchanged during handover due to sharing the context related to the MS between the Serving BS and the Target BS.

- Pre-allocate the dedicated resource for MS's network re-entry during the handover preparation phase before the handover action phase.

In the case of handover procedure from the Serving BS to the Target BS, regarding to the former item, HO (handover) process optimization (omitting SBC-REQ/RSP, PKM phase, REG-REQ/RSP, etc) is defined in IEEE Std. 802.16e-2005 [3]. In addition, regarding to the latter item, the allocation method of a non-contention-based ranging opportunity for the MS (i.e. Fast_Ranging_IE) is defined.

In the other hand, in the case of handover cancel procedure by which the MS attempting HO can return to the Serving BS, regarding to the former item, the HO cancel message (i.e. MOB_HO-IND message with the HO cancel option) is defined to allow the MS to resume normal operation communication without the other messages exchanged. However, regarding to the latter item, IEEE Std. 802.16e-2005 [3] doesn’t provide the corresponding method such as the above non-contention-based ranging opportunity. The MS is required to perform the bandwidth request ranging for sending the HO cancel message, as shown in Fig.1. This issue should've been considered. It should be fixed in the 802.16m amendment.

Fig.2 shows an example of the proposed HO cancel procedure, but doesn't restrict the other solution of the expedited HO cancel procedure.

| Fig.1. HO cancel procedure in the case of 802.16e | Fig.2. Example of the proposed HO cancel procedure |
| --- | --- |

# 3. Proposed Texts

[Insert the following text into a suitable place of Section representing Mobility Management]

-------- Start of the text proposal ---------------------------------------------------------------------------------------

IEEE 802.16m supports the handover cancel procedure by which the MS attempting HO can resume normal operation communication to the Serving BS without the exchange of any messages except the HO cancel message. This handover cancel procedure can include allocating the dedicated resource for the MS to send the HO cancel message.

-------- End of the text proposal ---------------------------------------------------------------------------------------

# References

[1] IEEE 802.16m System Requirements, IEEE 802.16m-07/002r4

[2] The Draft IEEE 802.16m System Description Document, IEEE 802.16m-08/003r3

[3] IEEE Std. 802.16e-2005, IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands, and IEEE Std. 802.16-2004/Cor1-2005, Corrigendum 1, December 2005