| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Symbol Vector Interleaving for Chase-Combining HARQ |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Yakun Sun, Jungwon Lee Marvell Semiconductor Inc. | E-mail: yakunsun@marvell.com |
| Re: | IEEE 802.16m-08/016 – Call for Contributions on Project 802.16m System Description Document (SDD). Specifically, HARQ PHY. |  |
| Abstract | Symbol vector interleaving in frequency and spatial domain across HARQ transmissions improves chase-combining performance, and enable the optimal symbol-level combining for HARQ. |  |
| Purpose | To be discussed and adopted by TGm for use in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Symbol Vector Interleaving for Chase-Combining HARQ

Marvell Semiconductor Inc.

# Introduction

Chase-combining as a simple and effective HARQ scheme should be considered for IEEE802.16m, and it is already specified in 16e legacy systems. Chase-combining HARQ, however, may not be very efficient when the channel varies slowly and the time-diversity is limited. We propose a symbol vector interleaving for chase-combining for slow-varying channels, which enables the optimal symbol-level combining for HARQ at the same time.

# HARQ with Symbol-level Combining for DL MIMO

In this section, we first briefly introduce symbol level combining. As a duplicate copy of signals is transmitted after the first attempt is failed, the mobile can have different approaches to combine the multiple copies of signals. A typical approach is to combine the log-likelihood-ration (LLR) of the corresponding bits, or bit-level combining (BLC). Although optimal for SISO channels, bit-level combining is clearly suboptimal in MIMO channels. Symbol-level combining, as shown in Figure 1, has been proved to be optimal for MIMO HARQ, which is equivalent to bit-level combining for SISO channels.

Figure 1. Symbol-level combining for MIMO channels

Assume the signal  is transmitted using chase-combining HARQ, where the received signal from the i-th retransmission is, with MIMO channel. The symbol level combiner represented the received signals at the N-th retransmission as

where is the equivalent MIMO channel after the pre-processor for the space-time codes (e.g., reorder and stack Hi for STBC). Symbol-level combining has been published in [2][3].

Figure 2 compared bit-level combining and symbol-level combining performance for chase-combining HARQ. Assume a VA-120km/h channel. STBC (Matrix-A) is used over a 2x2 MIMO channel with high correlation. The transmit signal is QPSK and rate-1/2 coded. Symbol-level combining shows a 2.5dB gain over bit level combining for 2 transmissions.

Figure 2. Link performance comparison of bit-level combing and symbol-level combining for VA-120km/h, 2x2 STBC.

# Symbol Vector Interleaving for Chase Combining

For MIMO chase combining retransmissions, the coded symbol vectors can be passed through a symbol interleaving to exploit frequency selectivity. Explicitly, a reordered symbol vector-to-subcarrier mapping is used for retransmissions. After a symbol deinterleaver at the receiver end, the symbol-level combining can be deployed for optimal demodulation and decoding. The system is given in Figure 3.

Figure 3. Symbol interleaving for Chase Combining

The symbol vector interleaving can be employed at different resolution. For example, a simple interleaver just rotates the allocated RU’s in the retransmissions. More complicated interleavers include randomly rearranging symbols vectors to subcarrier mapping. The complexity of symbol interleaver is minor, but it efficiently exploits the frequency selectivity and improves the performance at the help of symbol-level combining.

Figure 4 shows the performance with symbol-level combining and symbol interleaving with chase-combining HARQ. Assume an ITU PedB-3km/h channel. Spatial multiplexing (Matrix-B) is used over a 2x2 MIMO channel with high correlation. The transmit signal is 16QAM and rate-3/4 coded. Suppose AMC allocation is used, and all subcarriers are assigned to a mobile. Retransmission occurs after two 5ms frames. Symbol interleaving is implemented in two fashions: a logical band shift, or a random rearrangement across 2 logical bands. Symbol-level combining (SLC) shows a 2dB gain over bit level combining (BLC) for 2 transmissions, and symbol interleaving provides 2dB gain in addition.

Figure 4. Performance of symbol level combining and symbol interleaving for CC HARQ

The symbol vector interleaver is implemented in frequency domain in Figure 3. It can be further generalized in spatial domain. Namely, a different space-time coding scheme will be employed on the duplicate symbol vector at retransmissions.

Figure 5. General symbol-level interleaver for CC HARQ

A simple example is to construct STBC across HARQ transmissions [4][5]. Namely, on a subcarrier, the symbol vector is transmitted, and in the 2nd transmission, the alamouti-coding pair is transmitted. Symbol-level combining will treat retransmissions as STBC (Matrix-A). Figure 6 shows the performance with symbol-level combining and spatial symbol interleaving (HARQ STBC) with chase-combining HARQ. The same simulation parameters as Figure 4 are reused here. Spatial symbol vector interleaving provides 4dB gain.

Figure 6. Performance of spatial interleaver for CC HARQ

More complicated spatial symbol vector interleaving includes applying different precoding matrix across retransmissions and etc.

# Conclusions

The optimal receiver for MIMO chase-combining HARQ is symbol-level combining. To improve the performance of chase-combining over slow varying channels and employ symbol-level combining at the same time, a symbol vector interleaving concept is proposed, to rearrange the coded symbol vectors in frequency and spatial domain for HARQ retransmissions.

# References

- IEEE 802.16m System Requirements IEEE 802.16m-07/002r4

- “Performance comparison for bit level and symbol level Chase combining”, Texas Instruments, 3GPP R1-01-0472, May 21-25, Busan, Korea.

- "Optimal Combining Schemes for MIMO Systems with Hybrid ARQ," E. W. Jang, J. Lee, H.-L. Lou, and J. M. Cioffi, in Proceedings of IEEE International Symposium on Information Theory, June 2007, Nice, France, pp. 2286-2290.

- IEEE 802.16-2005: IEEE Standard for Local and Metropolitan Area Networks – Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – Amendment 2: Physical Layer and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands, Feb. 2006.

- IEEE C802.16e-04/113r2, Notel Networks, “Soft packet combining for STC re-transmission to improve HARQ performance in MIMO mode”, July 2004.

Although chase combining is discussed in this contribution, the same concept of symbol-level combining can be applied with Incremental Redundancy (IR).