| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Analysis on Staggered Super-Frame Structure |  |
| Date Submitted | 2008-07-10 |  |
| Source(s) | Xiaoyi Wang, Adrian Boariu, Shashikant Maheshwari, Yousuf Saifullah, Xin Qi Nokia Siemens Networks Jianmin Lu, Yunsong Yang, Zhigang Rong, Sean McBeath Huawei Dong Xiaolu,Du Ying CATR Qu Hongyun, Fang Huiying ZTE Corporation | E-mail: xiaoyi.wang@nsn.com lujianmin,yunsongyang@huawei.com dongxiaolu,duying@mail.ritt.com.cn qu.hongyun,fang.huiying@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/024: Call for comment on SDD |  |
| Abstract | This contribution proposes to remove the staggered Super-Frame structure currently described in the 802.16m SDD |  |
| Purpose | To be discussed and adopted by TGm for incorporation in the IEEE 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Analysis on Staggered Super-Frame Structure in SDD

Xiaoyi Wang, Adrian Boariu, Shashikant Maheshwari, Yousuf Saifullah, Xin Qi

Nokia Siemens Networks

Jianmin Lu, Yunsong Yang, Zhigang Rong, Sean McBeath

Huawei

Dong Xiaolu,Du Ying

CATR

Qu Hongyun, Fang Huiying

ZTE Corporation

# Background

The current IEEE 802.16m SDD defines a staggered super-frame structure. The staggering of super-frames is applied to a cluster of 4 neighbor cells, BS1-BS4.

# Performance Analysis

- Staggered structure doesn’t provide any gain for SFH reception
Signals from other BSs are interference, it doesn’t matter if they are data or SFH encoded inside a signal. Figure 1 shows the arrival of signals at the MS.

- Staggered structure doesn’t increase Neighbor-cell measurement opportunities
Neighbor-cell measurement is actually preamble detection. However, a staggered preamble doesn’t provide more opportunities for neighbor-cell measurement, since the MS can detect all the preambles at one time if they are aligned.

- Staggered super-frame structure doesn’t mean more scanning opportunities for HO ranging. 
Scanning opportunity is related to ranging area. Staggering of the ranging area, not frame, could provide more ranging opportunities. See figure 2.

- If the preamble is also staggered, SINR measurement is not reliable. 
In the staggered structure, when the MS measures preamble from any BS, the interference is data from other BSs. However, data is not always transmitted (unlike preamble). So, the SINR measurement is not accurate in the staggered structure.

- Staggered structure results in unnecessary complexity for MS
If the preamble is staggered, when 16m MS enter the network, it first searches the preamble. It is difficult for the MS to estimate which is the best preamble in the staggered structure, since the preambles arrived at different times and therefore may  have different fading. However, in an aligned structure, 16m MS obtain the preamble from all BSs at the same time, and the fading channels are much more coherent, so the MS can compare the strength and obtain the strongest preamble immediately.

- Staggered structure can not utilize macro diversity. 
In “aligned” superframe structure, macro diversity could be used to improve spectral efficiency and reliability of the transmission of PBCH. This contradicts section 11.6.2.2.1

Conclusion: staggered super frame structure brings no gain but complexity.

Figure 1.Example of preamble reception in staggered structure

Figure 2.Example of Staggered ranging channels

# Recommendation

Delete the section of staggered super frame structure.

# Proposed Text for SDD

Delete section 11.4.7.