| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Baseline Content on the Synchronization Channel for the 802.16m SDD |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Paul Cheng Chris Williams Project 802.16m Preamble Rapporteur Group Chairs | Paul.Cheng@Mediatek.com Chris.Williams@uk.Fujitsu.com |
| Re: | IEEE 802.16m-08/023, Charter and Scope of New TGm Rapporteur Groups |  |
| Abstract | Consolidated SDD text on the Preamble based on contributions submitted to 802.16m in session #55 and the 802.16m Rapporteur Group for Preamble design is provided in this contribution. |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Baseline Content on the Synchronization Channel

for the 802.16m SDD

Paul Cheng, Chris Williams

Project 802.16m Preamble Rapporteur Group Chairs

Introduction

This is the final (fourth) draft on Synchronization Channel (SCH) design towards developing SDD text for IEEE 802.16m. Comments on this draft are requested using the commentary database by 9th July 2008 (AOE). The RG Chairs will issue a compiled commentary database shortly after this deadline, and reply comments are due by noon 14th July (Denver local time).

Text does not highlight the proposed text that has a significant level of support, and is the default option. Green highlights show text for alternative options, one of which will be adopted based on sufficient support in the plenary meeting. Yellow highlights denote aspects requiring resolution, and comments are requested on these items. Blue highlights are the editors’ comments.

# Physical Layer

## Downlink Control Structure

### Downlink Control Information Classification

### Transmission of DL control information

#### Synchronization Channel

The synchronization channel is a DL physical channel which provides a reference signal for time, frequency and frame synchronization, RSSI estimation, BS/femtocell identification and sector identification for system acquisition, handover and super-frame header synchronization.

[Option – The SCH may also carry control information.]

##### Preamble requirements

[Editors’ comment: The text in this section has been moved to document C80216m-08_636 or later revision for further discussion and harmonisation.]

##### Preamble architecture

###### Overview

            - Hierarchy

Two structures are possible:

- Non-hierarchical, where only a single type of 16m synchronization symbol exists, which may be in addition to the legacy 16e preamble. Where a new 16m SCH is defined, the 16e preamble is not required as part of the SCH functionality (as defined in the SCH Requirements).

- Hierarchical, where more than one type of SCH symbol exists within the superframe. They may or may not use the legacy 16e preamble as one level of hierarchy.

Different Options

Option 0. The decision of hierarchy or non-hierarchy structure of the preamble is FFS, which will be decided in stage 3.

Option 1. No hierarchy of synchronization channel (SCH) exists. The SCH shall be transmitted with a span of one OFDM symbol through multiple transmit antennas, in order to facilitate time- and frequency-synchronization and cell-ID detection..

Option 2. Two levels of synchronization hierarchy exist. These are called the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH). The P-SCH is used for initial acquisition. The S-SCH is used for fine synchronization, cell/sector identification (ID), and channel measurements. Optional extension, P-SCH is common across a network deployment but has small number of options to signal system information (e.g. mixed or 16m-only deployment, channel bandwidth, etc. Detail is FFS). The use of P-SCH to carry ID information is also FFS.

            - Multiplexing

Synchronization symbols are TDM

            - Number of symbols in SCH

A complete instance of the SCH exists within a superframe. Multiple symbols within the superframe may comprise the SCH.

In mixed deployments, the presence of the legacy preamble is implicit.

Different Options

Option 0. The number of synchronization channel in each superframe is FFS, which will be decided in stage 3.

Option 1. There is one 16m synchronization symbol per frame

Option 2. There is/are [One|two] 16m synchronization symbol(s) per superframe

Option 3. [Where hierarchy exists] the P-SCH is [once|twice|four times] per superframe, S-SCH is once per frame, which may be a specific 16m S-SCH (legacy preamble is unused) or the legacy preamble forms the S-SCH.

            - Location of synchronization symbols

In mixed deployments, the presence of the 16e preamble in the first symbol of the 16e frame is implicit. The location of the SCH symbol(s) is fixed within the superframe.

[Where a SCH hierarchy exists, the separation between the P-SCH and S-SCH is fixed.]

Different Options

Option 1. The preamble is transmitted in the first subframe at the beginning of every [frame|superframe] with a span of one OFDM symbol

Option 2: Removed

Option 3. The SCH is within the SFH (location FFS) only (no reuse of legacy preamble)

Option 4. P-SCH is in the SFH (location is FFS), S-SCH is at legacy preamble location for mixed deployments and FFS in 16m only deployment;

            - Properties of P-SCH & S-SCH (if they exist)

[Editors’ note: This section only applies if the P-SCH/S-SCH hierarchy is agreed. Square bracketed items are for further discussion.]

The P-SCH has these properties:

  - Common to all sectors/cells

  - [Optionally supports limited signalling (e.g. mode, bandwidth, etc)]

  - Fixed bandwidth (actual bandwidth FFS)

  - [Carries ID information (e.g. femto ID) - FFS]

The S-SCH has these properties:

- [Same bandwidth as P-SCH | Full bandwidth]

- Carries ID information

###### Description of legacy support/reuse

16m system will exist in both greenfield and mixed (coexisting 16e and 16m equipment) deployments. In mixed deployments the 16e preamble will always be present. As discussed in the requirements, the 16m SCH is not to degrade the performance of legacy acquisition.

Different Options

Option 1. Only the 16e preambles are used as the synchronization channel.

For the next 2 options include this text as introduction:

The IEEE 802.16m SCH shall support the IEEE 802.16m MSs to perform time, frequency and frame synchronization procedures without requiring the legacy preamble. In mixed deployments, the IEEE 802.16m SCH and the legacy preamble shall be transmitted in distinct OFDM symbols of the super-frame.

[The time offset between the legacy 16e preamble symbol and 16m SCH is fixed.]

Option 2. The 16m system uses both a dedicated 16m synchronization symbol and the legacy 16e preamble symbols for DL synchronization.

Option 3. As option 2 for mixed deployment, but in Greenfield 16m system only 16m specific synchronization symbols are used (no reuse of legacy preamble is required).

###### Cell ID support

The number of IDs is at least [TBD].

Editors’ note: Value depends on agreement on the Requirements document (latest revision of C80216m-Preamble-08_019)

Sectors are distinguished by the synchronization channel; the number of sectors is [FFS].

Different Options

Option 1. The ID is derived from a single symbol [per frame|superframe] (without hierarchy).

Option 2. The ID is derived only from the S-SCH, which may comprise more than one symbol.

Option 3. The ID is derived from a combination of P-SCH and S-SCH.

###### Multicarrier and multi-bandwidth support

The SCH symbol is designed for the minimum channel bandwidth (5MHz). The total bandwidth available is the sum of the bandwidths of the individual channels (see figure below).

Different Options

Option 0. The location of the SCH in frequency is FFS

Option 1. To support multicarrier operation or wider bandwidths, the 16m [P-]SCH is centered in the middle of the total bandwidth.

Option 2. To support multicarrier operation, the [P-]SCH is on only one of the carriers (which one is FFS).

Option 3. To support multicarrier operation or wider bandwidths, the [P-]SCH is concatenated in frequency across all the carriers.

(See examples below)

Options for the location of SCH (shaded) for multicarrier operation.

In the case of hierarchical SCH, additionally define:

Option 4. To support multicarrier operation or wider bandwidths, the 16m S-SCH is centered in the middle of the total bandwidth.

Option 5. To support multicarrier operation, the S-SCH is on only one of the carriers (which one is FFS).

Option 6. To support multicarrier operation or wider bandwidths, the S-SCH is concatenated in frequency across all the carriers.

###### MIMO support and channel estimation

MIMO support is FFS depending on the content of other aspects of the SDD, such as for MIMO and interference mitigation. Where employed, MIMO support is achieved by transmitting SCH subcarriers from known antennas. Antennas are:

- Cyclic delay diversity (with antenna specific delay values

- Interleaved either within a symbol (multiple antennas can transmit within a single symbol but on distinct subcarriers)

- Across frames (only one antenna transmits in each symbol)

- Or some combination – actual approach is FFS.

The number of BS antennas supported for MIMO channel measurements is FFS, depending on the requirements of other 16m SDD content, such as DL MIMO and interference mitigation.

##### Preamble Sequence Design Properties

The SCH shall enable acquisition by autocorrelation. Use of cross-correlation is left to the implementation.

The power of synchronization channel can boosted to a [TBD] level.

The SCH (non-hierarchical) or P-SCH (hierarchical) is mapping with every [FFS] subcarriers on the frequency domain.

The S-SCH (hierarchical) is mapping with every [FFS] subcarriers on the frequency domain.