| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Baseline Content on the Synchronization Channel for the 802.16m SDD |  |
| Date Submitted | 2008-07-017 |  |
| Source(s) | Paul Cheng Chris Williams Project 802.16m Preamble Rapporteur Group Chairs | Paul.Cheng@Mediatek.com Chris.Williams@uk.Fujitsu.com |
| Re: | IEEE 802.16m-08/023, Charter and Scope of New TGm Rapporteur Groups |  |
| Abstract | Consolidated SDD text on the SCH based on contributions submitted to 802.16m in session #55 and the 802.16m Rapporteur Group for Preamble design is provided in this contribution. |  |
| Purpose | Revision document of C80216m-08_634 based on the agreed commentary from Preamble RG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Baseline Content on the Synchronization Channel

for the 802.16m SDD

Paul Cheng, Chris Williams

Project 802.16m Preamble Rapporteur Group Chairs

Introduction

This is the final (fourth) draft on Synchronization Channel (SCH) design towards developing SDD text for IEEE 802.16m. Comments on this draft are requested using the commentary database by 9th July 2008 (AOE). The RG Chairs will issue a compiled commentary database shortly after this deadline, and reply comments are due by noon 14th July (Denver local time).

Text does not highlight the proposed text that has a significant level of support, and is the default option. Green highlights show text for alternative options, one of which will be adopted based on sufficient support in the plenary meeting. Yellow highlights denote aspects requiring resolution, and comments are requested on these items. Blue highlights are the editors’ comments.

# Physical Layer

## Downlink Control Structure

### Downlink Control Information Classification

### Transmission of DL control information

#### Synchronization Channel

The synchronization channel (SCH) is a DL physical channel which provides a reference signal for time, frequency, and frame synchronization, RSSI estimation, channel estimation, and BS/femtocell identification and sector identification for system acquisition, and handover and super-frame header synchronization. The SCH supports supplementary functionalities such as channel estimation and RSSI measurement.

##### Synchronization channel requirements (Informative)

DEFINITIONS

| Convergence time | Time interval for the probability of error in SCH index detection to be less than [1] % under non-ideal assumptions on the timing and carrier synchronization, measured from the start of the acquisition process. |
| --- | --- |
| Correct detection | Choose a BS among the co-channel BS’s whose received powers averaged over the convergence time are within [3] dB of the BS with the highest received power |
| Coverage area | Area where the false detection probability is less than [1] % within the convergence time |
| Overhead | Total radio resources (time and frequency) per superframe that can not be used for other purpose because of SCH |
| Cell ID set | The cell ID set is the set of unique SCH symbols for differentiating between macrocell/femtocell/sector/relay transmitters |
| Multi-bandwidth support | Design of SCH for different bandwidths as specified in Table 2 of SDD (C802.16m-08_003r2, or latest revision) |
| Multi-carrier support | Design of SCH to support functionality described in SDD (C802.16m-08_003r2, or latest revision) Sections 8.1.3 & 11.4.1.3 |

11.x.2.1.1.1 Overhead

Mixed mode with legacy system

In mixed mode operation the SCH overhead shall be less than or equal to [ 4] % per super frame including the legacy preamble, where the [4]% is calculated based on the ratio of SCH resource and that of usable resource for transmitting data.

16m only mode

In 16m only mode operation the SCH overhead shall be less than or equal to [2] % per super frame, where the [2]% is calculated based on the ratio of SCH resource and that of usable resource for transmitting data.

11.x.2.1.1.2 Synchronization

The SCH will provide synchronization for:

- Time, including frame and superframe

- Frequency (up to [20]ppm initial tolerance of carrier frequency)

Synchronization performance must at least match that of 802.16e.

16m SCH must enable system acquisition without knowledge of the full channel bandwidth.

The 16m SCH shall not cause degradation of synchronization for coexisting legacy systems. Synchronization shall be robust for the full range of required mobile velocities as defined in the SRD (i.e. up to 350km/hr).

11.x.2.1.1.3 Coverage

The coverage area of IEEE 802.16m SCH shall not be worse than the minimum of the required coverage for broadcasting channel, control channel and unicast data channel at channel conditions under considerations.

11.x.2.1.1.4 Cell IDs

The cell ID shall be obtained from the SCH. To support femto-cell deployments, the number of unique cell IDs conveyed by the SCH shall be greater than or equal to FFS

11.x.2.1.1.5 MIMO support and channel estimation

IEEE802.16m SCH may support multi-antenna transmissions. The number of supported antennas is FFS.

Channel estimation supported from the SCH is FFS.

11.x.2.1.1.6 Multi-carrier Multi-bandwidth support

IEEE 802.16m SCH shall support multi-bandwidth and multi-carrier operations as defined in the latest revision of the SDD.

11.x.2.1.1.7 Measurement Support

IEEE 802.16m SCH shall support noise power estimation.

11.x.2.1.1.8 Sequence requirements

The PAPR and peak power shall be no larger that than those of the downlink signal (excluding SCH).

Synchronization channel architecture

###### Overview

            - Hierarchy

Two structures are possible:

- Non-hierarchical, where only a single type of 16m synchronization symbol exists, which may be in addition to the legacy 16e preamble. Where a new 16m SCH is defined, the 16e preamble is not required as part of the SCH functionality (as defined in the SCH Requirements).

- Hierarchical, where more than one type of SCH symbol exists within the superframe. They may or may not use the legacy 16e preamble as one level of hierarchy.

The decision of hierarchy or non-hierarchy structure of the synchronization channel is FFS.

[Option 1. No hierarchy of synchronization channel (SCH) exists. The SCH is transmitted with a span of one OFDM symbol through multiple transmit antennas, in order to enable time- and frequency-synchronization and cell-ID detection.]

[

Option 2. Two levels of synchronization hierarchy exist. These are called the primary synchronization channel (P-SCH) and secondary synchronization channel (S-SCH). The P-SCH is used for initial acquisition. The S-SCH is used for fine synchronization, and cell/sector identification (ID). The use of P-SCH to carry ID information is also FFS.]

            - Multiplexing

Synchronization symbols are TDM

            - Number of symbols in SCH

A complete instance of the SCH exists within a superframe. Multiple symbols within the superframe may comprise the SCH.

In mixed deployments, the presence of the legacy preamble is implicit.

The number of synchronization channel symbols in each superframe is FFS.

            - Location of synchronization symbols

In mixed deployments, the presence of the 16e preamble in the first symbol of the 16e frame is implicit. The location of the SCH symbol(s) is fixed within the superframe.

The location of SCH is FFS. [Where a SCH hierarchy exists, the time separation between the P-SCH and S-SCH is fixed.]

            - Properties of P-SCH & S-SCH (if they exist)

[Editors’ note: This section only applies if the P-SCH/S-SCH hierarchy is agreed. Square bracketed items are for further discussion.]

The P-SCH has these properties:

•	Common to all sectors/cells

•	[Optionally supports limited signalling (e.g. mode, bandwidth, etc)]

•	Fixed bandwidth (5MHz or less)

•	[Carries ID information (e.g. group ID, femto ID) - FFS]

The S-SCH has these properties:

- [Same bandwidth as P-SCH | Full bandwidth]

- Carries ID information

###### Description of legacy support/reuse

16m system will exist in both greenfield and mixed (coexisting 16e and 16m equipment) deployments. In mixed deployments the 16e preamble will be always present. As discussed in the requirements, the 16m SCH is not to degrade the performance of legacy acquisition.

The 802.16m SCH shall enable 802.16m MSs to synchronize in frequency and time without

requiring the legacy preamble.

###### Cell ID support

The number of IDs is at least [TBD].

Sectors are distinguished by the synchronization channel; the number of sectors is [FFS].

###### Multicarrier and multi-bandwidth support

The location of the SCH in frequency is FFS

###### MIMO support and channel estimation

Where employed, MIMO support is achieved by transmitting SCH subcarriers from known antennas. Antennas are:

- Cyclic delay diversity (with antenna specific delay values

- Interleaved either within a symbol (multiple antennas can transmit within a single symbol but on distinct subcarriers) or the different SCH sequences are transmitted from multi-antennas

- Across frames (only one antenna transmits in each symbol)

- Or some combination – actual approach is FFS.

The number of BS antennas supported for MIMO channel measurements is FFS, depending on the requirements of other 16m SDD content, such as DL MIMO and interference mitigation.

##### Synchronization channel Sequence Design Properties

The SCH enables timing synchronization by autocorrelation.

The power of synchronization channel can be boosted

The SCH (non-hierarchical) or P-SCH (hierarchical) is mapping with every other subcarrier on the frequency domain.

The S-SCH (hierarchical) is mapping with every [FFS] subcarriers on the frequency domain.