| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Requirements for 802.16m Synchronization Channel |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | Paul Cheng Chris Williams Project 802.16m Preamble Rapporteur Group Chairs | Paul.Cheng@Mediatek.com Chris.Williams@uk.Fujitsu.com |
| Re: | IEEE 802.16m-08/001 |  |
| Abstract | A harmonized set of requirements for the 802.16m Synchronization Channel is presented based on previous contributions. |  |
| Purpose | Revision of C80216m-08_636 for the comment agreed in Preamble RG meeting. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Synchronization Channel Requirements

for the 802.16m SDD

Paul Cheng, Chris Williams

Project 802.16m Preamble Rapporteur Group Chairs

Introduction

This is a proposed harmonized set of requirements for the synchronization channel (preamble) design of IEEE 802.16m. The numbering has been retained from document C80216m-08/634, should it be decided to incorporate harmonized requirements into that document. Items requiring resolution are in square brackets and highlighted in yellow.

# Physical Layer

## Downlink Control Structure

### Downlink Control Information Classification

### Transmission of DL control information

#### Synchronization Channel

##### Preamble requirements

DEFINITIONS

| Convergence time | Time interval for the probability of error in preamble index detection to be less than [1] % under non-ideal assumptions on the timing and carrier synchronization, measured from the start of the acquisition process. |
| --- | --- |
| Correct detection | Choose a BS among the co-channel BS’s whose received powers averaged over the convergence time are within [3] dB of the BS with the highest received power |
| Coverage area | Area where the false detection probability is less than [1] % within the convergence time |
| Overhead | Total radio resources (time and frequency) per superframe that can not be used for other purpose because of SCH |
| Cell ID set | The cell ID set is the set of unique SCH symbols for differentiating between macrocell/femtocell/sector/relay transmitters |
| Multi-bandwidth support | Design of SCH for different bandwidths as specified in Table 2 of SDD (C802.16m-08_003r2, or latest revision) |
| Multi-carrier support | Design of SCH to support functionality described in SDD (C802.16m-08_003r2, or latest revision) Sections 8.1.3 & 11.4.1.3 |

11.x.2.1.1.1 Overhead

Mixed mode with legacy system

In mixed mode operation the preamble overhead shall be less than or equal to [ 4] % per super frame including the legacy preamble, where the [4]% is calculated based on the ratio of preamble resource and that of usable resource for transmitting data.

16m only mode

In 16m only mode operation the preamble overhead shall be less than or equal to [2] % per super frame, where the [2]% is calculated based on the ratio of preamble resource and that of usable resource for transmitting data.

11.x.2.1.1.2 Synchronization

The SCH will provide synchronization for:

- Time, including frame and superframe

- Frequency (up to [20]ppm initial tolerance of carrier frequency)

Synchronization performance must at least match that of 802.16e.

16m preamble must enable system acquisition without knowledge of the full channel bandwidth.

The 16m SCH shall not cause degradation of synchronization for coexisting legacy systems. Synchronization shall be robust for the full range of required mobile velocities as defined in the SRD (i.e. up to 350km/hr).

11.x.2.1.1.3 Coverage

The coverage area of IEEE 802.16m preamble shall not be worse than the minimum of the required coverage for broadcasting channel, control channel and unicast data channel at channel conditions under considerations.

11.x.2.1.1.4 Cell IDs

The cell ID shall be obtained from the SCH. To support femto-cell deployments, the number of unique cell IDs conveyed by the preamble shall be greater than or equal to FFS

11.x.2.1.1.5 MIMO support and channel estimation

IEEE802.16m preamble may support multi-antenna transmissions. The number of supported antennas is FFS.

Channel estimation supported from the preamble is FFS.

11.x.2.1.1.6 Multi-carrier Multi-bandwidth support

IEEE 802.16m preamble shall support multi-bandwidth and multi-carrier operations as defined in the latest revision of the SDD.

11.x.2.1.1.7 Measurement Support

IEEE 802.16m preamble shall support noise power estimation.

11.x.2.1.1.8 Sequence requirements

The PAPR and peak power shall be no larger that than those of the downlink signal (excluding SCH).