| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover framework |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Stavros Tzavidas Motorola 1501 W. Shure Dr. Arlington Heights IL 60004 | E-mail: stavros.tzavidas@motorola.com |
| Re: | IEEE 802.16m-08/024 “Call for Contributions on Project 802.16m System Description Document (SDD)” on the topic of “Upper MAC Concepts” |  |
| Abstract | This document describes a proposed framework for handover in 802.16m |  |
| Purpose | Review and adopt proposed text changes in 802.16m system description document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Handover Framework for 802.16m

Stavros Tzavidas

Motorola

# Introduction

The current standard (802.16e as well as the extensions currently introduced in Rev2) offers a large number of options regarding handover (HO) initiation and execution, thus creating a large number of ambiguous scenarios. It is desirable to simplify the handover procedures in order to reduce ambiguity, reduce network complexity and enable simple and flexible network architectures. This contribution proposes a framework for handover procedures in 802.16m.

We use the following abbreviations: “S-BS” for Serving Base Station, “N-BS” for Neighboring Base Station, “MS” for Mobile Station and “HO” for Handover. Furthermore, the term “unavailability interval” is used to describe a time period during which the MS is not required to monitor transmissions from the S-BS. These can be explicitly allocated by the S-BS (similar to sleeping/scanning intervals in 802.16e). The specific mechanism that the MS uses to determine unavailability intervals is left for further study (FFS).

The overall goal is to introduce a network-controlled, MS-assisted handover algorithm.

The proposed framework includes the following elements:

- Scanning

  - MS scans neighbor BSs and reports scan results to S-BS.

  - N-BSs on same frequency as S-BS: MS can scan while simultaneously monitoring transmissions from S-BS.

  - N-BSs on different frequency: MS may be capable of scanning while simultaneously monitoring transmissions from S-BS. Otherwise MS may use any unavailability interval.

  - S-BS defines triggers based on which the MS sends scan reports to the S-BS.

  - Poll for scan report: BS can request (poll) the MS to send a scan report with the latest scanning results at any time

- HO initiation

  - BS-initiated HO shall be supported by the MS

  - MS-initiated HO: S-BS defines HO triggers based on which the MS initiates a HO.

- HO execution

  - Upon HO initiation S-BS controls:

    - Final T-BS(s) selection

    - Timing of HO (when the MS will disconnect from S-BS). Upon reception of HO message MS has specific deadline to execute HO.

    - Whether HO-IND will be sent or not (via an appropriate flag in BSHO-REQ/RSP)

  - HARQ is used for all HO messages to ensure reliability of reception

    - HARQ ACK provides acknowledgement for BS HO message (e.g BSHO-REQ/RSP)

- State transfer during HO

  - HARQ state transfer (HARQ continuation) at T-BS shall be possible to ensure continuation of service

    - Maintain PDU sequence numbers and all relevant HARQ parameters

  - Granular and binding signaling covering state transfer parameters

    - Example: HO process optimization is no longer simply informational

- Network Re-entry at target BS

  - The exact algorithm for fast network re-entry at T-BS is for further study (FFS).

# Proposed Text Changes

[add the following subsection in Section 10 “Medium Access Control Sub-layer” on page 24 of 802.16m-08/003r3]

10.x Handover Framework

The handover procedures that are supported are depicted in the following figures.

MS

S-BS

S-BS defines triggers for sending scan reports (specific message TBD)

N-BS in same frequency: MS scans without temporarily disconnecting from S-BS

N-BS in different frequency: MS can use any unavailability interval to scan. Specific mechanism for requesting and scheduling unavailability intervals is FFS

SCAN-REPORT

(when trigger condition satisfied)

Poll for scan report

SCAN-REPORT

Exact MAC management messages to be used are FFS

Figure X Neighbor Scanning Procedure

MS is capable of scanning N-BS(s) on the same frequency as S-BS while simultaneously monitoring transmissions from S-BS. The capability of the MS to scan N-BS(s) on different frequencies while monitoring S-BS transmissions is FFS. The MS may use any unavailability interval at the S-BS to scan N-BS(s).

The serving BS defines triggers/conditions based on which the MS should send a scan report the S-BS (specific mechanism for trigger definition is FFS). The S-BS can also request (poll) the MS to send a scan report at any time.

MS

S-BS

S-BS defines triggers for MS initiated HO

(specific message TBD)

Handover Command

(exact message FFS)

HO request

(when trigger condition satisfied)

S-BS may communicate with other BSs via the backbone and makes final decision on:

T-BS(s)

Timing of HO

State transfer parameters

Selected T-BS(s)

HO execution time

HO-IND requested (flag)

Granular and binding signaling for state transfer:

HARQ continuation indication

Other items FFS

HO-IND

(if requested by S-BS)

MS disconnects from S-BS and begins fast network re-entry at T-BS

(specific algorithm FFS)

HO execution time

(S-BS stops data allocations for MS after this time)

Selected T-BS (if more than one T-BS given in handover command)

HARQ is used for all HO messages

Exact MAC management messages to be used are FFS

Figure X Handover Initiation and Preparation

The supported algorithm is BS-controlled, MS-assisted handover. The S-BS can initiate a handover at any time. The S-BS can also enable handover initiation by the MS. When HO initiation by the MS is enabled, the S-BS defines the triggers/conditions based on which the MS initiates a handover (specific mechanism for trigger definition is FFS).

When a handover is initiated, the S-BS (possibly via backbone communication with other BSs) decides on the target BS(s) and the timing of the handover. The S-BS sends to the MS a handover command message which contains the following information:

- Selected T-BS(s)

- HO execution time

- Flag to indicate if HO-IND is required

- Granular and binding signaling for state transfer parameters

    - HARQ continuation

    - Other items are FFS

- Other parameter updates related to fast-network re-entry at T-BS

When multiple target BS(s) are specified in HO command, and HO indication is requested by S-BS, the MS may select a target BS in HO-IND.

At the time specified by the HO execution time parameter the S-BS stops data allocations for the MS. The MS disconnects from the S-BS and begins fast network re-entry procedure at the T-BS. The specific fast network re-entry procedure is FFS.

HARQ is used for all HO messages to ensure reliability of reception.