| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DL Pilot Structure for IEEE 802.16m |  |
| Date Submitted | 2008-06-17 |  |
| Source(s) | Chung-Lien Ho Zheng Yan-Xiu Ren-Jr Chen Chang-Lan Tasi Yu-Tao Hsieh Pang-An Ting Richard Li ITRI Yuval Lomnitz Huaning Niu Jong-kae (JK) Fwu Sassan Ahmadi Hujun Yin Intel Corp. Jungnam Yun Jianjun Li Zhengzi Li Dongjun Lee Jaehyeong Kim POSDATA Co. Ltd. Pei-Kai Liao Chih-Yuan Lin Paul Cheng MediaTek Inc. Taeyoung Kim Sangheon Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi , Heewon Kang Samsung Electronics | clho@itri.org.tw zhengyanxiu@itri.org.tw rjchen@itri.org.tw tsaichangl@itri.org.tw ythsieh@itri.org.tw pating@itri.org.tw richard929@itri.org.tw Yuval.Lomnitz@intel.com Huaning.Niu@intel.com Jong-kae.Fwu@intel.com Sassan.Ahmadi@intel.com Hujun.Yin@intel.com jnyun@posdata-usa.com jianjun.li@posdata.co.kr zhengzi@posdata.co.kr djthekid@posdata.co.kr jaekim@posdata.co.kr pk.liao@mediatek.com chihyuan.lin@mediatek.com paul.cheng@mediatek.com ty33.kim@samsung.com |
| Re: |  |  |
| Abstract | This contribution describes pilot pattern design considerations for DL PHY. Pilot density, pilot spacing, extrapolation of channel estimation, interlaced pilot structure, power boosting on pilots, pilot power across antennas and irregular sub-frame are discussed. Our text proposal also includes a pilot pattern satisfying our considerations. |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DL Pilot Structure for IEEE 802.16m

Chung-Lien Ho, Yan-Xiu Zheng, Ren-Jr Chen, Chang-Lan Tasi, Yu-Tao Hsieh,

Pang-An Ting, Richard Li

ITRI

Yuval Lomnitz, Huaning Niu, Jong-Kae (JK) Fwu, Sassan Ahmadi, Hujun Yin

# Intel Corporation

Jungnam Yun, Jianjun Li, Zhengzi Li, Dongjun Lee, Jaehyeong Kim

POSDATA Co. Ltd

Pei-Kai Liao, Chih-Yuan Lin and Paul Cheng

MediaTek Inc.

Taeyoung Kim, Sangheon Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi , Heewon Kang

Samsung Electronics

# Introduction

The pilot structure for IEEE 802.16m is initialized in #54 meeting. Some design considerations for pilot structure have been discussed. Based on the design considerations, a set of harmonized DL pilot structures is proposed for localized allocation of 1 and 2 stream scenarios.

# Design Considerations for Pilot Structure

According to the pilot structures proposed by several companies, we have come out some design considerations for the pilot structure. Based on the design considerations (rules), we then figure out a harmonized version for the pilot pattern. We found that this harmonized version can be unified for the common pilots and at the same time the dedicated pilots. Furthermore, we propose to unify the pilot structure for DL transmission. On the other hand, for different application purposes, different sizes of resource unit (RU) are also considered and included in our design rules.

  - Pilot Density

The pilot density directly affects the accuracy of channel estimation. Intuitively, higher pilot density means higher overhead and better channel estimation accuracy. To achieve a proper tradeoff between the overhead and accuracy, using 6 pilot tones per transmission data stream within a regular 18 x 6 RU is a reasonable. For other irregular RUs, for example, 18 x 5, 18 x 7, etc., the pilot structures are designed based on the regular RU. The corresponding pilot density then also depends on the size of RU case-by-case.

  - Pilot Spacing

The pilot spacing also affects the performance of channel estimation. The maximum pilot spacing between any two pilots in time domain and frequency domain should depend on the channel parameters, such as the coherent time and coherent bandwidth. To satisfy the system requirements of 16m, for example, the support of high speed mobility up to 350 km/h and a channel delay spread of no more than 5 us, we propose, from the sampling theorem, that the maximum pilot spacing may be limited to no more than 3 OFDM symbols in the time domain and no more than 9 subcarriers in the frequency domain. Based on the proposed pilot spacing, the channel estimate on tones other than the pilot tones can be appropriately interpolated/extrapolated.

  - Extrapolation of Channel Estimation

From the numerical perspective, the extrapolation, in general, is less accurate than interpolation. So to avoid the extrapolation for channel estimation, we propose to allocate the pilot sets near or at the RU boundaries.

  - Interlaced Pilot Structure

Interlaced pilot patterns can be optionally used by different base stations or sectors.

  - Power Boosting on Pilots

Pilot tones, in general, should be power boosted to enhance the channel estimation. However, directly pouring the power on pilot tones may induce symbol-by-symbol power fluctuation if the number of pilot tones for a particular pilot set is different in each of OFDM symbols. An alternative is to steal or borrow the power from the data and/or the null tones. This, however, will also cause the symbol-by-symbol power fluctuation on the data tones if the number of pilot tones for a particular pilot set is different in each of OFDM symbols. To remedy this, all the sets of pilots for different data streams should be evenly placed on each of OFDM symbols within a RU.

  - Pilot Power across Antennas

To reduce the power fluctuation across antennas, it is important that the power fluctuation across the antennas should be minimized for multi-antenna/stream transmission. Again, this can be done by evenly allocating all the pilot sets on each of OFDM symbols within a RU.

  - Subframe with Less or More OFDM Symbols

In 16m, the first and last OFDM symbols may be preserved for certain purposes, such as for control signaling and/or guard interval. However, reserving these two OFDM symbols will lead to symbol-by-symbol power fluctuation as mentioned above. For 18x3 RU, power fluctuation may also occur. Since a unified pilot structure is more preferable, three scenarios would be considered in pilot structure design so that the pilot density (overhead) can remain the same regardless of the number of OFDM symbols within a subframe; no power fluctuation occurs.

    - We can delete the last time domain column from a regular RU (18x6) to form another pilot structure on an irregular RU (18x5).

    - We can insert an extra column behind a regular RU (18x6) by duplicating the first column on a regular RU (18x6) to form another pilot structure on an irregular RU (18x7).

    - We can separate the pilot structure in a regular RU (18x6) into two identical pilot structures on two irregular RUs (18x3).

Many design considerations are discussed above. Depending on various needs in system designs or applications, some or all of these design considerations may be prioritized, add, or disregarded. For example, some of them may be implemented in full, in part, or not implemented at all.

# Proposed Pilot Patterns

Based on the above design considerations and proposed methodologies, the 1 and 2 stream pilot patterns are generated and show below in SDD text proposal.

The following texts are proposed for SDD DL Physical Structure:

------------------------------ Proposed SDD Text -----------------------------------

The pilots can be used for channel estimation, measurements (CQI and interference mitigation/cancellation), frequency offset estimation and time offset estimation. The pilots will be dedicated to localized and distributed subchannels. The pilots will be pre-coded, using the same pre-coding and/or boosting of the information burst(s) when such pre-coding is applied. The pilot patterns are optimized for both regular and irregular subframes.

Pilot patterns are proposed for efficiency and performance. Pattern A is used for 1 and 2 data streams DL dedicated and common pilot pattern.

Fig 11.x.3 – 1 Pattern A for 1/2 pilot streams.

For irregular subframe of 5 or 7 OFDMA symbol, the first or last OFDMA symbols will be deleted or added.
The interlaced pilot patterns can be generated by cyclic shifting the base pilot pattern. The interlaced pilot patterns are shown in Fig 11.x.3 – 2 and can be optionally used by different BSs. The use of interlaced pilot pattern is FFS.

Fig 11.x.3 – 2 Interlaced Pattern A for 1/2 pilot streams.