| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Final Draft: SDD Text on Downlink MIMO Schemes |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Minnie Ho, Jerry Pi, Yong Sun Rapporteur Group Chairs | Minnie.ho@intel.com zpi@sta.samsung.com sun@toshiba-trel.com |
| Re: | DL MIMO Rapporteur Group Discussions |  |
| Abstract | A consolidated SDD text on the DL MIMO schemes based on DL MIMO SDD text proposals submitted to the 802.16m email reflector is provided in this draft. |  |
| Purpose | For review and discussion in the Project 802.16m DL MIMO Rapporteur Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Final Draft: SDD Text on Downlink MIMO

Minnie Ho, Jerry Pi, Yong Sun

Rapporteur Group Chairs

In order to provide a general framework, SDD text for the DL MIMO Scheme in this draft has been organized by function. Reference to specific solutions and terminology has been avoided. Dependencies on text in the SDD that is still under development have been captured in notes wherever applicable.

[Chair’s Notes: After Draft #3 was released, we realized that in the .16e spec (and Rev2), the word “layer” is used instead of the word “codeword”, and the word “stream” is used instead of the word “layer”.  We need to follow the same terminology, since .16m and .16e will belong in the same standard.

The Pilot section is being handled by discussion within the DL_PHY group, and we will need to match text in the Pilot section with that developed by the DL_PHY group.

Signaling support for MIMO should be coordinated with DL control channel design.]

# 11.x  DL MIMO Transmission Scheme

# 11.x.1  DL MIMO Architecture and Data Processing

The architecture of downlink MIMO on the transmitter side is shown in the Figure 1.

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In MU-MIMO, multiple users can be scheduled in one RU.

If vertical encoding is utilized, there is only one encoder/modulator block (one “layer”). If horizontal encoding is utilized, there are multiple encoders / modulators (multiple “layers”). A “layer” is defined as a coding / modulation path fed to the MIMO encoder as an input, and a “stream” is defined as each output of the MIMO encoder that is passed to the beamformer / precoder.

User

1

data

User

2

data

User P

data

User i

data

MIMO

Encoder

Beamformer

/Precoder

IFFT

OFDM Symbol

Construction

IFFT

IFFT

Precoding Vector

/Matrix

Encoder

Encoder

Encoder

Encoder

Encoder

Encoder

Layer

control

Scheduler

Resource

Mapping

Feedback

CQI

CSI

ACK / NAK

Mode / Rank / Link Adaptation

Figure 1: MIMO Architecture

The encoder block contains the channel encoder, interleaver, rate-matcher, and modulator for each layer.

The Resource Mapping block maps the modulation symbols to the corresponding time-frequency resources in the allocated resource units (RUs).

The MIMO encoder block maps L (≥1) layers onto M (≥L) streams, which are fed to the precoding block.

The precoding block maps streams to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The OFDM symbol construction block maps antenna-specific data to the OFDM symbol.

The feedback block contains feedback information such as CQI and CSI from the MS.

The scheduler block will schedule users to resource blocks and decide their MCS level, MIMO parameters (MIMO mode, rank).  This block is responsible for making a number of decisions with regards to each resource allocation, including:

- Allocation type: Whether the allocation in question should be transmitted with a distributed or localized allocation

- Single-user (SU) versus multi-user (MU) MIMO: Whether the resource allocation should support a single user or more than one user

- MIMO Mode: Which open-loop (OL) or closed-loop (CL) transmission scheme should be used for the user(s) assigned to the resource allocation.

- User grouping: For MU-MIMO, which users should be transmitted to on the resource allocation

- Rank selection: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for the user allocated to the resource allocation.

- MCS level per layer: The modulation and coding rate on each layer must be determined.

- Boosting: The power boosting values to be used on the data and pilot subcarriers.

- Band selection: If localized resource allocation is used, where in the frequency band should the localized allocation be placed.

# Antenna Configuration

The BS employs a minimum of two transmit antennas. The MS employs a minimum of two receive antennas. The antenna configurations are (NT, NR) = (2, 2), (4, 2), (4, 4), (8, 2), and (8, 4), where NT denotes the number of BS transmit antennas and NR denotes the number of MS receive antennas. (8,8) with 8 streams is FFS.

# Layer to Stream Mapping

The number of streams, M, for SU-MIMO is M≤min(NT,NR), where M is no more than 4 (8 streams are FFS).  MU-MIMO can have up to 2 streams with 2 Tx antennas, and up to 4 streams for 4 Tx antennas and 8Tx antennas.

For SU-MIMO, Vertical encoding (SCW) is employed. [The support of horizontal encoding (MCW) for SU-MIMO is FFS].  For MU-MIMO, MCW (or horizontal) encoding is employed at the base-station while only one layer is transmitted to each mobile station.

# Stream to Antenna Mapping

The stream to antenna mapping depends on the MIMO scheme used.  The mapping can be defined using the following equation

y=P×S(x),                            (Equation 11.x.1.3-1)

where P is a pre-coding matrix, S(x) is an STC matrix, and x is the input layer vector.

# Resource mapping

The following table illustrates the MIMO mode permutation for various MIMO schemes.

Table 11.x.1.4-1. Supported resource channels in MIMO

| MIMO Scheme | Resource Mapping |
| --- | --- |
| Open-loop SU-MIMO | Distributed or Localized |
| Closed-loop SU-MIMO | Distributed or Localized |
| MU-MIMO | Distributed or Localized |

      - Pilots

# Signaling support for MIMO

# Signaling support for SU MIMO

# Signaling support for MU MIMO

In the downlink MU-MIMO, the precoding matrix shall be signaled via explicit signaling if common demodulation pilots are used, or via dedicated pilots.

# Transmission for Data Channels

# Single-user MIMO

Single-user MIMO schemes are used to improve per-link performance.

Both open-loop single-user MIMO and closed-loop single-user MIMO are supported for the antenna configurations specified in Section 11.x.1.1.

For open-loop single-user MIMO, both spatial multiplexing and transmit diversity schemes are supported. Note that in the case of open-loop single-user MIMO, CQI and rank feedback may still be transmitted to assist the base station’s decision of rank adaptation, transmission mode switching, and rate adaptation. Note that CQI, and rank feedback may or may not be frequency dependent.

For closed-loop single-user MIMO, codebook based precoding is supported for both TDD and FDD systems. CQI, PMI, and rank feedback can be transmitted by the mobile station to assist the base station’s scheduling, resource allocation, and rate adaptation decisions. Note that the CQI, PMI, and rank feedback may or may not be frequency dependent.

For closed-loop single-user MIMO, sounding based precoding is supported for TDD systems.

As described in section 11.x.1, the overall structure of MIMO processing has two parts. The first part is the MIMO encoder and second part is the precoder.

The MIMO encoder is a batch processor that operates on M input symbols at a time. The input to the MIMO encoder is represented by an M  1 vector

x=s1s2⋮sM,                           (Equation 11.x.2.1-1)

where si is the i-th input symbol within a batch. The output of the MIMO encoder is an M  NF MIMO STC matrix z = S(x), which serves as the input to the precoder. The output of the precoder is denoted by a matrix NT  NF matrix

y=y1,1y1,2⋯y1,NFy2,1y2,2⋯y2,NF⋮⋮⋱⋮yNT,1yNT,2⋯yNT,NF,                 (Equation 11.x.2.1-2)

where yi,j is the output symbol to be transmitted via the i-th physical antenna on the j-th subcarrier/symbol. Note NF is the number of subcarriers or symbols used to transmit the MIMO signals derived from the input vector x. For open-loop SU-MIMO, the rate of a mode is defined as R=MNF.

# Open-loop SU-MIMO

A number of antenna configurations and transmission rates are supported in open-loop SU-MIMO. Among them, 2Tx, 4Tx, and 8Tx antennas with rate 1 transmission are defined as Transmit Diversity modes. The operation of these modes is specified in Section 11.x.2.1.1.1. The other modes, including 2Tx, 4Tx, and 8Tx antennas with rate 2 transmission, 4Tx and 8Tx antennas with rate 3 transmission, and 4Tx and 8Tx antennas with rate 4 transmission, are defined as Spatial Multiplexing modes. The operation of these modes is specified in Section 11.x.2.1.1.2. The dimensions of the vectors and matrices for open-loop SU-MIMO are shown in the following table:

Table 11.x.2.1.1-1. Matrix dimensions for open-loop SU-MIMO modes

| NT | Rate | M | NF |
| --- | --- | --- | --- |
| 2 | 1 | 1 | 1 |
| 2 | 1 | 2 | 2 |
| 4 | 1 | 1 | 1 |
| 4 | 1 | 2 | 2 |
| 8 | 1 | 1 | 1 |
| 8 | 1 | 2 | 2 |
| 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 1 |
| 8 | 2 | 2 | 1 |
| 4 | 3 | 3 | 1 |
| 8 | 3 | 3 | 1 |
| 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 1 |

On a given frequency resource k [size is FFS], the precoding matrix P can be defined using the following equation:

P(k) = D(k)W(k).                  (Equation 11.x.2.1.1-1)

The precoder is composed of two matrices. The first matrix W(k) is an NT  M matrix, where NT is the number of transmit antennas and M is the numbers of streams. The matrix W(k) is selected from a predefined unitary codebook, and changes every u subcarriers. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. [The detailed unitary codebook, and the parameter u are FFS.] The second matrix D(k) is an NT  NT diagonal matrix as follows,

D(k)=ejθ0k0⋯00ejθ1k⋯0⋮⋮⋱⋮00⋯ejθ(NT−1)k,             (Equation 11.x.2.1.1-2)

where k denotes frequency resource index and θi,i= 0, 1, 2, ..., NT-1 denotes the phase shift for the i-th transmit antenna across two adjacent frequency resources. [The value of  is FFS.]

For OL SU-MIMO, the following schemes are FFS: rate-1 STBC/SFBC and rate-2 Double STBC/SFBC, 2-D POD for rate-1 and rate-2, rate-3 hybrid SM+STBC/SFBC, differential STBC/SFBC, SM+Antenna hopping.

# Transmit Diversity

The following transmit diversity modes are supported for open-loop single-user MIMO:

- 2Tx rate-1: STBC/SFBC, and rank-1 precoder

- 4Tx rate-1: STBC/SFBC with precoder, and rank-1 precoder

- 8Tx rate-1: STBC/SFBC with precoder, and rank-1 precoder

In Transmit Diversity mode, the MIMO encoder generates 2Tx STBC/SFBC, and then multiplied by NT  2 matrix and NT  NT diagonal matrix as described in section 11.x.2.1.1.

For the transmit diversity modes, the input to the MIMO encoder is represented a 2  1 vector

x=s1s2.                     (Equation 11.x.2.1.1.1-1)

The output of the MIMO encoder is a 2  2 matrix

z=s1−s2s2s1z=s1s2−s2s1.                   (Equation 11.x.2.1.1.1-2)

For the 2Tx rate-1 mode, the output of the precoder is a 2  2 matrix

y=z.                       (Equation 11.x.2.1.1.1-3)

For the 4Tx rate-1, the output of the precoder is a 4  2 matrix

y=D×W×z,                   (Equation 11.x.2.1.1.1-4)

where W is a 4  2 precoder and D is a 4  4 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

For the 8Tx rate-1, the output of the precoder is a 8  2 matrix

y=D×W×z,                   (Equation 11.x.2.1.1.1-5)

where W is a 8  2 precoder and D is a 8  8 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

# Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM

  - 4Tx rate-2: rate 2 SM with precoding

  - 8Tx rate-2: rate 2 SM with precoding

- Rate-3 spatial multiplexing modes:

  - 4Tx rate-3: rate 3 SM with precoding

  - 8Tx rate-3: rate 3 SM with precoding

- Rate-4 spatial multiplexing modes:

  - 4Tx rate-4: rate 4 SM

  - 8Tx rate-4: rate 4 SM with precoding

For the rate-2 spatial multiplexing modes, the input to the MIMO encoder is represented as a 2  1 vector

x=s1s2.                    (Equation 11.x.2.1.1.2-1)

The output of the MIMO encoder is a 2  1 vector

z=x.                      (Equation 11.x.2.1.1.2-2)

For the 2Tx rate-2 mode, the output of the precoder is a 2  1 vector

y=z.                      (Equation 11.x.2.1.1.2-3)

For the 4Tx rate-2 mode, the output of the precoder is a 4  1 vector

y=D×W×z,                    (Equation 11.x.2.1.1.2-4)

where W is a 4  2 precoder and D is a 4  4 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

For the 8Tx rate-2 mode, the output of the precoder is a 8  1 vector

y=D×W×z,                    (Equation 11.x.2.1.1.2-5)

where W is a 8  2 precoder and D is a 8  8 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

For the rate-3 spatial multiplexing modes, the input to the MIMO encoder is represented as a 3  1 vector

x=s1s2s3.                        (Equation 11.x.2.1.1.2-6)

The output of the MIMO encoder is a 3  1 vector

z=x.                          (Equation 11.x.2.1.1.2-7)

For the 4Tx rate-3 mode, the output of the precoder is a 4  1 vector

y=D×W×z,                    (Equation 11.x.2.1.1.2-8)

where W is a 4  3 precoder and D is a 4  4 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

For the 8Tx rate-3 mode, the output of the precoder is a 8  1 vector

y=D×W×z,                    (Equation 11.x.2.1.1.2-9)

where W is a 8  3 precoder and D is a 8  8 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

For the rate-4 spatial multiplexing modes, the input to the MIMO encoder is represented as a 41 vector

x=s1s2s3s4.                    (Equation 11.x.2.1.1.2-10)

The output of the MIMO encoder is a 4  1 vector

z=x.                       (Equation 11.x.2.1.1.2-11)

For the 4Tx rate-4 mode, the output of the precoder is a 4  1 vector

y=z.                       (Equation 11.x.2.1.1.2-12)

For the 8Tx rate-4 mode, the output of the precoder is a 8  1 vector

y=D×W×z,                   (Equation 11.x.2.1.1.2-13)

where W is a 8  4 precoder and D is a 8  8 diagonal phase matrix. Note that W and D may be frequency dependent as described in section 11.x.2.1.1.

# Closed-loop SU-MIMO

# Precoding technique

In FDD and TDD systems, unitary codebook based precoding are supported.

In TDD systems, sounding based precoding is supported.

# Power Loading Technique

# Feedback for SU-MIMO

In FDD systems and TDD systems, a mobile station may feedback some of the following information in Closed loop SU-MIMO mode:

- Rank (Wideband or sub-band)

- Sub-band selection

- CQI (Wideband or sub-band, per layer)

- PMI (Wideband or sub-band for serving cell and/or neighboring cell)

- Doppler estimation

For codebook based precoding, the feedback from a mobile station shall be based on the same codebook as used by base station for transmission.

The feedback information may be transmitted via a physical layer control channel or via a higher layer signaling message.

In TDD systems, a mobile station may transmit a sounding signal on the uplink.

# Multi-user MIMO

Multi-user MIMO schemes are used to enable a resource allocation to communicate data to two or more MSs. 802.16m uses Multi-user MIMO to boost system throughput.

Multi-layer transmission with one stream per user is supported for MU-MIMO. MU-MIMO includes the MIMO configuration of 2Tx antennas to support up to 2 users, and 4Tx or 8Tx antennas to support up to 4 users.

# Precoding technique

The precoding for MU-MIMO can be either standardized or vendor-specific. Up to four MSs can be assigned to each resource allocation.

In MU-MIMO systems, the received signal of the f-th subcarrier in the i-th MS (without considering co-channel interference) can be described as:

yi,f=Hi,fj=1Kvj,fxj,f+ni,f                    (Equation 11.x.2.2.1-1)

where K is the number of the allocated users, Vj,f is the precoding vector of the f-th subcarrier for the transmit signal to the j-th MS, xj,fis the transmit signal of the f-th subcarrier to the j-th MS and ni,fis the noise of the f-th subcarrier in the j-th MS.

If dedicated pilots are used, the form and derivation of  the assembled precoding matrix, , can be either standardized or vendor-specific. If the columns of the assembled precoding matrix are orthogonal to each other,  it is defined as unitary MU-MIMO. Otherwise, it is defined as non-unitary MU-MIMO. Note that beamforming is enabled with this precoding mechanism. Non-linear precoding is FFS.

# Unification with SU

Predefined and flexible adaptation between SU-MIMO and MU-MIMO are supported.

# Feedback for MU-MIMO

# CQI feedback

For CQI feedback, the mobile station measures the downlink pilot channel, computes the channel quality information (CQI), and reports the CQI on the uplink feedback channel. Both wideband CQI and subband CQI may be transmitted by a mobile station. Wideband CQI is the average CQI of a wide frequency band. In contrast, sub-band CQI is the CQI of a localized sub-band.

# CSI feedback

Channel state information feedback may be employed for MU-MIMO. Codebook-based feedback is supported in both FDD and TDD. Sounding-based feedback is supported in TDD.

The unified codebook for SU and MU is employed.

An enhanced UL sounding channel is used to feedback CSI-related information by the MS to facilitate vendor-specific adaptive closed-loop MIMO precoding.  For sounding-based precoding, the enhanced UL sounding channel can be configured to carry a known pilot signal from one or more MS antennas to enable the BS to compute it precoding/beamforming weights by leveraging TDD reciprocity.  The sounding waveform can be configured to occupy portions of the frequency bandwidth in a manner similar to the sounding waveform used in the legacy system.  To facilitate analog-feedback-based precoding, the enhanced UL sounding channel can be configured to carry unquantized CSI-related information (e.g., an unquantized encoding of the DL spatial covariance matrix or an unquantized encoding of the eigenvectors of the DL spatial covariance matrix).  The unquanitized CSI-related information can be specific to a particular specified portion of the band (narrowband feedback) or specific to the entire bandwidth (wideband feedback).

# Rank and Mode Adaptation

To support the numerous radio environments for 802.16m systems, both MIMO mode and rank adaptation are supported. BSs and MSs may adaptively switch between DL MIMO techniques depending on parameters such as antenna configurations and channel conditions. Parameters selected for mode adaptation may have slowly or fast varying dynamics. By switching between DL MIMO techniques an 802.16m system can dynamically optimize throughput or coverage for a specific radio environment.

The MIMO modes include open-loop MIMO like transmit diversity, spatial multiplexing, and closed-loop MIMO, etc. The adaptation of these modes is related with the system load, the channel information, MS speed and average CINR. Switching between SU-MIMO and MU-MIMO is also supported.

Both dynamic and semi-static adaptation mechanisms are supported in 16m. For dynamic adaptation, the mode/rank may be changed frame by frame. For semi-static adaptation, MS may request adaptation. The decision of rank and mode adaptation is made by the BS. The adaptation occurs slowly, and feedback overhead is less.

# Transmission for Control Channel

# Transmission for Broadcast Control Channel

A SU open-loop technique that provides diversity gain will be used here. The detailed transmit diversity scheme for the Broadcast Control Channel is FFS.

# Transmission for Unicast Control Channel

A SU technique that provides diversity or beamforming gain will be used here. The detailed transmit diversity scheme for Unicast Control Channels is FFS.

# Advanced Features

# Multi-cell MIMO

Multi-cell MIMO techniques are supported for improving sector throughput and cell-edge throughput through multi-BS collaborative precoding, network coordinated beamforming, or inter-cell interference nulling. Both open-loop and closed-loop multi-cell MIMO techniques can be considered. For closed-loop multi-cell MIMO, CSI feedback via codebook based feedback or sounding channel will be used. The feedback information may be shared by neighboring base stations via network interface. Mode adaptation between single-cell MIMO and multi-cell MIMO is utilized.

# MIMO for Multi-cast Broadcast Services

Open-loop spatial multiplexing schemes as described in Section 11.x.2.1.1.2 are used for MBS.

Appendix:

The desired properties of the codebook are:

- Good performance in uncorrelated, correlated, and dual-polarized channels

- Low feedback and signaling overhead

- Low computational complexity

- Low memory requirement

- Low PAPR / Power balanced

The following codebook should be considered:

- DFT-based codebook

- 16e codebook

The desired properties of the uplink sounding signals are:

- Low cross-correlation sequences

- Low PAPR

- Low overhead