| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power Control in 802.16m |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | Kanchei (Ken) Loa, Yung-Ting Lee, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Hsueh Tsai, Tsung-Yu Tsai Institute for Information Industry Whai-En Chen National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Yang-Han Lee, Yih Guang Jan Tamkang University | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16m-08/024: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) on the topic of “Power Control” |  |
| Abstract | This contribution proposes the text for power control in 802.16m SDD |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power Control in 802.16m

Kanchei (Ken) Loa, Yung-Ting Lee, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Hsueh Tsai,

Tsung-Yu Tsai

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Shiann-Tsong Sheu, Chih-Cheng Yang

National Central University

Yang-Han Lee, Yih Guang Jan

Tamkang University

Introduction

In current IEEE 802.16e system, power control schemes shall be used to maintain the quality of the radio link between the MS and the BS and to minimize the overall system interference. Basically, the power control of 16m systems shall be designed based on the following major considerations,

- Frame structure and the DL/UL control structure

- Open loop or closed loop power control

- Power control of DL channels including the common and dedicated channels

- Power control of UL channels including the random access and dedicated channels

- Power control for BS/MS in spatial diversity (e.g., MIMO)

- Power control for BS/MS in macro diversity (e.g., soft/softer handover)

- Power control for fast moving MS

- Power control for self-organization/optimization networks (e.g., pico-cell, femto-cell and relay networks)

- Sub-carrier power boost (e.g., preamble/pilot boosting)

- Power control with fractional frequency reuse (FFR) in different scenarios

- Joint decision of power control and rate adaptation

- Differential or aggregate power adjustment

In this contribution, we first propose two-stage power control in 802.16m SDD. That is, the outer loop power control and the inner loop power control. The outer loop power is used to set the target SINR based on the required bit/block error rate, whereas the inner loop power control is to dynamically adjust the transmit power to achieve the target SINR. For the DL, we propose

- The transmit power of the BS in E-MBS and unicast zones may be different.

- In a sub-frame, the transmit power of the BS is fixed.

- For MSs in a sub-frame, the required bit/block error rate should be achieved by using rate adaptation.

For the UL, we propose

- Open loop power control for random access channel. As a result, the MS could determine the transmit power for the first attempt based on the transmit power information sent by the BS and the measured DL signal strength.

- Closed loop power control for dedicated channel. As a result, the MS could determine the transmit power based on the power control information feedbacked by the BS.

- For the fast-moving MS, due to the very dynamic changes of the channel response, the power control would not be able to compensate the fast fading channel effect. As a result, the power control shall be used to compensate the distance-dependent path loss and slow fading only. Moreover, the fast-moving MS could request to change the power control mode from open loop to closed loop and vice versa. The BS could also send the unsolicited power control mode change command to the MS.

For the case when multiple BSs are involved to receive data from the MS (e.g., in macro diversity or soft/softer handover), we propose the transmit power for UL should be cooperatively controlled by the BSs.

Proposed Text for SDD

----------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the following subclause in the 802.16m-08/003r3]

11.x Power Control

In a 16m system, the power control scheme shall be designed for DL and UL based on the frame structure, DL/UL control structures, diversity and fractional frequency reuse (FFR). Two power control modes, namely open loop power control and closed loop power control should be adopted to maintain the quality of the radio link between the MS and the BS and to minimize the overall system interference. For closed loop power control, two power control stages, naming outer loop power control and inner loop power control shall be used. The outer loop power control is used to set the target SINR of a specific connection and is out of scope of 16m specifications. The inner loop power control is used to cope with the channel effect to achieve the target SINR and shall be defined for DL and UL in the 16m specifications.

11.x.1 DL power control

The BS should be capable of controlling the transmit power per sub-frame. This is useful for self-organization/optimization networks (e.g., pico-cell, femto-cell and relay networks) and for different application scenarios. For example, The transmit power of the BS in E-MBS and unicast zones may be different. In a sub-frame, the transmit power of the BS is fixed. In this sub-frame, the required SINR for a specific MS is achieved by scheduling, segment/subchannel power boost, and jointly with rate adaptation based on the measurement reports from the MS and/or the measurement results of UL received data at the BS.

11.x.2 UL power control

The UL power control for an MS to send data to the BS should be either open loop or closed loop. In UL open loop power control, the MS should determine the transmit power based on the transmission parameters sent by the BS and the received signal strength. In UL closed loop power control, MS should determine the transmit power based on the power control parameters encoded in the DL control signal. The UL closed loop power control shall be subframe-based. That is, the maximum power control frequency of UL closed loop power control is 400Hz with four switching points in the frame structure.

For the fast-moving MS, the UL transmit power should be determined by the MS according to the power control parameters sent by the BS and/or the DL signal strength measured at the MS. In case that the closed loop power control cannot compensate the fast fading channel effect caused by MS high mobility, the BS may request to change the UL power control mode from closed loop to open loop.

Power control in spatial diversity (e.g., MIMO) and macro diversity (e.g., soft handover and softer handover) are essential to improve the overall system efficiency and should be defined in the 16m specifications. In these cases, the involved BSs should cooperatively control the UL transmit power of the MS.

----------------------------------------------------End of the Text----------------------------------------------------------