| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized Intra-frequency Handover |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Whai-En Chen National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Kanchei (Ken) Loa, Yi-Ting Lin, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin, Yi-Hsueh Tsai, Chun-Yen Hsu Institute for Information Industry Yang-Han Lee, Yih Guang Jan Tamkang University | stsheu@ce.ncu.edu.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/024- Call for comments and contributions on project 802.16m system description document (SDD) Topic: Upper MAC concepts and methods (mobility) |  |
| Abstract | This contribution proposes to enable optimized handover for intra-frequency handover. |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimized Intra-frequency Handover

Whai-En Chen

National Ilan University

Shiann-Tsong Sheu, Chih-Cheng Yang

National Central University

Kanchei (Ken) Loa, Yi-Ting Lin, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin, Yi-Hsueh Tsai, Chun-Yen Hsu

Institute for Information Industry (III)

Yang-Han Lee, Yih Guang Jan

Tamkang University

Introduction

The handover latency for an MS includes the control plane latency and the data plane latency. However, the maximum control-plane latency specified in the appendix 2.2 of SDD (80216m-08_003r3; see Table 1) is equal to the maximum interruption time of the intra-frequency handover specified in 6.2.3 of SRD (80216m-07_002r4; see Table 2). To prevent interruption during handover, we proposed that the optimized handover shall be enabled when an MS performs intra-frequency handover.

Table 1- Maximum handover interruption

| Handover type | Max. interruption time (ms) |
| --- | --- |
| Intra-frequency | 30 |
| Inter-frequency | 100 |

Table 2- Data plane and control plane latency

| Control plane | < 30ms |
| --- | --- |
| Data plane | 4.67 ms + T’ASN_GW+TR6 |
| Total | < 34.67 ms + T’ASN_GW+TR6 |

Fig. 1- Data plane access latency. (source: SDD)

Proposed Text for SDD

[Insert the section 10.x “Handover”]

[Insert the section 10.x.y “Intra-frequency Handover”]

-------------------------------  Text Start  ---------------------------------------------------

10.x Handover

10.x.y Intra-frequency

When an MS performs intra-frequency handover, the optimized handover shall be enabled.

-------------------------------  Text End  ---------------------------------------------------