| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Sector-Specific Common Pilot Structure in IEEE802.16m Downlink |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII, Katsuhiko HIRAMATSU Matsushita Electric (Panasonic) | Voice: + 81-50-3687-6390 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE802.16m-08/024 – Call for Contributions on Project 802.16m System Description Document(SDD), shoot for “Comments for DL PHY pilot structure in IEEE 802.16m-08/003r3” topic |  |
| Abstract | We show the improved performance of the sector-specific pilot compared to the non sector-specific pilots and propose to include the sector-specific common pilots in TGm SDD . |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sector-Specific Common Pilot Structure in IEEE802.16m Downlink

Takaaki KISHIGAMI, Isamu YOSHII, Katsuhiko HIRAMATSU

Matsushita Electric (Panasonic)

# Introduction

At the TGm #55 meeting, RG DL PHY discussed the downlink common pilot structure and agreed the simulation parameters and the performance metrics[1]. Here, we focus on the interference limit case and discuss how to multiplex pilot among sectors/cells.

Since in 16e all sector/cell transmits common pilots in same locations, inter-sector or inter-cell interference might be huge impact on the channel estimation. In order to avoid inter-sector or inter-cell interference, following orthogonal multiplexing schemes could be useful [2].

  - Sector/cell-specific orthogonal sequence is multiplied to the basic pilot pattern.

  - Sector/cell-specific frequency/time offset is applied to the basic pilot pattern.

Already some contributions mentioned to introduce some sector-specific pilot schemes [3]-[7]. Also, in the uplink, same pilot concepts were proposed [8].

In following discussions, we explain the benefits of the sector-specific common pilot structures and show the performance gain comparing with 16e pilot allocation by means of the link level simulation based on the Pilot Evaluation criteria contained in Appendix A of the DL PHY Rapporteurs Report [1].

Hence, we propose to include the sector-specific common pilots using sector specific offset and/or orthogonal sequence in TGm SDD[9] in order to achieve throughput requirements [10].

# Sector Specific Common Pilot configuration

2.1 common pilots using sector-specific orthogonal sequence

Figure 1 shows common pilots using a sector-specific orthogonal sequence. The example is an orthogonal sequence using phase rotation in a BS with three sectors and a basic pilot pattern based on the 802.16e Downlink PUSC [11]. In the sector-specific orthogonal sequence, frequency domain spreading is used by adding phase rotation. In the figure, the phase rotation for Pk illustrates the pilot symbol transmitted from antenna #k. The phase rotation(φ) of 0, 2/3, and 4/3 is added every adjacent pilot subcarrier to sector #1, #2, and #3, respectively.

When the proposed sector-specific orthogonal sequence is used for the pilot channel, the inter-sector interference on the pilot channel from the contiguous sectors can be avoided by dispreading among neighbor pilot subcarrier, leading to accurate channel estimation.

φ= 0 (for Sector 1)2)4π3 (for Sector 3)

Figure 1 – Example pilot assignment using sector-specific orthogonal sequence

(Based on 802.16e Downlink PUSC [11])

2.2 common pilots using sector-specific offset sequence

Figure 2 shows an example pilot assignment using sector-specific offset assignment. The sector specific offset is applied to the pilot pattern in sector #1(hereafter, we call it ‘basic pilot pattern’). The offset in sector #k is (k-1)ΔSC. (Figure 2 shows the case of ΔSC =1.)  The offset of pilot positions is given in frequency direction for the basic pilot pattern as shown in Figure 1 . The sector specific offset is applied to all pilot symbols (P1, P2, … ). In the non-orthogonal pilot channel structure using the sector specific offset, the intra- BS interference on the pilot signals can be reduced compared to the case without sector specific offset although the interference from the pilot channel to the data channel is increased at the same time.

Figure 2 – Example Pilot assignment using sector-specific offset assignment

# Simulation Evaluations

We compared three pilot structures as shown in Figure 3. The first one is the sector-specific orthogonal pilots as descried in Section 2.1. The other two structures are based on the non-orthogonal pilots. For non-orthogonal pilots, we evaluated the case with sector specific offset (ΔSC =1) and without sector specific offset. The position of the basic pilot pattern is shown in Figure 1. The transmission power of the pilot symbol is set to 3dB higher than that of the data symbol.

(a)orthogonal pilots     (b)Non-orthogonal pilots      (c)Non-orthogonal pilots

without sector specific offset   with sector specific offset

Figure 3– Evaluated pilot structures

We investigated the effect of the sector-specific pilots in the same BS from the viewpoint of the throughput performance in downlink. Table 2 lists the radio parameters assumed in the link-level simulations.

The antenna pattern [12] is depicted in Fig. 4(a). The resultant relative gain considering inter-sector interference, sec = Psect#1/ (Psect#2 + Psect#3), is given in Fig. 4(b)[2]. sec represents the difference in the average received signal power at a MS, corresponding to the difference in the distances from the two sector antennas.

Figure 5 plots the throughput performance in case of sec =6,9,12,15 dB. In the figures, the red, green and blue lines respectively indicate the performance using the sector-specific orthogonal sequence, non-orthogonal pilots with sector-specific offset and non-orthogonal pilot without sector-specific offset.

We find that the common pilots using the sector-specific pilots can improve the throughput performance. In the figure, the throughput performance using the sector-specific orthogonal sequence is improved compared to that of non-orthogonal pilots with sector-specific offset.

Table2 – Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2.5Ghz |
| System bandwidth | 10MHz |
| FFT size | 1024 |
| Sub-carrier frequency spacing(fs) | 10.94kHz |
| Useful symbol interval (Ts=1/fs) | 91.4usec |
| Guard interval (Tg=Ts/8) | 11.4usec |
| Pilot power boost level | 3dB |
| Resource unit(RU) | 2 Localized RU [1RU: (Psc, Nsym)=(18,6)] |
| Antenna configuration | 1Tx–2Rx (MRC) |
| Channel coding | Turbo coding (original rate = 1/3) |
| Turbo decoding | Max-Log-Map (8 iterations) |
| MCS | QPSK, R=1/2,3/4; 16QAM, R=1/2,3/4 |
| Channel model | Vehicular A (VA) 120km/h |
| Channel estimation | MMSE |
| Antenna pattern of each sector [12] (horizontal) | θ3dB = 70 degrees, Am = 20 dB |
| The resultant relative gain considering inter-sector interference (sec) | sec=6, 9 , 12, 15 dB |

(a) Antenna gain of each sector                       (b) sec value

Figure 4 – Antenna gain and sec value[2]

Figure 5 – Throughput performance (1x2 SIMO, VA120km/h)

# Conclusions

We presented the improved performance of the sector-specific downlink common pilots compared to the non sector-specific pilots based on 16e in link-level simulation evaluation considering inter-sector interference based on the Pilot Evaluation criteria [1].

Hence, we propose to include the sector-specific common pilots using sector specific offset and/or orthogonal sequence in TGm SDD in order to achieve throughput requirements.

# Text Proposal

Insert the following text into SDD Section 11 – Physical Layer [9]

-------------------------------  Text Start ---------------------------------------------------

11.x: Downlink Common Pilot patterns

Downlink common pilots shall have the sector-specific common pilot structure using sector specific offset and/or orthogonal sequence

-------------------------------  Text End  ---------------------------------------------------

# References

[1]IEEE C80216m-08/518, “Rapporteur Group for Project 802.16m Downlink Physical Structure Chairs Report.”

[2]3GPP TSG RAN WG1 Meeting #46, R1-062099

[3]IEEE C80216m-08/172r1, Nortel, “Proposal for SDD text for IEEE 802.16m DL Physical Structure”

[4]IEEE C802.16m-08/121r1, Intel, “Symbol structure design for 802.16m - resource blocks and pilots”

[5]IEEE C80216m DL_PHY-08/25r2, LGE, “Proposal for IEEE 802.16m Downlink Pilot Structure for MIMO”

[6]IEEE C802.16m-08/512r1, Ericsson, “Results and Considerations for Downlink Pilot Structures.”

[7]IEEE C802.16mDL_PHY-08/040, Panasonic, “Sector-Specific Common Pilot Structure in IEEE802.16m Downlink.”

[8]IEEE C802.16mUL_PHY-08_004, Intel, “UL Physical Structure Design: 802.16m SDD Text Proposal from Intel”

[9]IEEE 802.16m-08/003, “The Draft IEEE 802.16m System Description Document”

[10]IEEE 802.16m-07/002r4, “802.16m System Requirements”

[11]IEEE Std 802.16e-2005.

[12]IEEE 802.16m-08/004r1, “IEEE 802.16m Evaluation Methodology Document (EMD)”