| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Baseline Content on the Uplink Control Structure for the 802.16m SDD |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | Roshni Srinivasan, Andrea Bacioccola Project 802.16m UL Control Rapporteur Group Chairs | roshni.m.srinivasan@intel.com andrea.bacioccola@nokia.com |
| Re: | IEEE 802.16m-08/023, Charter and Scope of New TGm Rapporteur Groups |  |
| Abstract | Consolidated SDD text on the UL Control Structure based on contributions submitted to the 802.16m UL control structure in session #55 and the 802.16m Rapporteur Group for UL Control is provided in this contribution. |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Baseline Content on the Uplink Control Structure for the 802.16m SDD

Roshni Srinivasan, Andrea Bacioccola

Project 802.16m UL Control Rapporteur Group Chairs

# Introduction

Text to be included in Section 11 of the 802.16m System Description Document (SDD) [1] under the UL Control Structure section is proposed in this contribution. Input from TGm members’ contributions on the 802.16m UL control structure in session #55 [2], contributions and discussions in the UL Control Rapporteur Group [3], as well resolutions to comments reviewed during the afternoon session of UL Control Rapporteur Group discussions on Tuesday, July 15, 2008 in TGm Session #56 are captured.

# Editors’ Notes

In order to provide a general framework for the DL control structure, content in this contribution has been organized by function. Reference to specific solutions and terminology associated with these solutions has not been included. Text describing procedures for implementation has also not been included.

Black font is used for consolidated text. Alternatives that require further harmonization or resolution by TGm are identified by colored, bracketed text and relevant editors’ notes.

Dependencies on text in other sections of the SDD that are still under development have been captured in notes wherever applicable.

# References

- IEEE 802.16m-08/003r3, Project 802.16m System Description Document (SDD)

- C802.16m-UL_ctrl-08/002r1, Initial Contribution List for Project 802.16m Rapporteur Group for Uplink Control

- IEEE C802.16m-08/726, Project 802.16m UL Control Structure Rapporteur Group Report

# UL Control Structure

Details of the UL control structure are described in the following sections.

# UL Control Information Classification

The UL control channels carry multiple types of control information to support air interface layer procedures. Information carried in the control channels is classified as follows.

# Editors’ Notes:

# Text included in this section depends on SDD text being developed by other Rapporteur Groups (MIMO, HARQ).

# Channel quality feedback

Channel quality feedback provides information about channel conditions as seen by the MS. This information is used by the BS for link adaptation, resource allocation, power control etc. Channel quality measurement includes narrowband and wideband measurements. CQI feedback overhead reduction is supported through differential feedback or other compression techniques. Examples of CQI include Physical CINR, Effective CINR, band selection, etc. Channel sounding can also be used to measure uplink channel quality.

# MIMO feedback

MIMO feedback provides spatial characteristics of the channel that are required for MIMO operation. The precoder matrix index, precoding vector index, rank adaptation, channel covariance matrix elements, power loading factor, eigenvectors and channel sounding are examples of MIMO feedback information.

# HARQ feedback

HARQ feedback (ACK/NACK) is used to acknowledge DL transmissions. Multiple codewords can be acknowledged in a single ACK/NACK transmission.

# Synchronization

UL synchronization signals are needed to acquire UL synchronization during initial access or handover and also to periodically maintain synchronization. This includes reference signals for measuring and adjusting the uplink timing offset.

# Bandwidth request

Bandwidth requests are used to provide information about the needed uplink bandwidth to the BS. Bandwidth requests are transmitted through indicators or messages. Bandwidth request messages can include information about the status of queued traffic at the MS such as buffer size and quality of service, including QoS identifiers.

# MBMS feedback

# Editors’ Notes :

# This section is a placeholder for text to be developed based on SDD text that will be added to Section 15 of the SDD (Support for Enhanced Multicast Broadcast Service).

MBMS feedback provides information for DL MBMS transmission to one or multiple cells.

# UL Control Channels

# Editors’ Notes:

# Text included in this section depends on SDD text being developed by other Rapporteur Groups (MIMO, HARQ).

# UL Fast Feedback Channel

The UL fast feedback channel carries channel quality feedback and MIMO feedback. Transmission of UL bandwidth request information on the UL fast feedback channel is FFS.

# The UL fast feedback channel is classified into multiple logical channels per user. The mapping of these logical channels into one or multiple physical channels is FFS.

# Multiplexing with other control channels and data channels

The UL fast feedback channel is FDM with other UL control and data channels.

The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an MS can be periodic and the allocations are configurable. For periodic allocations, the specific type of feedback information carried on each fast feedback opportunity can be different.

The UL fast feedback control channel carries one or more types of fast feedback information. The use of TDM/FDM or CDM to multiplex fast feedback channels from one or more users is FFS.

# PHY structure

Transmission on the fast feedback channel can be event-driven.

The transmission format of the fast feedback channel can be adaptive.

The structure of the resource blocks, pilots and resource mapping for the primary and secondary UL fast feedback control channels are TBD.

# UL HARQ Feedback Channel

This channel is used to carry HARQ feedback information.

# Multiplexing with other control channels and data channels

The UL HARQ feedback channel starts at a pre-determined offset with respect to the corresponding DL transmission.

The UL HARQ feedback channel is FDM with other control and data channels.

CDM is used to multiplex multiple HARQ feedback channels.

# PHY structure

The structure of UL HARQ feedback channel resource blocks, pilots and resource mapping are TBD.

# Editors’ Notes:

The following text is consistent with option 2 proposed for the multiplexing of multiple HARQ feedback control channels in 11.x.2.2.1. If option 2 is selected in 11.x.2.2, this text must be added.

[The design of the spreading code used to multiplex multiple HARQ feedback channels is TBD.]

# UL Sounding Channel

The UL sounding channel is used by an MS to send a sounding signal for MIMO feedback, channel quality feedback and acquiring UL channel information at the BS.

# Multiplexing with other control information and data

The BS can configure an MS to transmit an UL sounding signal on specific UL sub-bands.  The sounding signal is transmitted over predefined subcarriers within the intended sub-bands.  The periodicity of the sounding signal for each MS is configurable.

The UL sounding channel is FDM with other control and data channels.

# PHY structure

Sounding from single or multiple antennas and multiple users are supported to provide full MIMO channel information for DL transmission. Power allocation, sounding sequence design and mapping to subcarriers is TBD.

# UL Ranging Channel

The UL ranging channel is used for UL synchronization. The UL ranging channel can be further classified into ranging channel for non-synchronized mobile stations and synchronized mobiles stations. A random access procedure, which can be contention based or non-contention based is used for ranging. Contention-based random access is used for initial ranging, periodic ranging and handover. Non-contention based random access is used for periodic ranging and handover.

# Ranging Channel for non-synchronized mobile stations

The ranging channel for non-synchronized MSs is used for initial access. The use of the ranging channel for non-synchronized MSs for handover is FFS.

# Multiplexing with other control channels and data channels

The UL ranging channel for non-synchronized MSs starts at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for non-synchronized MSs is FDM with other UL control channels and data channels.

# PHY structure

The ranging sequence design and mapping to subcarriers are TBD.

# Ranging Channel for synchronized mobile stations

The ranging channel for synchronized MSs is used for periodic ranging and handover.

# Multiplexing with other control channels and data channels

The UL ranging channel for synchronized MSs starts at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for synchronized MSs is FDM with other UL control channels and data channels.

# PHY structure

The ranging sequence design and mapping to subcarriers are TBD.

    - Bandwidth Request Channel

Contention based [or non-contention based] random access is used to transmit a bandwidth request indicator on this control channel. [Inclusion of addition information in a bandwidth request indicator such as bandwidth request size, MS-ID, flow identifier, UL transmit power report and CINR report is FFS.]

# Multiplexing with other control channels and data channels

The bandwidth request channel starts at a pre-determined location with the configuration defined in a DL broadcast control message.

The bandwidth request channel is FDM with other UL control and data channels.

# PHY structure

The structure of bandwidth request channel resource blocks, pilots and resource mapping are TBD. The ranging sequence design and mapping to subcarriers are TBD. [The ranging sequence can be defined per user class, per service class and per sector. Users are classified based on operator-specific network performance requirements and service level agreements.]

# UL Inband Control Signaling

UL control information can be multiplexed with data on the UL data channels as MAC headers or MAC management messages. Inband control signaling can contain information such as UL bandwidth requests or bandwidth assignment updates.

# Mapping of UL control information to UL control channels

# Editors’ Notes:

# This table needs to be updated as the mapping of UL control information to UL control channels is developed.

| Information | Channel |
| --- | --- |
| Channel quality feedback | UL Fast Feedback Channel |
| MIMO feedback |  |
| [HARQ feedback FFS] |  |
| Bandwidth request FFS |  |
| HARQ feedback | UL HARQ Feedback Channel |
| Sounding | UL Sounding Channel |
| Synchronization | UL Ranging Channel |
| Bandwidth request | Bandwidth Request Channel |
|  | Mapping to other control channels (e.g. Fast Feedback) FFS |
|  | UL Inband Control Signaling |
| MBMS feedback | FFS |