| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink Interference Suppression among Neighbor Cells |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Qi Li, Hongjie Si, Martin Liu Huawei | E-mail: sihongjie@huawei.com |
| Re: | IEEE 80216m-08_024 Call for Comments and Contributions on Project 802.16m System Description Document (SDD) Topic: Interference Mitigation |  |
| Abstract | This contribution proposes an interference suppression scheme in uplink for 16m. |  |
| Purpose | Discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink Interference Suppression among Neighbor Cells

Qi Li, Hongjie Si, Martin Liu

Huawei

# Introduction

16m SRD [1] required more improvement over 16e system in terms of the peak data rate, average sector throughput, and cell-edge user throughput. However, the uplink ICI (inter-cell interference) is a considerable problem to limit further improvement on uplink performance. Especially with universal frequency reuse, the uplink signal from cell edge users will significantly impact on the adjacent cells. To solve this problem, multi-BS cooperative receiver has been proposed to perform jointly reception between multiple coordinated BSs over the same radio resources. Obviously, this scheme requires exchanging traffic and control information between BSs over backhaul, therefore large overhead exists and is hardly to be reduced.

The transmit antennas of MS can be two in the extended MIMO configuration in 16m SRD. In the future, the transmit antennas of MS may be not less than two. To mitigate overhead between coordinated BSs, we propose an uplink interference cancellation design which perform MS precoding to zero force signals to adjacent cells. The proposed scheme just need channel information exchange between coordinated BSs, and so overhead could be reduced compared to multi-BS cooperative receiver.

# Proposed scheme

When a MS is on cell edge, the serving cell and adjacent cells can simultaneously receive uplink signal over same radio resources. To improve cell throughput, it is expected that the signals non-serving cell received are minimized. Precoding is considered as a reasonable method to suppress signals towards non-serving cells. For example, as Figure 1, when one MS is located on edge of two cells, cell 1 is assumed to be serving cell and cell 2 is interfered cell, precoding is performed at MS to cancel interference to cell 2.

Figure 1  An illustration of cell edge user

In Figure 1, uplink channel from MS to BS 1 and BS 2 is denoted as  and . To cancel the interference to cell 2, the signal that BS 2 received from the MS is expected to be suppressed dramatically. For this aim, precoding is utilized according to following operations:

- BS 2 estimates the channel  directly from the MS pilots or other reference signals.

- BS 2 transmits estimated channel information  to BS 1 through backhaul or other paths.

- BS 1 computes the precoding matrix  of MS which is the null space to .

- BS 1 transmits  to MS on downlink channel.

- MS precodes transmitting data with matrix .

Note that the precoding matrix  should be a nonzero matrix, otherwise, no signal is transmitted. To guarantee the existence of a nonzero precoding matrix, a sufficient condition is that the number of the transmit antennas of MS is larger than the number of receive antennas of BS 2. But in general, BS has four receive antennas or more. If the MS has two transmit antennas, just one receive antenna is limited at BS 2. So antennas selectivity or antennas combination could be adopted to generate one effective receive antenna from four physical receive antennas at BS 2.

In TDD, overhead of inter-BS channel information transmission can be avoided according to reciprocity property. Downlink estimate could be used for uplink transmission. In this way, MS can acquire the channel information to BS 2. But the null space computation is implemented at MS. So the overhead reduction of inter-BS is at the cost of complexity of MS.

# Potential System Impact

- Pilot design and channel measurement

In proposed scheme, each BS should not only perform channel estimation to MSs belonging to itself but also to MSs belonging to adjacent cell. For this purpose, it is preferred to use orthogonal pilots among adjacent cell so that the uplink channels can be separated correctly.

- Overhead on the backhaul

Adjacent BSs needs channel information exchange through backhaul, which will cause additional overhead on the backhaul.

- Receive antennas process

To guarantee the existence of null space, a linear process is performed at BS to make several physical receive antennas be equivalent to one receive antenna. For example, the linear process includes antennas selectivity and antennas combination.

- Null space computation

To suppress interference to adjacent cell, null space is computed as precoding vector. In general, the size of effective uplink channel matrix is 1x2, in which effective receive antennas of BS is equivalent to one. SVD could be used to calculate the null space vector, and the complexity of this process is acceptable since small size of effective channel matrix.

# Conclusions

The proposed scheme can be used at the cell edge for better uplink ICI control and to improve the cell-edge uplink throughput. Therefore, we recommend supporting uplink interference suppression at neighbor cells in IEEE 802.16m.

# References

[1] 80216m-07_002r4, IEEE 802.16m System Requirements.

# Text Proposal to SDD

Include the following text in Section 11.x.x of IEEE 802.16m-08_003r3.

------------------------------------Start text proposal------------------------------------

11.x.x  Interference mitigation

For cell edge users, nulling precoding is utilized to suppress uplink interference to neighbor cells.

------------------------------------End text proposal------------------------------------