| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Obtaining Downlink Channel Knowledge for MIMO-OFDM System Using Hybrid Channel Feedback |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Mingyang Sun, Yinggang Du, Martin Liu, Yunsong Yang, Tao Wu, Xin Chang, Xueqin Gu, Yuanjie Li, Yi Wang Huawei | Voice: + 86 755 28970181 E-mail: duyinggang@huawei.com |
| Re: | IEEE 802.16m-08/024 Call for Comments and Contributions on Project 802.16m System Description Document (SDD), for “Uplink MIMO schemes” topic. |  |
| Abstract | The contribution presents a technique of obtaining downlink channel state information technique using hybrid channel feedback and provides the base station with complete broadband channel information. |  |
| Purpose | To incorporate the proposals into the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Obtaining Downlink Channel Knowledge for MIMO-OFDM System Using Hybrid Channel Feedback

Mingyang Sun, Yinggang Du, Martin Liu, Yunsong Yang, Tao Wu, Xin Chang, Xueqin Gu, Yuanjie Li, Yi Wang

Huawei

# Introduction

In order to achieve the maximal system capacity, complete broadband channel knowledge between all transmit antennas and all receive antennas is required at the base station. In Orthogonal Frequency Division Multiplexing (OFDM) systems, this means that channel knowledge is required at each of the K frequency-domain subcarriers.

Obtaining complete channel knowledge at the base station in Time Division Duplexing (TDD) systems is relatively straightforward using uplink sounding as long as the uplink and downlink arrays are calibrated at the base station. In uplink sounding, the mobile transmits a training sequence from each of its antennas (possibly on the same time-frequency resource). Then the base station estimates the uplink channel from each of the mobile antennae to each of the base station receive antennae using standard channel estimation techniques. The base station then translates the uplink channel into a downlink channel based on information obtained from array calibration.

However, usually mobile stations have more receiver antennas than transmitter antennas. So, even in TDD mode, base station cannot have full channel status information by using channel reciprocity because each mobile station can transmit by only part of its antennas. For example, if a base station has K transmission antennas & K reception antennas (which are identical), and a mobile station has m transmitter antennas & n receiver antennas (n > m). Then, for down link MIMO transmission the base station requires to know the channel status information of Kxn channel elements. However, because the mobile station can transmit back in the reverse link by using only m antennas, the base station can have channel status information of Kxm channel elements.

So, it needs another way of feedback mechanism for TDD case if n > m. There are some conventional ways for this case:

- Do not utilize channel reciprocity property. This implies use of feedback mechanism which is used in FDD mode such as codebook feedback. In this case, it will result in more overhead because of not using the advantage of TDD systems.

- Use CSI for Kxm channel elements only. In this case, because a base station can identify only part of channel information, its transmission efficiency will be lower than full CSI information.

- Transmit antenna switching at mobile station. In this case, mobile station switches transmission antennas periodically such that within the period the base station can identify full Kxn channel elements. However, in this case it takes more time for the base station to identify whole channel information, and also mobile station needs additional implementation cost.

This contribution introduces a bandwidth efficient means of obtaining complete channel knowledge in TDD system and provides the base station with complete broadband channel information. This method is called hybrid channel feedback (HCFB). The downlink channel information received by non-transmission antenna is carried by sounding channel of transmission antenna.

# Proposed hybrid channel feedback (HCFB) scheme

The BS can obtain the downlink channel information via following processing procedure:

- Figure1 Pilot feedback system

We assume downlink antenna configuration is M*N and uplink antenna configuration is m*n, the sounding channels of multiple uplink users are multiplexed by CDM. As shown in figure 1, the downlink pilots are transmitted from the BS. The downlink pilots received by the MS are:

(1)

(2)

Where  is the downlink pilots received by the i-th MS, n*1 matrix,  is downlink channel matrix, n*M matrix, Pdlkis downlink pilot, M*1 matrix and  is additive gauss noise.

Assuming M=4, N＝2, m＝1，n＝2, so

- ，                            （3）

- and  is separately the downlink pilots received by the first and second antenna,  and  is downlink channel matrix received by the first and second antenna,  and  additive gauss noise received by the first and second antenna.

We assume the first antenna is transmission antenna, then  is multiplied by an orthogonal code then is added to the uplink pilots in the uplink pilots channel correspondingly and the hybrid pilots are transmitted to the BS.

- （4）

- （5）

- （6）

Where YBSk is hybrid pilots received by the BS from multiple users, N*1 matrix,  is uplink channel matrix from i-th user to BS, N*m matrix,  is uplink pilot of the i-th user and  is orthogonal code of the i-th user.

The downlink pilots and uplink pilots satisfy following orthogonal conditions:

npul(t)pul(n−t)=a                      n=00             0<n<Lul                                         (7)

npdl(t)pdl(n−t)=a                      n=00             0<n<Ldl                                         (8)

npul(t)pdl(n−t)=0             0<n<Lul                                          (9)

Where Ldlis the maximum length of downlink channel. Lulis the maximum length of uplink channel. With the orthogonality between the uplink pilots and downlink pilots, the BS can estimate the uplink and downlink channel parameters separately.

Several pilot design schemes are provided to satisfy above orthogonal conditions:

1. Uplink pilot is cycle shift of downlink pilot sequence. When an interval of the cycle shift is longer than the multi-path channel delay the receiver can extract the multi-path channel of uplink and downlink from hybrid pilots using time-window based on the interval.

2. Uplink pilot and downlink pilot are spreaded by different orthogonal codes and are added to hybrid pilots.

3. Downlink pilot is spreaded by an orthogonal code and is added to the uplink pilots. When spread factor is enough large, downlink channel estimation is not affected by uplink sounding signal. So the BS can estimate the uplink and downlink channel parameters separately.

4. Different uplink pilots are spreaded by different orthogonal codes.

First BS estimates the uplink channel parameters:

- （10）

（11）

After uplink channel parameters is estimated, BS may estimate the downlink channel information from BS to the first antenna by channel reciprocity.

Then BS estimates the downlink channel parameters from BS to the second antenna:

- （12）

- （13）

Because Hulk is known, so  may be acquired.

- (14)

To mitigate destruction of pilot sequences orthogonality due to frequency selective fading, we propose three kind of enhanced technique:

- DFT is applied before the downlink pilots are transmitted from the BS and received downlink pilots is added to uplink pilots with DFT processing.

- Base station spreads the downlink pilots over narrowband bandwidth before the downlink pilots are transmitted from the BS and MS de-spreads the downlink pilots first, then re-spreads it with user orthogonal code and adds it to uplink pilots.

- Narrowband downlink pilots are transmitted from the BS and random hops across entire bandwidth, MS feedbacks part downlink channel state information.

# Advantage

In comparison with codebook based channel state information feedback, the hybrid channel feedback technique provides the base station with full broadband channel state information in the case of channel unreciprocity in TDD system and enable base station to achieve better system capacity using the closed-loop multiple data stream MIMO techniques, e.g. MU-MIMO.

In addition, the channel state information is processed in base station and mobile complexity is reduced.

# Conclusion

In IEEE 802.16 SDD, channel state information feedback should be discussed. It would be worth introducing the hybrid channel feedback (HCFB) in SDD as a new channel state information feedback technique.

# Text Proposal to SDD

------------------------------------Start text proposal------------------------------------

[Adopt the following text in the ToC of P802.16m System Description Document (SDD)]

11.x.y Channel state information feedback

11.x.y.z Based on non-codebook channel state information feedback

Non-codebook channel state information feedback mode should be supported, e.g. hybrid channel feedback (HCFB), i.e., the downlink channel information received by non-transmit antenna is carried by sounding channel of transmit antenna.

------------------------------------End text proposal------------------------------------

# References

[1] IEEE 802.16m-07/002r4, IEEE 802.16m System Requirements, 2007-10-19