| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO Mode Selection for UL Feedback Channel |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Yinggang Du, Allan Xu Huawei | E-mail: {duyinggang, zhier81} @huawei.com |
| Re: | IEEE 802.16m-08/024: Call for Comments and Contributions on Project 802.16m System Description Document (SDD) . Target topic: “Uplink MIMO schemes” |  |
| Abstract | MIMO mode selection is mainly used in traffic channel, here we recommend UL feedback channel also can support different MIMO modes. |  |
| Purpose | Discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO Mode Selection for UL Feedback Channel

Yinggang Du, Allan Xu

Huawei

# 1. Introduction

On traffic channel, different MIMO modes can be selected, for example, SM, STTD, D-STTD, etc. And BS shall indicate the selected MIMO mode in assignment message, so MS can receive/send data according the indicated MIMO mode. But for control channel, informing the MIMO mode indicates some problems: if we support different MIMO modes on assignment message, how to indicate the selected MIMO mode? We first indicate the MIMO mode on assignment message, then MS can decode the assignment message, the performance is not good. If we support different MIMO modes on DL/UL control channel, for example, DL/UL ACK, power control, CQICH etc, the overhead is big problem. In this proposal, we can support selection of MIMO mode for UL feedback channel, and at the same time the overhead is reasonable.

# 2. Proposed Solution

Because MIMO mode selection on assignment message and BCH impacts performance badly, here we just let it open. We just focus on other control channel, for example, UL feedback channel.

MS or BS can selection different MIMO modes according different channel conditions, for example, BS schedules some MSs with SM mode to increase throughput when channel condition is good enough. But at some other time BS schedules some 	MSs with STBC/SFBC mode to improve performance when channel condition is not so good. But BS shall indicate the selected MIMO mode when it schedules the MSs, if we introduce this mechanism into control channel, the overhead is so large. For example, for ACK channel, one bit information is transmitted, but BS shall indicate the MIMO mode, so the overhead is unacceptable.

So here we consider the channel condition doesn’t change so fast, that is if BS selects STBC for one MS for one UL feedback channel, then BS will select same MIMO mode for this MS for same UL feedback channel at next time. So we just assume semi-static MIMO mode for UL feedback channel. If BS indicates the MIMO mode in semi-static mode, then the overhead decreases, at the same time UL feedback channel can also use MIMO technology to enhance the UL feedback channel performance or accommodate more UL feedback information according different channel condition. If BS wants minimum overhead of indicating MIMO mode for UL feedback channel, it can just indicate MSs the MIMO mode information in BCH.

Considering UL baseline is 1 TX * 2 RX, so MIMO mode selection is optional for UL feedback channel.

# Text Proposal

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Insert the following text into SDD Section 11 – Physical Layer]

11.x.x: MIMO mode selection for UL feedback channel

MIMO mode selection for UL feedback channel is supported. The method of indicating the selected mode is FFS.

-------- End of proposed text ---------------------------------------------------------------------------------------------