| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SDD Text on Uplink MIMO Schemes |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Shaohua Li, Xin Qi, Jianfeng Kang, Peter Wang, Chao Wei, Nokia Siemens Networks Zexian Li Nokia | shaohua.li@nsn.com Zexian.li@nokia.com |
| Re: | IEEE 802.16m-08/024 (“Call for comments and contributions on Project 802.16m SDD”) Target topic: “Uplink MIMO schemes” |  |
| Abstract | This contribution provides text on uplink MIMO schemes in project 802.16m system description document (SDD) as relevant to uplink MIMO schemes |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SDD Text on Uplink MIMO Schemes

Shaohua li, Xin Qi, Jianfeng Kang, Peter Wang, Chao Wei

Nokia Siemens Networks

Zexian Li

Nokia

Proposed SDD text:

# 11.x  UL MIMO Transmission Scheme

# 11.x.1  UL MIMO Architecture and Data Processing

The architecture of uplink MIMO on the transmitter side is shown in the Figure 1.

In SU-MIMO, only one user is scheduled in one Resource Unit (RU). In collaborative MIMO, multiple users can be scheduled in one RU.

Figure 1. UL-MIMO Architecture

The encoder block contains the channel encoder and modulator for each codeword.

The MIMO encoder block maps one codeword onto M layers which are fed to the precoding block. A layer corresponds to a spatial stream.

The precoding block maps layers to antennas by generating the antenna specific data symbols according to the selected MIMO mode.

The OFDM symbol construction block maps antenna specific data to the OFDM symbol. The permutation and pilot insertion are implemented in this block.

The UL signaling block contains control information such as MCS selection, MIMO mode selection, the PVI (prefer vector index).

When only one transmit antenna is configured, MIMO encoder and precoder are not present in Figure 1.

# Antenna Configuration

The MS employs one or two transmit antennas. The antenna space is ½ lambda…

# Codeword to Layer Mapping

The number of spatial streams, M, for SU-MIMO is M≤min(NT,NR), where M is no more than 2, NT,NRdenote number of transmitting antenna and receiving antenna, respectively.  For Collaborative MIMO, the collaborative users shall have the same stream number and same MIMO schemes. The number of spatial steams for collaborative MIMO is no more than 2 for each MSs.. For SU-MIMO and collaborative MIMO, only SCW is supported.

# Layer to Antenna Mapping

The layer to antenna mapping can be represented as in the Figure 2.

Antenna 1

Layer 1

Layer M

Layer to Antenna Mapping

…

Antenna NT

…

Codeword to Layer Mapping

Codeword 1

Figure 2. Layer to antenna mapping

The layer to antenna mapping depends on the MIMO scheme used.  The mapping can be defined using the following equation

y=P×S(x),

where P is a pre-coding matrix, S(x) is an STC/SM matrix, and x is the input layer vector.

# Resource mapping

The following table illustrates the MIMO mode permutation for various MIMO schemes.

| MIMO Scheme | Resource Mapping |
| --- | --- |
| Open Loop SU-MIMO | Distributed or Localized |
| Closed Loop SU-MIMO | Localized |
| Open Loop Collaborative MIMO | Distributed or Localized |
| Closed Loop Collaborative MIMO | Localized |

# Transmission for Data Channels

# Single-user MIMO

Single-user MIMO schemes are used to improve per-link performance.

Both open-loop single-user MIMO and closed-loop single-user MIMO shall be supported for the antenna configuration specified in Section 11.x.1.1. A variety of transmission modes are supported in each case.

For open-loop single-user MIMO, both spatial multiplexing and transmit diversity schemes shall be supported.

For closed-loop single-user MIMO, codebook based precoding shall be supported for both TDD and FDD systems.

# Open-loop SU-MIMO

A number of antenna configurations and transmission rates are supported in open-loop SU-MIMO. Among them, 2Tx antennas with rate 1 transmission are defined as Transmit Diversity modes. The operation of these modes is specified in Section 11.x.2.1.1.1. The other modes, including 2Tx antennas with rate 2 transmission, are defined as Spatial Multiplexing modes. The operation of these modes is specified in Section 11.x.2.1.1.2.

For open loop SU-MIMO, the precoding matrix P can be defined using the following equation:

P = I.

I is an NT NT identity matrix, where NT is the number of physical antennas at transmitter.

# Transmit Diversity

In transmit diversity mode, rate 1 STBC/SFBC with 2 transmit antennas shall be supported for open-loop single-user MIMO. The input to the MIMO encoder is represented a 2  1 vector

x=s1s2.

The output of the MIMO encoder is a 2  2 matrix

.

The output of the precoder is a 2  2 matrix

y=z.

# Spatial Multiplexing

In spatial multiplexing mode, rate 2 spatial multiplexing with 2 transmit antennas shall be supported for open-loop single-user MIMO. The input to the MIMO encoder is represented as a 2  1 vector

x=s1s2.

The output of the MIMO encoder is a 2  1 vector

z=x.

The output of the precoder is a 2  1 vector

y=z.

# Closed-loop SU-MIMO

# Precoding technique

In FDD and TDD systems, unitary codebook based precoding shall be supported.

# Collaborative MIMO

Collaborative MIMO schemes are used to enable a resource allocation to communicate data to two or more MSs. 802.16m uses collaborative MIMO to boost system throughput.

Both open-loop collaborative MIMO and closed-loop collaborative MIMO shall be supported for the antenna configurations specified in Section 11.x.1.1.

For open-loop collaborative MIMO, both spatial multiplexing and transmit diversity schemes shall be supported.

For closed-loop collaborative MIMO, codebook based precoding shall be supported for both TDD and FDD systems.

For collaborative MSs, they should have the same stream number and the same MIMO schemes.

# Transmission for Uplink Control Channel

MIMO schemes can be used for Uplink control channels, e.g. fast feedback channel, ACK/NAK channel, and ranging channel.