| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modulation for LDPC Codes |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Rongdao Yu, Martin Liu, Chongli Liu Huawei | Voice: 86-755-28970182 E-mail: liuchongli@huawei.com |
| Re: | IEEE 802.16m-08/024 - Call for Contributions on Project 802.16m System Description Document (SDD), for “Hybrid ARQ” topic. |  |
| Abstract | This document proposes a kind of modulation scheme for LDPC Codes. |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modulation for LDPC Codes

Rongdao Yu ,Martin Liu, Chongli Liu

Huawei

# Introduction

LDPC code is a good code with perfect performance and thought to be a candidate code in WiMax. However, there still remain several problems which prevent the code from standardization, e. g. implementation of HARQ, MIMO and modulation. In this contribution, we have proposed solutions for these problems and also with some evaluation based the theoretical analyzed tools.

# Modulation for LDPC Codes

As a block code, a LDPC code can be illustrated by its parity check matrix or bipartite graph equivalently. The degree distribution (λ, ρ), which indicates the fraction of edges with different degrees, is the most important parameter for the performance [1, 2, 3]. The variable nodes with large degrees have more contribution to the system performance. Following is a parity check matrix of a LDPC and its degree distribution.

H=10101010110111110100011001011011deg23123242

In high order modulation scheme, e. g. 16 QAM or 64 QAM, bits in a symbol may have different transmission power. In Figure 1, we’d like to take 16 QAM as an example to illustrate the new scheme. In Figure 1, we could find that b1 and b3 have larger power than b0 and b2 do.

Figure 1 Constellation of 16 QAM

## Description of the scheme

Figure 2 Structures for the Scheme

In order to illustrate the scheme, here we use the structure as in Figure 2.The data firstly are coded by LDPC encoder and then fed into the multiplexer (MUX in Figure2). In the MUX entity, the bits are mapped into 2 streams by their degrees where the important bits with large degrees are in stream 1 and the rest are in stream 2.

Then the 2 streams are modulated by the Modulation entity.

- Figure 3 Bits of LDPC code mapped into 2 streams by degree in MUX

- (L means large degree, S means small degree)

Figure 3 shows the mapping process in MUX entity, however, it is only an example and the actually mapping relationship is determined by the encoder and probably is different with Figure 3. In the figure, bits with large degrees which are green ones are mapped into Stream 1, and bits with small degrees which are yellow ones would be mapped into Stream 2. Hence the important bits with large degrees are transmitted in Stream 1 and the rest are in Stream 2.

In Figure 1, we could find that b1 and b3 have larger power than b0 and b2 do. Since the bits with larger degrees have more contribution to the decoding process, they would be prior to be mapped onto b1 and b3 while the rest bits with smaller degrees onto b0 and b2.

# Simulation

| Simulation parameter | value |
| --- | --- |
| Bandwidth | 10M |
| FFT size | 1024 |
| CP Ratio | 1/8 |
| Modulation | 16QAM |
| Code Type | LDPC |
| Code Rate | 1/2 |
| Channel | ITU6 |

From the simulation result, we can conclude that the proposed scheme can get 0.5dB gain over than the normal scheme at BER@1e-2, 1dB gain at BER @1e-1.

# Conclusion

In this contribution, the different contribution of bits with different degrees has been investigated and the modulation for LDPC Codes has been presented in this contribution. The simulation of the new modulation scheme is done and the result also shows the new scheme outperform the random mapping.

## References

- Y Cao, Jian Gu, Lin Qi, Dacheng Yang , Degree Distribution Based HARQ for Irregular LDPC, Electronics Letters Vol. 42, Issue 6, March 2006, pp 363-364

- S. Y. Chung, T. J. Richardson, and R. L. Urbanke, Analysis of sum-product decoding of low-density parity-check codes using a Gaussian approximation, IEEE Trans. Inform. Theory, vol. IT 47, Feb. 2001, pp. 657-670.

- T. J. Richardson, A. Shokrollahi, and R. L. Urbanke, Design of capacity approaching irregular low density parity check codes, IEEE Trans. Inform. Theory, vol. 47, Feb. 2001, pp.619 - 637.

# Text Proposal to SDD

------------------------------------Start text proposal------------------------------------

[Adopt the following text in the ToC of P802.16m System Description Document (SDD)]

11. x.x.x  Link Adaptation

For modulation of LDPC code , bits with large degree should be mapped to the bits with high reliability in constellation of 16QAM and 64QAM.

------------------------------------End text proposal------------------------------------