| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Rate Compatible LDPC Codes |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Chongli Liu, Ying Jin, Rongdao Yu Huawei | E-mail: liuchongli@huawei.com jinying12@huawei.com |
| Re: | IEEE 802.16m-08/024:Call for Comments and Contributions on Project 802.16m System Description Document (SDD) Topic: Hybrid ARQ (PHY aspects) |  |
| Abstract | This proposal proposes code rate 1/3 LDPC codes with good performance by splitting and extending code rate 1/2 base model matrix of LDPC codes in 802.16e. |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Rate Compatible LDPC Codes

Chongli Liu, Ying Jin, Rongdao Yu

Huawei

# Introduction

In wireless communication system, channel coding is critical to ensure wireless link reliability. LDPC codes have approaching Shannon limit performance and linear encoding complexity and can support parallel decoding, which makes LDPC codes very suitable for high-rate and reliable wireless communication. In 802.16e [1], LDPC codes have different code rates and support multiple code lengths, where the lowest code rate is 1/2. Since code rate 1/2 is somewhat high, we propose code rate 1/3 LDPC codes and the corresponding IR-HARQ scheme in order to improve link quality and enlarge cell coverage.

# Proposed Scheme

In order to get LDPC codes with good performance and low decoding complexity, the proposed code rate 1/3 base model matrix for LDPC codes is obtained by splitting and extending code rate 1/2 base model matrix for LDPC codes in 802.16e[1]. The size of the base model matrix is and the corresponding base model matrix is given in Figure 1.

Figure 1 Base model matrix for the proposed code rate 1/3LDPC code

The shiftscorresponding to expansion factor in the base model matrix for code rate 1/3 LDPC codes are used to determine the shift sizes for all other code lengths. The shift sizes  for a code size corresponding to expansion factor are derived from as equation below, where f is the index of the code length, and denotes the floor function that gives the nearest integer towards.

(1)

The codeword structure for the proposed LDPC code is shown in Figure 2 below. The parity bits for the proposed LDPC code consist of three parts, parity bits corresponding to code rate 1/2 LDPC code, parity bits obtained by row splitting and parity bits obtained by extending. In order to get rate compatible LDPC codes, the parity bits corresponding to code rate 1/2 LDPC code and the parity bits obtained by row splitting can be interleaved, where the parity bits corresponding to the two parts are partitioned into parity blocks by the unit of expansion factor. The parity blocks are interleaved according to the interleaving pattern, where the parity blocks corresponding to code rate 1/2 LDPC code are ahead of the parity blocks obtained by row splitting. After interleaving, information bits, the interleaved parity blocks and the parity bits obtained by extending are multiplexed into one codeword. The interleaved LDPC codeword structure is shown in Figure 3.Finally; rate compatible LDPC codes can be obtained by puncturing the interleaved codeword.

Figure 2 Codeword structure for the proposed code rate 1/3LDPC code

Figure 3 Interleaved codeword structure for the proposed code rate 1/3LDPC code

The interleaving pattern for the parity blocks corresponding to code rate 1/2 LDPC code and the parity blocks obtained by splitting is{5,14,9,1,7,11,3,15,6,13,10,0,2,12, 8,4},where index 0 denotes the first parity block for the proposed code rate 1/3 LDPC code and index 1 denotes the second parity block for the proposed code rate 1/3 LDPC code, and so on.

For the IR -HARQ scheme, the bit selection for subpacket generation is similar with that of CTC in 802.16e [1], where it is done based on the interleaved codeword structure in Figure 3.

# Simulation Results

The performance for the proposed code rate 1/3 LDPC code and the performance comparison between the IR-HARQ scheme for the proposed LDPC code and chase combining (CC) scheme for code rate 1/2 LDPC codes in 802.16e[1] are evaluated over the AWGN channel. The simulation parameters are given in Table 1 and Table 2. The simulation results are shown in Figures below. Simulation results in Figure 5 show that the gain of the IR-HARQ scheme for the proposed LDPC code is about 0.6dB at BLER = 10-2.

Table 1 Simulation parameters for the proposed LDPC codes

| Code Rate | 1/3 |
| --- | --- |
| Data Length | 288,480,720,1152 |
| Expansion Factor | 24,40,60,96 |
| Decoding Algorithm | Log-BP |
| Maximal Iteration Number | 50 |
| Modulation | QPSK |

Table 2 Simulation parameters for the performance comparison between IR scheme and CC scheme

| Data Length | 1152 |
| --- | --- |
| Expansion Factor | 96 |
| Maximal retransmission Number | 2 |
| Number of bits for first transmission subpacket and retransmission subpacket | 2304 |
| Decoding Algorithm | Log-BP |
| Maximal Iteration Number | 50 |
| Modulation | QPSK |

Figure 4 BLER performances for different data lengths with code rate 1/3

Figure 5 BLER performance comparison for HARQ schemes

# Conclusion

This proposal proposes code rate 1/3 LDPC codes with good performance and low decoding complexity. It can flexibly support multiple code lengths and code rates by puncturing. Since the proposed LDPC codes are designed based on LDPC codes in 802.16e [1], it has good legacy properties.

# Reference

[1] P802.16Rev2/D5 (June 2008) DRAFT Standard for Local and metropolitan area networks: Air Interface for Broadband Wireless Access Systems.

# Text Proposal

--------------------------------------------------- Start of the Text --------------------------------------------------------------

[Adopt the following text in System Description Document (SDD)]

11.x Low density parity check (LDPC) codes

The code rate 1/3 base model matrix for LDPC codes is obtained by splitting and extending code rate 1/2 base model matrix for LDPC codes in 802.16e. The size of the base model matrix is and the corresponding base model matrix is given in Figure xx.

Figure xx  Base model matrix for code rate 1/3 LDPC code

The shiftscorresponding to expansion factor in the base model matrix for code rate 1/3 LDPC codes are used to determine the shift sizes for all other code lengths. The shift sizes  for a code size corresponding to expansion factor are derived from as equation below, where f is the index of the code length, and denotes the floor function that gives the nearest integer towards.

(xx)

The codeword structure for code rate 1/3 LDPC code is shown in Figure xx. The parity bits for the LDPC code consist of three parts, parity bits corresponding to code rate 1/2 LDPC code, parity bits obtained by row splitting and parity bits obtained by extending. In order to get rate compatible LDPC codes, the parity bits corresponding to code rate 1/2 LDPC code and the parity bits obtained by row splitting can be interleaved, where the parity bits corresponding to the two parts are partitioned into parity blocks by the unit of expansion factor. The parity blocks are interleaved according to the permutation pattern, where the parity blocks corresponding to code rate 1/2 LDPC code are ahead of the parity blocks obtained by row splitting. After permutation, information bits, the interleaved parity blocks and the parity bits obtained by extending are multiplexed into one codeword. The interleaved LDPC codeword structure is shown in Figure xx. Finally; rate compatible LDPC codes can be obtained by puncturing the interleaved codeword.

Figure xx Codeword structure for code rate 1/3 LDPC code

Figure xx Interleaved codeword structure for code rate 1/3 LDPC code

The interleaving pattern for the parity blocks corresponding to code rate 1/2 LDPC code and the parity blocks obtained by splitting is{5,14,9,1,7,11,3,15,6,13,10,0,2,12,8,4},where index 0 denotes the first parity block for code rate 1/3 LDPC code and index 1 denotes the second parity block for code rate 1/3 LDPC code, and so on.

--------------------------------------------------End of the Text -----------------------------------------------------------------