| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Power Management for Multi-Carrier Support |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Lina Liu,Junxian Mo,Jia Lin,Jiping Li Huawei Technologies Co.,Ltd Xiaolu Dong, Ying Du CATR Xin Su, Xiaofeng Zhong Tsinghua | E-mail: liulina@huawei.com mojunxian@huawei.com dongxiaolu@mail.ritt.com.cn; duying@mail.ritt.com.cn suxin@mail.tsinghua.edu.cn； zhongxf@tsinghua.edu.cn |
| Re: | IEEE 802.16m-08/024, “Call for Comments and Contributions on Project 802.16m System Description Document (SDD),” – Call for Contributions |  |
| Abstract | A proposal for Power management in Multi-carrier |  |
| Purpose | To be discussed and adopted by TGm for use in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Power management for Multi-Carrier Support

Lina Liu,Junxian Mo, Jia Lin, Jiping Li

Huawei Technologies Co.,Ltd.

Xiaolu Dong, Ying Du

CATR

Xin Su, Xiaofeng Zhong

Tsinghua

# 1. Introduction

In IEEE 802.16 Session #55, the frame structure and general concepts for multi-carrier downlink control were accepted. The carriers involved in a multi-carrier system can be divided into primary carriers and secondary carriers. The resource allocation can span across multiple RF carriers.

This contribution considers the resource assignment for Multi-Carrier.

# 2. Proposal

In the current SDD, a wideband MS may have different capabilities, such as operation over one channel at a time or aggregation across channels, operation over contiguous or non-contiguous. If the primary carrier and the secondary carrier are non contiguous carriers, the wideband MS needs to monitor its primary carrier and secondary carrier using two independent RF chains, and the power consumption is high. To resolve this problem, we propose an MS-based active window concept for the secondary carrier.

- Active window is the time duration on secondary carrier for one wideband MS. The duration can be super frame, frame, sub frame, etc. The MS only opens the RF chain to receive and buffer those frames in the active window.

- The wideband MS may gate the RF chain and should not receive or buffer any frame in the non- active window.

- The MS shall receive its active window information from its primary carrier, because of the delay of decoding, the active window information shall be transmitted earlier than the corresponding active window.

Illustrated below:

Notes:

RF carrier2 is the primary carrier, and the primary carrier provides the active window information for the secondary carriers.The MS should only operate in the active window, e.g. receive and buffer frames. This reduces power consumption at the MS.

The primary carrier can operate using the same sleep mode definitions as defined for single carrier operation.

[Proposed text]

[Insert the following Text in sub-clause 19.1]

## 19.1 Multi-carrier operation Principles

## 19.2 Multi-carrier power saving operation

The power saving operation of primary carrier shall be same as the single carrier. The power saving information of secondary carriers shall be negotiated between the MS and BS through the primary carrier.

The primary carrier shall indicate the active duration on the secondary carrier; the MS should only receive and buffer those frames in the active duration of the secondary carrier. The BS may only schedule data bursts for the MS in its active duration