| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Baseline content on HARQ for 802.16m SDD by Rapporteur Group Chairs for HARQ |  |
| Date Submitted | 2008-07-17 |  |
| Source(s) | Sassan Ahmadi, Shashikant Maheshwari, Thierry Lestable HARQ Rapporteur Group Chairs | sassan.ahmadi@intel.com shashi.maheshwari@nsn.com Thierry.lestable@samsung.com |
| Re: | IEEE 802.16m-08/023, Charter and Scope of New TGm Rapporteur Groups |  |
| Abstract | This contribution provides consolidated text on HARQ based on contributions submitted to the 802.16m HARQ in session#55, and submitted to the 802.16m Rapporteur Group for HARQ (protocol and timing). |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Baseline content on HARQ for 802.16m SDD

Sassan Ahmadi, Shashikant Maheshwari, Thierry Lestable

HARQ Rapporteur Group Chairs

# Introduction

This contribution proposes consolidated text to be included in section 10 within the 802.16m System Description Document (SDD) [1], under the HARQ section.

This relies on the consolidated commentary database [7] obtained through HARQ RG discussions during session#56, whilst taking also into account the contributions from TGm members to the session #55 [2], together with the discussions and additional contributions ([3], [4] and [5]) within the Rapporteur Group, and that are finally captured within the RG report [6].

# Editors’ Notes

Black font is used for consolidated text. However there remains still lot of bracketed text within this contribution which needs to be addressed during TGm, since no clear consensus could be reached.

Besides, special attention should be paid to yellow parts highlighting key decision points of the HARQ SDD.

For sake of clarity, and in order to trigger further progress, a complementary contribution [5] have been submitted by RG, containing overall comments and additional/alternative proposals extracted from TGm member’s contributions to HARQ Rapporteur Group.

# References

- IEEE 802.16m-08/003r3, Project 802.16m System Description Document (SDD)

- C802.16m_08-004-H-ARQ-contribution-and-feature-list.xls

- C802.16m_08-005-H-ARQ-classification-and-comparison-of-solutions.xls

- C802.16m-HARQ-08_008r3, HARQ RG Chairs' initial recommendations on definitions of the HARQ performance metrics as well as recommendations on common assumptions to be used for evaluation of the HARQ proposals

- IEEE C802.16m-08/767r1, Proposed Baseline Content on HARQ for the 802.16m SDD text, with comments

- IEEE C802.16m-08/769r2, Project 802.16m HARQ Rapporteur Group Report

- IEEE C802.16m-08/867.cmt, Final consolidated commentary database

# Proposed SDD text for HARQ functions

SDD Text Proposal for HARQ Protocol and timing, part of MAC Sublayer (section 10).

# HARQ Functions

HARQ is supported in downlink and uplink packet (re)transmissions in both BS and MS to improve robustness and performance.

The HARQ operation relies on N-process (multi-channel) stop-and-wait protocol.

## HARQ in the Downlink

### HARQ Timing and Protocol

[editor’s note: Decision could not be made on the choice of HARQ schemes for downlink, currently 3 options are being considered 1) synchronous, 2) asynchronous, and 3) combination of the previous schemes]

Option1:

IEEE 802.16m uses asynchronous HARQ scheme in the downlink.

Option 2:

IEEE 802.16m uses synchronous HARQ scheme in the downlink.

Option 3:

IEEE 802.16m uses both asynchronous and synchronous HARQ schemes in the downlink.

The following HARQ parameters and their associated values are defined:

- Maximum retransmission delay.: FFS

- Maximum number of retransmissions: FFS

- Maximum number of HARQ processes FFS

- ACK/NACK delay: FFS

[Placeholder for figures illustrating the choice of HARQ scheme(s) in Downlink]

The HARQ ACK/NACK delay is defined for FDD and for each TDD DL/UL ratio and for each mixed mode scenarios

### HARQ Operation with Persistent Allocation

[Editor’s note: This section is provided as place holder. This section will be filled when details of HARQ operation for persistent allocation is presented and discussed]

### HARQ Re-transmissions

[editor’s note: the working assumption will depend on decision taken w.r.t. section 10.x.1.1]

Option-1: For synchronous HARQ, resource at the retransmissions in the downlink can be fixed or adaptive according to control signaling.

Option-2: For asynchronous HARQ, IEEE 802.16m uses an adaptive HARQ scheme in the downlink.  In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, control signaling is required to indicate the resource allocation and transmission format along with other HARQ necessary parameters.

Option-3: For synchronous HARQ, resource at the retransmissions in the downlink can be fixed or adaptive according to control signaling. For asynchronous HARQ, IEEE 802.16m uses an adaptive HARQ scheme in the downlink.  In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, control signaling is required to indicate the resource allocation and transmission format along with other HARQ necessary parameters.

## HARQ in the Uplink

### HARQ Timing and Protocol

[Decision could not be made on the choice of HARQ schemes for uplink, currently 3 options are being considered, 1) synchronous, 2) asynchronous, and 3) combination of the previous schemes]

Option1:

IEEE 802.16m uses asynchronous HARQ scheme in the uplink.

Option 2:

IEEE 802.16m uses synchronous HARQ scheme in the uplink.

Option 3:

IEEE 802.16m uses both asynchronous and synchronous HARQ schemes in the uplink.

The following HARQ parameters and their associated values are defined:

- Maximum retransmission delay: FFS

- Maximum number of retransmissions: FFS

- Maximum number of HARQ processes FFS

- ACK/NACK delay: FFS

[Placeholder for figures illustrating the choice of HARQ scheme(s) in Uplink]

### HARQ Operation with Persistent Allocation

[Editor’s note: This section is provided as place holder. This section will be filled when details of HARQ operation for persistent allocation is presented and discussed]

### HARQ Re-transmissions

[editor’s note: Option will depend on decision taken w.r.t. section 10.x.2.1]

Option-1: For synchronous HARQ, resource at the retransmissions in the uplink can be fixed or adaptive according to control signaling.

Option-2: For asynchronous HARQ, IEEE 802.16m uses an adaptive HARQ scheme in the uplink.  In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, control signaling is required to indicate the resource allocation and transmission format along with other HARQ necessary parameters.

Option-3: For synchronous HARQ, resource at the retransmissions in the uplink can be fixed or adaptive according to control signaling. For asynchronous HARQ, IEEE 802.16m uses an adaptive HARQ scheme in the uplink.  In adaptive asynchronous HARQ, the resource allocation and transmission format for the HARQ retransmissions may be different from the initial transmission. In case of retransmission, control signaling is required to indicate the resource allocation and transmission format along with other HARQ necessary parameters.

## HARQ and ARQ Interactions

[Editor’s note: This section is provided as place holder. This section will be filled when details of HARQ and ARQ interactions is presented and discussed]