| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced HARQ scheme with Signal Constellation Rearrangement |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Ryohei KIMURA*, Christian Wengerter**, Isamu YOSHII*, Katsuhiko HIRAMATSU* * Matsushita Electric (Panasonic) ** Panasonic R&D Center Germany | Voice: +81-50-3687-6584 E-mail: kimura.ryohei@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16m-08/016 - Call for Contributions on Project 802.16m System Description Document (SDD), shoot for “Hybrid ARQ (protocol and timing)” topic. |  |
| Abstract | Ｗe apply a signal constellation rearrangement to the Chase combining scheme for the HARQ scheme supported in IEEE802.16e. We present the basic idea of the proposed scheme and evaluate the packet error rate performance. The performance of the proposed scheme can be drastically improved for both 16QAM and 64QAM. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Enhanced HARQ scheme with Signal Constellation Rearrangement

Ryohei KIMURA*,  Christian Wengerter**,

Isamu YOSHII*, Katsuhiko HIRAMATSU*

* Matsushita Electric (Panasonic)

** Panasonic R&D Center Germany

# Introduction

Chase combining (CC) [1] is one of the HARQ schemes supported in IEEE802.16e [2]. In this contribution, we present a HARQ bit-mapping scheme, called “signal constellation rearrangement (CoRe)” for CC in the case of 16QAM and 64QAM [3][4].

The proposed scheme “CC with CoRe” shows a significant performance gain with respect to the original CC without CoRe. Therefore, CoRe should be included in the system description document (SDD).

# Signal constellation rearrangement for quadrature amplitude modulation

## Basic idea

The 2M-QAM modulated symbol consists of M-bits. For the 2M-QAM with M larger than 2 (e.g. 8QAM (PSK),16QAM, 64QAM, 256QAM,…), the reliabilities of the bits Gray-mapped onto the modulated symbol vary from the most significant bits (MSBs) i1 and q1 to the least significant bits (LSBs) i2 and q2 as shown in Figure 1. As the reliability variations between these two different kinds of bits increase, the error rate performance is getting worse with respect to having equal bit reliabilities. Considering a HARQ scheme retransmitting (at least partially) identical symbols and employing an identical signal constellation and mapping for all transmissions, the variations in bit reliabilities increase over retransmissions (i.e. the original CC). Particularly, this is the case when soft-combining the received packets by maximal ratio combining (MRC) at modulation symbol level or by adding LLRs at bit level. By rearranging the signal constellations for retransmissions the proposed scheme averages out the bit reliabilities over the retransmissions.

### Signal constellation rearrangement for 16QAM and 64QAM

Figure 1 shows the signal constellation in the case of 16QAM supported in IEEE802.16e.  The bit i1, i2, q1 and q2, is located to the 1st bit, 2nd bit, 3rd bit and 4th bit (i.e. i1i2q1q2) within a symbol. Due to Gray-mapping bit i1 and q1 is on average more reliable than i2 and q2.

Figure 1. Signal constellation with Gray-mapping in the case of 16QAM

As we present in the previous section 2.1, there are variations in the mean bit reliabilities depending on the significance of the bit within a symbol. Therefore, we use the simple rearrangement rule such as reordering and inversion of the logical bit values according to Table 1 to average out the bit reliabilities over the retransmissions.

Table 1. Rearrangement rule for each transmission in the case of 16QAM

| Transmission No. | Bit pattern | Explanation |
| --- | --- | --- |
| 1 | i1i2q1q2 | - None |
| 2 | i2i1q2q1 | - Logical inversion of i1 and q1 / swapping i1 with q1 and i2 with q2 |
| 3 | i2i1q2q1 | - Swapping i1 with i2 and q1 with q2 |
| 4 | i1i2q1q2 | - logical inversion of i2 and q2 |
| Further transmission |  | - Repeatedly using the signal constellations form 1st-4th transmissions |

The same rearrangement rule as 16QAM can be applicable for 64QAM. There are 3 level of bit reliabilities in the case of 64QAM. Figure 2 shows the signal constellation in the case of 64QAM supported in IEEE802.16e and Table 2 shows the rearrangement rule for each transmission in the case of 64QAM.

Figure 2. Signal constellation with Gray-mapping in the case of 64QAM

Table 2. Rearrangement rule for each transmission in the case of 64QAM

| Transmission No. | Bit pattern | Explanation |
| --- | --- | --- |
| 1 | i1i2i3q1q2q3 | - None |
| 2 | i2i3i1q2q3q1 | - 1-bit circular shift for the in-phase and orthogonal components individually |
| 3 | i3i1i2q3q1q2 | - 2-bits circular shift for the in-phase and orthogonal components individually |
| 4 | i1i2i3q1q2q3 | - Logical inversion of i2, i3 , q2 and q3 |
| 5 | i2i3i1q2q3q1 | - 1-bit circular shift for the in-phase and orthogonal components individually - Logical inversion of i1, i3 , q1 and q3 |
| 6 | i3i1i2q3q1q2 | - 2-bits circular shift for the in-phase and orthogonal components individually - Logical inversion of i1, i2 , q1 and q2 |
| Further transmission |  | - Repeatedly using constellations form 1st-6th transmissions |

# Simulation results

Table 3 provides a list of simulation parameters.

Table 3. Simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2.5Ghz |
| System bandwidth | 10MHz |
| FFT size | 1024 |
| Sub-carrier frequency spacing(fs) | 10.94kHz |
| Useful symbol interval (Ts=1/fs) | 91.4usec |
| Guard interval (Tg=Ts/8) | 11.4usec |
| Number of information bits for packet | 128bits(16QAM) / 192bits(64QAM) |
| Antenna configuration | 1-by-1 |
| Channel coding | Turbo coding (original rate = 1/3) |
| MCS | 16QAM/64QAM, R=1/2 |
| Channel model | AWGN |
| Channel estimation | Ideal |
| Maximum number of transmissions | 4(16QAM) / 6(64QAM) |

## Comparison between the original chase combining and the chase combining with constellation rearrangement in the case of 16QAM

Figure 3 shows the packet error rate (PER) performance as a function of the average received signal energy per information bit-to-AWGN power spectrum density ratio (Eb/N0) with the number of transmission as a parameter in the case of 16QAM. The performance gain for PER=10-1 of the CC with CoRe over the original CC is about 1.4dB for the 2nd transmission, about 2.1dB for the 3rd transmission and about 2.9dB for the 4th transmission. The performance gain is increasing in proportion to the number of transmissions, because the effect of averaging out the reliabilities is improved.

Figure 3. Packet error rate vs. average received Eb/N0 in the case of 16QAM

## Comparison between the original Chase combining and the Chase combining with constellation rearrangement in the case of 64QAM

Figure 4 shows the PER performance as a function of the average received Eb/N0 with the number of transmission as a parameter in the case of 64QAM. The performance gain for PER=10-1 of the CC with CoRe over the original CC is about 2.1dB for the 2nd transmission, about 3.2dB for the 3rd transmission, about 4.5dB for the 4th transmission, about 5.4dB for the 5th transmission and about 6.0dB for the 6th transmission. The performance gain is improved proportionally to the number of transmissions for the same reason as for the 16QAM case.

Figure 4. Packet error rate vs. average received Eb/N0 in the case of 64QAM

# Conclusion

In this contribution a Chase combining scheme with signal constellation rearrangement is presented. By averaging out the bit reliabilities over the retransmissions, CC with CoRe shows a significantly improved packet error rate performance compared to the original CC for both 16QAM and 64QAM. The performance gain is improving with an increasing number of retransmissions.

We proved the efficiency of applying the CoRe to the CC. Therefore, CC with CoRe shall be described in the SDD as shown below.

---------------------------- Begin Proposed Text --------------------------------------------------------------------------

X. HARQ

A Chase combining scheme with signal constellation rearrangement for 16QAM and 64QAM modulation shall be used.

---------------------------End of Text Proposal ---------------------------------------------------------------------------

# Reference

- D. Chase, “Code combining: A maximum-likelihood decoding approach for combining an arbitrary number of noisy packets,” IEEE Trans. Commun., Vol. COM-33, pp. 385-393, May 1985.

- IEEE Std. 802.16e-2005, “Part 16: Air Interface for Broadband Wireless Access Systems,” Approved 7 December 2005.

- 802.16m-07/292r1, ITRI, “Enhanced HARQ technique using Constellation Rearrangement”

- R1-01-0237, Panasonic, “enhanced HARQ method with signal constellation rearrangement,” 3GPP TSG RAN WG1, Las Vegas, USA February 27- March 2, 2001.

- S. Le Goff, A. Glavieux, C. Berrou, “Turbo-codes and high spectral efficiency modulation,” IEEE SUPERCOMM/ICC ’94, vol.2, pp.645-649, 1994

- Ch. Wengerter, A. Golitschek Edler von Elbwart, E. Seidel, G. Velev, M.P. Schmitt, “Advanced hybrid ARQ technique employing a signal constellation rearrangement,” IEEE VTC 2002 Fall, vol. 4, pp. 2002-2006 , 2002.

# Appendix

One possible implementation described in Section 2.2 is to use the same the signal constellation for all transmissions as shown in Figure 1 and reorder and inverse the logical bit values according to Table 1. There is another implementation that the ordering of the bit-to-symbol mapping is identical for all transmissions, however, the signal constellations for each transmission are changed. In this section we analyze the advantageous effect of the LLR averaging for the quadrature amplitude modulation, based on this implementation in the case of 16QAM and 64QAM. It is noted that this implementation can achieve an identical averaging of the bit reliabilities as described in Section 2.

The LLR which is a soft-metric for the reliability of a demodulated bit b(=0 or 1) from a received modulation symbol r = x+ jy is defined as follows[5],

LLR(bp)=lnPr(b=1|r)Pr(b=0|r), (1)

where p in bp denotes the bit position. As can be seen from Figure 1, the mappings of the in-phase component bits and the quadrature component bits on the signal constellation are orthogonal. Therefore, it is sufficient to focus on the in-phase component bits i1 and i2. The same conclusions then apply for q1 and q2. The LLR is given by the following equations [6],

LLR(bp)=lnPr(b=1|r)Pr(b=0|r)=lnb=1exp−K(x−xk(1))2b=0exp−K(x−xk(0))2, (1)

where x denotes the in-phase component of the normalized received modulation symbol r, xk(b) denotes the in-phase component of the normalized transmit modulation symbol and K is a factor proportional to the signal-to-noise ratio(SNR).

By using the following approximation,

lnj−b=1expzj≈max−zj=min(zj), (2)

equation (1) can be approximated by the following equation,

LLR(bp)≈Kminb=0x−xk(0)2−minb=1x−xk(1)2 =Kminb=0xk(0)2−2xk(0)x−minb=1xk(1)2−2xk(1)x. (3)

## 16QAM case

Under the assumption of a uniform signal constellation x1 = 3x0 in the case of 16QAM equations (1) can be fairly good approximated by the following equations.

LLR(b=i1)≈−8Kx0x−8Kx02−4Kx0x−8Kx0x+8Kx02, (x<−2x0),(−2x0≤x<2x0), (2x0≤x)LLR(b=i2)≈4Kx0x−8Kx02−4Kx0x−8Kx02, (x<0), (0≤x) , (4)

Table 3 shows the mean LLRs for bits mapped on the in-phase component of the signal constellation for the mapping in the case of 16QAM as shown in figure 1 according to equation (3) (substituting 4Kx02 by ). In case of transmitted modulation symbols 01q1q2 and 11q1q2, where q1 and q2 are arbitrary, the magnitude of the mean LLR (i1) is higher than of the mean LLR (i2). This represents that the actual LLR for i1 depends on the content of i2; e.g. in Figure 1 i1 has a higher mean reliability in case the logical value for i2 is equal to “1” (the leftmost and rightmost columns). Hence, assuming a uniform distribution of transmitted modulation symbols, on average, 50 % of the MSBs i1 have about three times the magnitude in LLR of i2.

Table 3. Mean LLRs for bits mapped on the in-phase component of the signal constellation for the mapping in figure 1 accoding to equation (3)

| Symbol(i1i2q1q2) | Mean value of x | Mean LLR (i1) | Mean LLR (i2) |
| --- | --- | --- | --- |
| 00q1q2 | x0 | −4Kx02=−A | −4Kx02=−A |
| 01q1q2 | x1 | −16Kx02=−4A | 4Kx02=A |
| 10q1q2 | -x0 | 4Kx02=A | −4Kx02=−A |
| 11q1q2 | -x1 | 16Kx02=4A | 4Kx02=A |

From Table 3 it can be observed that there are variations in the mean bit reliabilities for multilevel modulation formats depending on the significance of the bit within a symbol and on the content of the transmitted modulation symbol.

Figure 5 shows the signal constellation in the case of 16QAM for each transmission and Table 4 shows LLRs for bits mapped on the in-phase component of the signal constellation according to the mappings for the 1st transmission to 4th transmission. Assuming a superior decoding performance for uniformly distributed LLRs of all transmitted bits, the objective is to find mapping rules for retransmissions with the constellations from Figure 5 that equalize the mean LLRs after combining. Using the mappings for requested retransmissions according to Table 5 the soft-combined LLRs after requested retransmissions are averaged. This is shown in Table 5 for an AWGN channel, where the soft-combined LLRs for i1 and i2 after each transmission are given with and without applying the CoRe. It is noted that exactly the same principle applies to the q1 and q2 bits mapped onto the quadrature component. From Table 5 it can be observed that in case of 16QAM for an AWGN channel the averaging is perfect after 4 requested transmissions, hence, a total of 4 different mappings is sufficient.

Figure 5. The signal constellation in the case of 16QAM for each transmission

Table 4. LLRs for bits mapped on the in-phase component of the signal constellation according to mapping according to mapping the 1st transmission to 4th transmission

| Symbol (i1i2q1q2) | 1st transmission |  | 2nd transmission |  | 3rd transmission |  | 4th transmission |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i2) | Mean LLR (i2) | Mean LLR (i1) | Mean LLR (i2) |
| 00q1q2 | -A | -A | -A | -4A | -A | -A | -4A | -A |
| 01q1q2 | -4A | A | -A | 4A | -A | A | -A | A |
| 10q1q2 | A | -A | A | -A | A | -4A | 4A | -A |
| 11q1q2 | 4A | A | A | A | A | 4A | A | A |

Table 5. Cumulative LLRs for bits mapped on the in-phase component of the signal constellation for an AWGN channel

| Transmission No. | Corresponding figure | Symbol (i1i2q1q2) | With CoRe |  | Without CoRe |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i1) | Mean LLR (i2) |
| 1 | Figure 5(a) | 00q1q2 | -A | -A | -A | -A |
|  |  | 01q1q2 | -4A | A | -4A | A |
|  |  | 10q1q2 | A | -A | A | -A |
|  |  | 11q1q2 | 4A | A | 4A | A |
| 2 | Figure 5(b) | 00q1q2 | -2A | -5A | -2A | -2A |
|  |  | 01q1q2 | -5A | 5A | -8A | 2A |
|  |  | 10q1q2 | 2A | -2A | 2A | -2A |
|  |  | 11q1q2 | 5A | 2A | 8A | 2A |
| 3 | Figure 5(c) | 00q1q2 | -3A | -6A | -3A | -3A |
|  |  | 01q1q2 | -6A | 6A | -12A | 3A |
|  |  | 10q1q2 | 3A | -6A | 3A | -3A |
|  |  | 11q1q2 | 6A | 6A | 12A | 3A |
| 4 | Figure 5(d) | 00q1q2 | -7A | -7A | -4A | -4A |
|  |  | 01q1q2 | -7A | 7A | -16A | 4A |
|  |  | 10q1q2 | 7A | -7A | 4A | -4A |
|  |  | 11q1q2 | 7A | 7A | 16A | 4A |

## 64QAM case

Under the assumption of a uniform signal constellation x1 = 3x0, x2 = 5x0, x3 = 7x0 for 64QAM, equations (1) can be fairly good approximated by the following equations.

LLR(b=i1)≈−16Kx0x−48Kx02−12Kx0x−24Kx02−8Kx0x−8Kx02−4Kx0x−8Kx0x+8Kx02−12Kx0x+24Kx02−16Kx0x+48Kx02,(x<−6x0),(−6x0≤x<−4x0),(−4x0≤x<−2x0),(−2x0≤x<2x0),(2x0≤x<4x0),(4x0≤x<6x0),(6x0≤x)LLR(b=i2)≈−8Kx0x−40Kx02−4Kx0x−16Kx02−8Kx0x−24Kx028Kx0x−24Kx024Kx0x−16Kx028Kx0x−40Kx02,(x<−6x0),(−6x0≤x<−2x0),(−2x0≤x<0),(0≤x<2x0),(2x0≤x<6x0),(6x0≤x)LLR(b=i3)≈−4Kx0x−24Kx024Kx0x+8Kx02−4Kx0x+8Kx024Kx0x−24Kx02,(x<−4x0),(−4x0≤x<0),(0≤x<4x0),(4x0≤x), (5)

Table 6 shows the mean LLRs for bits mapped on the in-phase component of the signal constellation for the mapping in the case of 64QAM as shown in figure 2 according to equation (4) (substituting 4Kx02 by ). The LLR of i1 and i2 depends on the content of the other bits. On the other hand, The magnitude in LLR of i3 is always constant.

Table 6. Mean LLRs for bits mapped on the in-phase component of the signal constellation for the mapping in figure 2 accoding to equation (4)

| Symbol(i1i2i3q1q2q3) | Mean value of x | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) |
| --- | --- | --- | --- | --- |
| 000q1q2q3 | x1 | −16Kx02=−4A | −4Kx02=−A | −4Kx02=−A |
| 001q1q2q3 | x0 | −4Kx02=−A | −16Kx02=−4A | 4Kx02=A |
| 010q1q2q3 | x2 | −36Kx02=−9A | 4Kx02=A | −4Kx02=−A |
| 011q1q2q3 | x3 | −64Kx02=−16A | 16Kx02=4A | 4Kx02=A |
| 100q1q2q3 | -x1 | 16Kx02=4A | −4Kx02=−A | −4Kx02=−A |
| 101q1q2q3 | -x0 | 4Kx02=A | −16Kx02=−4A | 4Kx02=A |
| 110q1q2q3 | -x2 | 36Kx02=9A | 4Kx02=A | −4Kx02=−A |
| 111q1q2q3 | -x3 | 64Kx02=16A | 16Kx02=4A | 4Kx02=A |

Figure 6 shows the signal constellation in the case of 64QAM for each transmission and Table 6 shows LLRs for bits mapped on the in-phase component of the signal constellation according to the mappings for the 1st transmission to 6th transmission. Assuming a superior decoding performance for uniformly distributed LLRs of all transmitted bits, the objective is to find mapping rules for retransmissions with the constellations from Figure 6 that equalize the mean LLRs after combining. Using the mappings for requested retransmissions according to Table 6 the soft-combined LLRs after requested retransmissions are averaged. This is shown in Table 7 for an AWGN channel, where the soft-combined LLRs for i1, i2 and i3 after each transmission are given with and without applying the CoRe. It is noted that exactly the same principle applies to the q1, q2 and q3 bits mapped onto the quadrature component. From Table 7 it can be observed that in case of 64QAM for an AWGN channel the averaging is perfect after 6 requested transmissions, hence, a total of 6 different mappings is sufficient.

Figure 6. The signal constellation in the case of 64QAM for each transmission

Table 6. LLRs for bits mapped on the in-phase component of the signal constellation according to mapping according to mapping the 1st transmission to 4th transmission

| Symbol (i1i2i3q1q2q3) | 1st transmission |  |  | 2nd transmission |  |  | 3rd transmission |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) |
| 000q1q2q3 | -4A | -A | -A | -A | -4A | -A | -A | -A | -4A |
| 001q1q2q3 | -A | -4A | A | -A | -9A | A | -A | -A | 4A |
| 010q1q2q3 | -9A | A | -A | -A | 4A | -A | -4A | A | -A |
| 011q1q2q3 | -16A | 4A | A | -A | 9A | A | -4A | A | A |
| 100q1q2q3 | 4A | -A | -A | A | -A | -4A | A | -A | -9A |
| 101q1q2q3 | A | -4A | A | A | -16A | 4A | A | -A | 9A |
| 110q1q2q3 | 9A | A | -A | A | A | -4A | 4A | A | -16A |
| 111q1q2q3 | 16A | 4A | A | A | 16A | 4A | 4A | A | 16A |

| Symbol (i1i2i3q1q2q3) | 4th transmission |  |  | 5th transmission |  |  | 6th transmission |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) |
| 000q1q2q3 | -16A | -4A | -A | -A | -16A | -4A | -4A | -A | -16A |
| 001q1q2q3 | -9A | -A | A | -A | -A | 4A | -4A | -A | 16A |
| 010q1q2q3 | -A | 4A | -A | -A | 16A | -4A | -A | A | -9A |
| 011q1q2q3 | -4A | A | A | -A | A | 4A | -A | A | 9A |
| 100q1q2q3 | 16A | -4A | -A | A | -9A | -A | 4A | -A | -A |
| 101q1q2q3 | 9A | -A | A | A | -4A | A | 4A | -A | A |
| 110q1q2q3 | A | 4A | -A | A | 9A | -A | A | A | -4A |
| 111q1q2q3 | 4A | A | A | A | 4A | A | A | A | 4A |

Table 7. Cumulative LLRs for bits mapped on the in-phase component of the signal constellation for an AWGN channel

| Transmission No. | Corresponding figure | Symbol (i1i2q1q2) | With CoRe |  |  | Without CoRe |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) | Mean LLR (i1) | Mean LLR (i2) | Mean LLR (i3) |
| 1 | Figure 6(a) | 000q1q2q3 | -4A | -A | -A | -4A | -A | -A |
|  |  | 001q1q2q3 | -A | -4A | A | -A | -4A | A |
|  |  | 010q1q2q3 | -9A | A | -A | -9A | A | -A |
|  |  | 011q1q2q3 | -16A | 4A | A | -16A | 4A | A |
|  |  | 100q1q2q3 | 4A | -A | -A | 4A | -A | -A |
|  |  | 101q1q2q3 | A | -4A | A | A | -4A | A |
|  |  | 110q1q2q3 | 9A | A | -A | 9A | A | -A |
|  |  | 111q1q2q3 | 16A | 4A | A | 16A | 4A | A |
| 2 | Figure 6(b) | 000q1q2q3 | -5A | -5A | -2A | -8A | -2A | -2A |
|  |  | 001q1q2q3 | -2A | -13A | 2A | -2A | -8A | 2A |
|  |  | 010q1q2q3 | -10A | 5A | -2A | -18A | 2A | -2A |
|  |  | 011q1q2q3 | -17A | 13A | 2A | -32A | 8A | 2A |
|  |  | 100q1q2q3 | 5A | -2A | -5A | 8A | -2A | -2A |
|  |  | 101q1q2q3 | 2A | -20A | 5A | 2A | -8A | 2A |
|  |  | 110q1q2q3 | 10A | 2A | -5A | 18A | 2A | -2A |
|  |  | 111q1q2q3 | 17A | 20A | 5A | 32A | 8A | 2A |
| 3 | Figure 6(c) | 000q1q2q3 | -6A | -6A | -6A | -12A | -3A | -3A |
|  |  | 001q1q2q3 | -3A | -14A | 6A | -3A | -12A | 3A |
|  |  | 010q1q2q3 | -14A | 6A | -3A | -27A | 3A | -3A |
|  |  | 011q1q2q3 | -21A | 14A | 3 | -48A | 12A | 3A |
|  |  | 100q1q2q3 | 6A | -3A | -14A | 12A | -3A | -3A |
|  |  | 101q1q2q3 | 3A | -21A | 14A | 3A | -12A | 3A |
|  |  | 110q1q2q3 | 14A | 3A | -21A | 27A | 3A | -3A |
|  |  | 111q1q2q3 | 21A | 21A | 21A | 48A | 12A | 3A |
| 4 | Figure 6(d) | 000q1q2q3 | -22A | -10A | -7A | -16A | -4A | -4A |
|  |  | 001q1q2q3 | -12A | -15A | 7A | -4A | -16A | 4A |
|  |  | 010q1q2q3 | -15A | 10A | -4A | -36A | 4A | -4A |
|  |  | 011q1q2q3 | -25A | 15A | 4A | -64A | 16A | 4A |
|  |  | 100q1q2q3 | 22A | -7A | -15A | 16A | -4A | -4A |
|  |  | 101q1q2q3 | 12A | -22A | 15A | 4A | -16A | 4A |
|  |  | 110q1q2q3 | 15A | 7A | -22A | 36A | 4A | -4A |
|  |  | 111q1q2q3 | 25A | 22A | 22A | 64A | 16A | 4A |
| 5 | Figure 6(e) | 000q1q2q3 | -23A | -26A | -11A | -20A | -5A | -5A |
|  |  | 001q1q2q3 | -13A | -16A | 11A | -5A | -20A | 5A |
|  |  | 010q1q2q3 | -16A | 26A | -8A | -45A | 5A | -5A |
|  |  | 011q1q2q3 | -26A | 16A | 8A | -80A | 20A | 5A |
|  |  | 100q1q2q3 | 23A | -16A | -16A | 20A | -5A | -5A |
|  |  | 101q1q2q3 | 13A | -26A | 16A | 5A | -20A | 5A |
|  |  | 110q1q2q3 | 16A | 16A | -23A | 45A | 5A | -5A |
|  |  | 111q1q2q3 | 26A | 26A | 23A | 80A | 20A | 5A |
| 6 | Figure 6(f) | 000q1q2q3 | -27A | -27A | -27A | -24A | -6A | -6A |
|  |  | 001q1q2q3 | -17A | -17A | 27A | -6A | -24A | 6A |
|  |  | 010q1q2q3 | -17A | 27A | -17A | -54A | 6A | -6A |
|  |  | 011q1q2q3 | -27A | 17A | 17A | -96A | 24A | 6A |
|  |  | 100q1q2q3 | 27A | -17A | -17A | 24A | -6A | -6A |
|  |  | 101q1q2q3 | 17A | -27A | 17A | 6A | -24A | 6A |
|  |  | 110q1q2q3 | 17A | 17A | -27A | 54A | 6A | -6A |
|  |  | 111q1q2q3 | 27A | 27A | 27A | 96A | 24A | 6A |