| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation with Coordinated Symbol Repetition |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Hidenori KAYAMA*, Isamu YOSHII**, Katsuhiko HIRAMATSU** *Panasonic Mobile Communications R&D Lab. (Panasonic) **Matsushita Electric (Panasonic) | Voice: +81-50-3687-6588 E-mail: kayama.hidenori@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16m-08/024 - Call for Contributions on Project 802.16m System Description Document (SDD), shoot for “Interference Mitigation” topic. |  |
| Abstract | Interference mitigation with coordinated symbol repetition, where repeated symbol mapping pattern is identical among the neighboring cells, is proposed. In this method, an MMSE receiver can eliminate interference signals in not only spatial domain but also frequency domain and requires less numbers of reception antennas than that of the mitigation scheme in spatial domain. This method has more inter cell interference mitigation ability than legacy bit repetition without interference mitigation. This contribution proposes to discuss inter cell interference mitigation method to achieve the system requirement in SDD. |  |
| Purpose | For discussion of inter cell interference mitigation |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Mitigation with Coordinated Symbol Repetition

Hidenori Kayama*, Isamu YOSHII**, Katsuhiko HIRAMATSU**

*Panasonic Mobile Communications R&D Lab. (Panasonic)

**Matsushita Electric (Panasonic)

# Introduction

Improving the cell edge user throughput is important aspect in the requirement [1]. Legacy system employs the bit repetition to improve the signal to interference power ratio (SIR) in cell boundary.

This contribution is the re-submission of [2] and presents interference mitigation with coordinated symbol repetition (CSR) among cells aiming at improved cell edge user throughput. We propose the interference mitigation with CSR as new main functionality on interference mitigation.

# Interference Mitigation with Coordinated Symbol Repetition (CSR) [3]

Figure 1 shows a block diagram of the interference mitigation with CSR. When desired signals on frequency f1 and f2 are identical and interference signals on frequency f1 and f2 are also identical, an MMSE receiver can cancel the interference signals by nature. The MMSE receiver can cancel (RF -1) interference signals where RF is the number of repeated symbols in frequency domain (Repetition Factor).

Figure 2 shows an example of the coordination when RF is two. As shown in Figure 3, frequency domain mapping of the repeated symbols is not limited to the neighboring subcarriers. A frequency domain interleaver can be employed as long as the repeated symbol mapping is identical among the neighboring cells.

Figure 1 Interference mitigation mechanism with the CSR.

Figure 2 Coordinated symbol repetition among cells

Figure 3 Coordinated symbol repetition among cells with frequency symbol interleaver

In terms of calculation complexity, an ordinary MMSE receiver can mitigate inter cell interference. In addition, when identical repeated symbol are mapped in the frequency domain, as long as the channel state doesn’t fluctuate during some time period, the MMSE weight for repetition symbols can be reused throughout that period and this leads to reduce complexity of MMSE weight calculation.

# Comparison with interference mitigation applied to legacy bit repetition

In this section, we consider the case where interference mitigation is applied to the legacy bit repetition and compare with the proposed method.

Figure 4 shows interference mitigation for bit repetition employing spatial interference mitigation with multiple (N) reception antennas. An MMSE receiver can mitigate (N-1) interference signals.

On the other hand, the interference mitigation with CSR, (N*RF-1) interference signals can be mitigated because its dimension is extended to space and frequency domain and the MMSE processing can eliminate interference in both domain.

Therefore, the interference mitigation with CSR has as RF times freedom as that of the legacy bit repetition and needs less numbers of reception antennas to mitigate same number of interference signals than that for the legacy bit repetition. For example, in case of two reception antennas, the interference mitigation with CSR can mitigate two (assuming number of nearest cells in hexagonal cell deployment) dominant interference signals, while those for bit repetition can not.

Figure 4 Spatial interference mitigation with 2 reception antennas.

# Comparative evaluation of legacy bit repetition and proposed method

We compared the block error rate (BLER) performance of the legacy bit repetition without interference mitigation and the interference mitigation with CSR.

(a) Simulation assumptions

A two-cell model with one target cell and one interfering cell is assumed in this simulation. The same information bit rate and the same overall coding rate of 1/4 are set to for both cases. Rest of the simulation assumptions are summarized in Table 1. Figure 5 shows a block diagram of transmitter side. For the bit repetition, part of the output bits of the rate 1/3 turbo encoder are repeated by 2 times in rate matching and resulted in an overall code rate of 1/4. For the symbol repetition, the output bits of the rate 1/3 turbo coder are punctured to rate 1/2 and symbol level repetition in frequency domain is carried out with a repetition factor of two. The mappings of the repeated symbols are coordinated between target and interfering cells.

Table 1 Simulation assumptions [*]

| Cell Layout | one target cell and one interfering cell |
| --- | --- |
| Number of reception antennas at MS [**] | 1 |
| Transmission BW | 10MHz |
| Sub-frame duration | 0.5ms |
| Sub-carrier spacing | 15kHz |
| Sampling frequency | 15.36MHz |
| FFT size | 1024 |
| Number of occupied sub-carriers | 601(DC sub-carrier is null) |
| Decoder algorithm | Max-Log-MAP with 8 iterations |
| Modulation | QPSK |
| Channel environments | Typical Urban 3km/h |
| Channel estimation | Ideal |

[*] Note that although simulation assumption is not reference, characteristic tendency is same.

[**] We assumed one reception antenna since the effect of interference mitigation in both two reception antennas with RF=2 case and one reception antenna with RF=2 case.

Figure 5 Block diagram of transmitters for bit repetition and the CSR

(b) Simulation results

Figure 6 shows the block error rate performance of the bit repetition without interference mitigation and the interference mitigation with CSR. The interference mitigation with CSR mitigates the inter cell interference by means of MMSE reception, while BLER performance of legacy bit repetition without interference mitigation has “floor” saturation.

Figure 6 BLER comparison

# Conclusion

Improving cell edge throughput is a key issue to achieve the system requirement. The interference mitigation with CSR serves better inter cell interference mitigation ability than legacy bit repetition without interference mitigation. We propose to adopt the interference mitigation with CSR as new main functionality on interference mitigation to SDD discussion.

# Text Proposal

Insert the following text into SDD Section 11 – Physical Layer [4]

-------------------------------  Text Start  ---------------------------------------------------

Interference mitigation techniques using conjugate data repetition (CDR)

shall be supported for data and common control channels. CDR includes

simple repetition of modulated data. Use sector specific orthogonal and/or

quasi-orthogonal pilots

-------------------------------  End Start  ---------------------------------------------------

# Reference

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements”

[2] IEEE 802.16m-07/211, “Interference Mitigation with Coordinated Symbol Repetition”

[3] 3GPP R1-050829, TSG-RAN WG1 #42 London, UK, August 29-September 2, 2005

[4] IEEE 802.16m-08/003, “The Draft IEEE 802.16m System Description Document”