| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Analysis of Signaling Requirements for Interference Management |  |
| Date Submitted | 2008-07-08 |  |
| Source(s) | Luciano Sarperi, Mythri Hunukumbure, Chris Williams, Kevin Power, Yanling Lu Fujitsu Laboratories of Europe | E-mail: {Luciano.Sarperi,Mythri.Hunukumbure, Chris.Williams,Kevin.Power,Yanling.Lu}@uk. fujitsu.com |
| Re: | IEEE 802.16m-08/024 Call for Contributions on Project 802.16m SDD Topic: Interference Mitigation |  |
| Abstract | This contribution provides a summary and analysis of the signaling requirements of interference management contributions, with a focus on UL signaling. Text for the SDD is proposed. |  |
| Purpose | Proposal to consider inclusion of suggested text in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Analysis of Signaling Requirements for Interference Management

Luciano Sarperi, Mythri Hunukumbure, Chris Williams, Kevin Power and Yanling Lu

Fujitsu

# Introduction

The System Requirements Document (IEEE 802.16m-07/002r4) states:

- IEEE 802.16m shall support interference mitigation schemes.

- IEEE 802.16m shall support flexible frequency re-use schemes.

This contribution provides a summary and analysis of the signaling requirements by the interference management contributions in TGm until session #55. The reviewed contributions have been compiled from the list of interference management contributions summarized in IEEE C802.16m-08/142 and additionally from contributions submitted thereafter. In particular, the interference management related contributions submitted for session #55 in the context of DL MIMO have been included. Since not all contributions have included simulation results it is difficult to evaluate the proposed interference mitigation schemes in terms of performance at this stage. However, the interference mitigation specific UL signaling mechanisms which are otherwise likely not to be covered by the fast feedback channel or MAC channel measurement reports, should be included in the SDD to ensure that at this stage the SDD supports the signaling required by the majority of interference management proposals. Although backhaul signaling is part of the SDD (IEEE 802.16m-08/003r3), it will not be covered by the 802.16m standard in detail and can therefore be treated with lower priority.

Draft text to be included in the 802.16m System Definition Document (SDD) is also proposed.

# Summary of Signaling Requirements

Table 1 summarises the signaling requirements of the analysed interference management contributions, split into UL signaling, DL signaling and backhaul signaling. Contributions which do not define the signaling requirements have still been included in order to provide a complete overview of interference management related contributions.

Table 1 Summary of Signaling Requirements

| Reference # | Title | UL Signaling | DL Signaling | Backhaul Signaling |
| --- | --- | --- | --- | --- |
| C802.16m-07/115r1 | Inter-cell Interference Mitigation based on IDMA | None | None | None |
| C802.16m-07/165 | Dynamic Interference Coordination | None | None | Frequency patterns and boost/attenuation value describing restrictions |
| C802.16m-07/211 | Interference Mitigation with Coordinated Symbol Repetition | None | None | None |
| C802.16m-07/218r5 | A New Approach for Inter-cell Interference Randomization based on Cell/Sector-specific Interleaver Pattern | None | None | None |
| C802.16m-07/237 | Uplink Interference Cancellation of Layered Superposed OFDMA (LS-OFDMA) Transmissions | None | None | None |
| C802.16m-07/244r1 | Collaborative MIMO | CQI | None | BS to central scheduler: CQI. Central scheduler to BS: scheduling decision |
| C802.16m-07/278r1 | Uplink Interference Coordination/Control for 802.16m | N/A | Fractional power control information | UL signal quality (e.g. IoT) |
| S802.16m-08/005 | Interference Mitigation Performance with 4 antenna BS | None | None | None |
| C802.16m-08/017 | Frame Structure to Support Inter-cell Interference Mitigation for Downlink Traffic Channel using Co-MIMO and FFR | Preamble CINR, interferer's cell ID, preferred beam index | None | Not defined |
| C802.16m-08/034r1 | Adaptive Frequency Reuse for Interference Management in IEEE 802.16m System | Interference level | Not defined | BS to radio resource control element: Interference level. Radio resource control element to BS: Interference mitigation parameters |
| C802.16m-08/039 | 802.16m basic frame structure for improved intra-system coexistence | None | None | None |
| C802.16m-08/075 | Proposal for the Self-organization & Self-optimization in IEEE 802.16m System | SINR, INR, intererer's cell ID | None | Not defined |
| C802.16m-08/183 | Base Station Cooperation and Channel Sounding | CSI | None | CSI |
| C802.16m-08/238 | Frame structure for intra-system interference management | None | None | None |
| C802.16m-08/275 | EEE 802.16m UL Common Feedback | NACK signal when about to be in outage, sent by SS to interfering BS directly | None | None |
| C802.16m-08/317r4 | Inter-BS Coordination for Interference Management in IEEE 802.16m | Not defined | Not defined | E.g. Interference characteristics, interference measurements |
| C802.16m-08/393 | Inter-Base Station Coordination for Interference Management | Long term channel measurements, UL pilots/sounding | Power adaptation parameters | Interference over thermal noise (IoT), resources/transmission formats allocated, power allocation, CSI, beamforming weights, interference information, handover management, timing synchronisation, HARQ management |
| C802.16m-08/403r3 | Downlink Symbol Structure Requirements for Supporting Adaptive Frequency Reuse in IEEE 802.16m | Geometry and instantaneous SINR | System configuration (bandwidth, power levels, system costs of reuse partitions) | Not defined |
| IEEE C802.16m-08/430 | PMI Restriction for the downlink Closed-loop MIMO | Precoding matrix index with respect to interfering BS | None | Precoding matrix index with respect to interfering BS and related information |
| C802.16m-08/443r2 | Inter-Cell Interference Management in DL/UL Control | Interference reduction request | None | None |
| C802.16m-08/488r1 | Classification of DL MIMO Modes | None | None | None |
| C802.16m-08/449 | Efficient MIMO downlink intercell interference cancellation using pre-coding | Precoding vectors with respect to BS in preferred list (interfering BS) | None | Precoding vectors with respect to BS in preferred list and current resource allocation |
| C802.16m-07/203r1 | Adaptive Frequency Reuse in IEEE 802.16m System | Not defined | Not defined | Not defined |
| C802.16m-07/243 | Closed-loop MIMO Macro Diversity | Not defined | Not defined | Not defined |
| C802.16m-07/282r3 | Updated Proposal for IEEE 802.16m System Architecture and Protocol Structure | Not defined | Not defined | Not defined |
| C802.16m-07/313r2 | ToC proposal for interference management of IEEE 802.16m SDD | Not defined | Not defined | Not defined |
| C802.16m-08/044r1 | Network MIMO for Inter-cell Interference Mitigation | Not defined | Not defined | Not defined |

# Common UL Signaling

The UL feedback required by the majority of the proposed interference mitigation schemes listed in Table 1 is: Interfering Cell ID, Interference reduction request, Precoding vector (or precoding vector index with codebook based approach) with respect to interfering BS, Preamble SINR, Data region SINR and Long term SINR.

Out of these, the following signaling requirements are likely not to be covered by the standard fast feedback channel or MAC channel measurement reports and should therefore be included in the draft SDD:

- Interfering Cell ID

- Interference reduction request

- Precoding vector / precoding vector index with respect to interfering BS

In particular, feedback of the Precoding vector / precoding vector index with respect to the interfering BS is crucial for interference mitigation of closed-loop MIMO transmissions, proposed in C802.16m-08/449 and IEEE C802.16m-08/430. Note that the long term SINR could be obtained at the BS by averaging the data or preamble SINR reports from the MS.

# Proposed Text

Insert the following text into SDD sections indicated below

---------------------------text start-----------------------------

11.x Interference Mitigation

11.x.1 Measurement and Reporting

A mobile station may feedback the following interference management related information:

- Interfering Cell ID

- Interference reduction request

- Precoding vector (or precoding vector index with codebook based approach) with respect to interfering BS

---------------------------------text end---------------------------