| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fractional Frequency Reuse in Uplink |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | JongYoung Han, HanGyu Cho LG Electronics | Voice : +82-31-450-1852 E-mail: {fanaticey, hgcho}@lge.com |
| Re: | IEEE 802.16m-08/024 “Call for Comments and Contributions on Project 802.16m System Description Document (SDD)” in response to the following topic – Interference Mitigation |  |
| Abstract | Propose the FFR in uplink and show the preferred FFR configuration for interference mitigation |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fractional Frequency Reuse in Uplink

JongYoung Han, HanGyu Cho

LG Electronics

1. Introduction

According to 802.16m system requirements [1], performance of cell edge user is required to be twice improved relatively over the Wireless MAN-OFDMA system. In uplink, since the variance of interference is very large, the target performance requirement is more challenging.

Generally, fractional frequency reuse (FFR) scheme is known to be useful for the enhancement of the cell edge user performance with a reasonable tradeoff with average user throughput.

In this contribution, we show the system-level performance gain from the FFR in uplink and propose the FFR to be adopted in the 802.16m uplink design. Furthermore, we show the preferred FFR configuration and operation, which is practical in terms of signaling overhead and operational feasibility. Some power control issue in FFR is also addressed in a high-level.

2. System-level performance gain from FFR

2.1 Fractional frequency reuse (FFR)

Fractional frequency reuse partitions the frequency resources for use of different reuse factors. Figure 1 shows the concept of hard FFR and soft FFR assuming that reuse 1 and reuse 1/3 are used, respectively. In hard FFR, some resources are dedicated to edge users in one cell and are not allowed to be used by any users in other cells, which results in loss of spectral efficiency. In soft FFR, (inner) users in other cells are allowed to use the resources assigned to the edge users in other cell with some power restrictions. Since we can restrict the power to zero, hard FFR is a special case of soft FFR. In this contribution, we only consider soft FFR which is more promising in terms of spectral efficiency.

Figure 2 shows the cell configurations for hard FFR and soft FFR, respectively.

Figure 1. FFR partition and PSD

Figure 2. Cell configuration with FFR

2.2 System-level performance comparison

In this section, we compares the non-FFR case and FFR case based on system-level simulation of which assumptions and parameters are shown in Appendix.

Figure 3 compares the CDF of user throughput and the detailed results are summarized in Table 1 and 2 for ISD (inter sector distance) = 500m and ISD = 1500m, respectively. As shown in the Figure and Tables, FFR brings cell edge user throughput gain 42% and 25% at the cost of small average user throughput loss 1% and 1.4% for ISD = 500m and ISD = 1500m, respectively.

(a) ISD = 500m                                   (b) ISD = 1500m

Figure 3. CDF of user throughput

Table 1. Throughput/spectral efficiency comparison (ISD = 500m)

|  | Without FFR | With FFR |
| --- | --- | --- |
| Average user throughput | 293 kbps (0.781bps/Hz) | 290 kbps (-1.0%) (0.773ps/Hz) |
| 5% tile user throughput | 62.48 kbps (0.0166bps/Hz) | 88.74 kbps (+42.0%) (0.0237/Hz) |

Table 2. Throughput/spectral efficiency comparison (ISD = 1500m)

|  | Without FFR | With FFR |
| --- | --- | --- |
| Average user throughput | 290 kbps (0.773bps/Hz) | 286 kbps (-1.4%) (0.763 bps/Hz) |
| 5% tile user throughput | 46.2 kbps (0.0123bps/Hz) | 57.9 kbps (+25.3%) (0.0154bps/Hz) |

3. FFR configuration in UL

In this section, we show our preference on FFR configuration in UL. Our main design criterion on FFR is to confirm the feasibility and real performance gain.

Table 3 shows the FFR configuration. As shown in the table we categorize into only two-modes: FFR-off and FFR-on which is reasonable and realistic in the aspect of signaling overhead, operational complexity, and cell configuration. For flexibility, we support two reuse factors 1/3 and 2/3 and also support flexible partitioning between reuse 1 region and reuse 1/3 (or 2/3) region.

Table 3. FFR configuration

|  | Reuse 1 case (no FFR) | FFR case | Signaling |
| --- | --- | --- | --- |
| # of partitioning | 1 | 4 | 2bit signaling |
| Reuse factor | 1 | 1/3, 2/3 |  |
| BW partition | BWT | BWT/(3+x)*[1,1,1,x] | 2bit signaling |

Summary is as follows:

- Only two modes: FFR-off and FFR-on (4 partitions)

- Adaptive FFR

  - Adaptive to traffic load under BS coordination

  - Adaptive reuse factor

  - Adaptive BW size

- Semi-static FFR

  - Dynamic (period is less than 20ms) FFR is impractical

  - BS to BS sharing of FFR configuration by backbone

  - BS to MS signaling of FFR configuration using broadcast channel per superframe

- Soft FFR

  - Hard FFR is a special case

4. Consideration on power control in FFR

In uplink, mean and variance of interference level (or IoT) is generally large. FFR can help to reduce mean and variance of interference level which is beneficial in the aspect of link adaptation and power control.

Power control in uplink can be considered along with FFR to be fit to each feature of FFR group. Some examples are as follows:

- Tx. power restriction or MCS level restriction for inner bands in reuse 1/3 (2/3) region

  - Different restriction can be applied according to geometry

- Different power control scheme can be applied to users in the edge band

- Inter-cell power control considering FFR groups

  - E.g. explicit (or signaling-based) inter-cell power control for the edge band (or edge users) and implicit inter-cell power control for the inner band

Detailed power control scheme is likely to be more implementation-oriented, but, it seems necessary to consider FFR when designing power control scheme, especially for inter-cell power control.

5. Conclusion

We would like to replace conclusion with the following proposed text for SDD.

================================= Start of Proposed Text ====================================

11.x.x. Interference Management

11.x.x.x Uplink

11.x.x.x.x Fractional Frequency Reuse (FFR)

Fractional frequency reuse (FFR) is considered in uplink. The information for FFR configuration such as the number of FFR groups, bandwidth and power level for each group, and etc., is semi-statically shared among BSs and properly signaled to MSs.

=================================== End of Text Proposal ===================================

# References

[1] IEEE 80216m-07_002r4.pdf “IEEE 802.16m System Requirements Document (SRD)”

Appendix

<Simulation Assumptions>

| Scenario /Parameters | Baseline Configuration |
| --- | --- |
| Site-to-Site Distance | 1.5km / 0.5km |
| Carrier Frequency | 2.5GHz |
| Operating Bandwidth | 10MHz for TDD |
| BS Height | 32m |
| MS Tx Power | 23dBm / 17dBm |
| MS Height | 1.5m |
| Penetration Loss | 10dB |
| Pathloss model | Loss(dB)=130.62+37.6log10(R)+cable lossR=distance in km, cableloss 2dB |
| Lognormal Shadowing Standard Deviation | 8dB |
| Correlation Distance for Shadowing | 50m |
| Channel Type | ITU Ped B |
| Mobility | 3km/hr |
| Spatial Channel Model | ITU with spatial correlation |
| Basic Modulation | QPSK, 16QAM |
| Duplexing Scheme | TDD |
| Subchannelization | Band AMC |
| Resource Allocation Granularity | Band AMC Type: 2x3 |
| Multi-antenna Transmission Format | 1x2 SIMO |
| Receiver Structure | MMSE |
| Data Channel Coding | Convolutional Turbo Coding (CTC) |
| Scheduling | Proportional fairness for full buffer data only 10 active users / sector Fixed control overhead of 3 symbols, 15 symbols for data, 5 partitions of 48 sub-channels |
| Link Adaptation | 7 levels, QPSK(1/2) with repetition 1/2/3/4, QPSK(3/4), 16QAM(1/2), 16QAM(3/4) |
| Link to System Mapping | MMIB |
| HARQ | Adaptive HARQ, Maximum 4 retransmissions, retransmission duration 3 frames |
| Interference Model | Average interference on used band |
| Frequency Reuse Factor(FRF) | 3 sectors with FRF=1, FRF 1/3 for FFR operation |
| Users dropped | Uniformly in entire cell |