| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m ID Management for Location Privacy |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | GeneBeck Hahn, KiSeon Ryu and Ronny Yongho Kim LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-7188 E-mail: gbhan@lge.com, ksryu@lge.com and ronnykim@lge.com |
| Re: | IEEE 802.16m-08/024: Call for Contributions on Project 802.16m System Description Document (SDD) Specific Topic: “Upper MAC Addressing” |  |
| Abstract | This contribution describes the IEEE 802.16m ID management to ensure location privacy. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m ID Management for Location Privacy

Gene Beck Hahn, Ki Seon Ryu and Ronny Yong Ho Kim

LG Electronics

# Problem Definition

The threat to location privacy is made from active or passive attacks to compromise the MS (Mobile Station) MAC address. There are few ongoing works to offer strong level of user identity and location confidentiality. Ideally, the user identity and location information shall be protected against various types of attacks. There have been a number of researches illustrating the necessity of location privacy support in wireless networks [3][4][5][6]. When location privacy is not supported, attackers can find where a user is, and can further use this information to track the mobile user. This is because a user’s communications can be easily correlated in case all communications come from the same address and the address can be used to trace the user’s movements. As we can see, there are few safeguards on the location privacy although location-based services are emerging as the future killer applications in the personal wireless devices. The wireless privacy protection act of 2003, currently under consideration by the US congress, proposes to amend the communications act of 1934 “To require customer consent to the provision of wireless call location information.”

In legacy IEEE 802.16 systems, MAC address is a globally unique value for each IEEE 802.16-based device. In many situations, MAC address is treated as a means for authentication or as an identifier to grant a varying level of network privilege to a user. That is, MAC address of a user is matched to the authentication credentials and network permits the communication depending on a list of authorized MAC addresses [1]. Hence, the location of a user can be easily inferred from fixed MAC address.

According to IEEE 802.16m SRD, IEEE 802.16m shall include a security function which provides the necessary means to achieve the protection and confidentiality of user-generated and user-related data (e.g., location privacy, user identity). As depicted before, MS MAC address (i.e., user identity) is sent from MS to BS in an unprotected way (during initial ranging, location update and re-entry), result of which violates the security aspects of IEEE 802.16m SRD. Hence, low cost solution to protect the user and identity location in legacy IEEE 802.116 system shall be proposed [2].

# IEEE 802.16m ID Management using Temporary Identifier

During initial ranging, BS allocates a new temporary identifier that can be used instead of MS MAC address. This can avoid the compromise of MS MAC address sent via air interface while minimizing the network overhead incurred from the use of a temporary identifier. For doing this, network shall store the mapping of temporary identifier, MS MAC address and CID (Connection Identifier) prior to the initiation of authorization phase.

Figure 1 describes the procedure of allocating temporary identifier to MS during initial ranging. As we can see in Figure 1, temporary identifier is sent from MS to BS instead of MS MAC address. That is, a lightweight procedure of temporary identity allocation meets the requirements of IEEE 802.16m SRD with regard to location privacy.

Figure 1: Initial Ranging Procedure via Allocation of Temporary Identifier for IEEE 802.16m MS

For the case of location update or network re-entry, paging identifier (Paging ID) is allocated from paging controller while MS enters into idle mode. The paging controller keeps the mapping of paging identifier and MS MAC address. The subsequent paging messages can be received via this Paging ID. Figure 2 shows the allocation of Paging ID during idle mode initiation.

Figure 2: Allocation of Paging Identifier from Paging Controller for IEEE 802.16m MS Allocation

If MS awakes from idle mode and performs location update, Paging ID can then be sent from MS to paging controller via target BS. That is, the paging controller is notified of the new location of MS. In case the paging controller is changed, new paging controller tries to retrieve the location information for corresponding MS from old paging controller. Besides, new Paging ID is allocated to the MS via target BS. In case the paging group is changed, new paging group ID is sent to MS via target BS. Again, the MS MAC address is not used and this also meets the requirements of IEEE 802.16m SRD with regard to location privacy. Figure 3 shows the ranging procedure during location update via paging ID allocation.

Figure 3: Ranging Procedure during Location Update via Allocation of Paging Identifier

Figure 4: Ranging Procedure during Network Reentry via Allocation of Paging Identifier

If MS tries to perform network reentry, it also uses a paging ID as in the case of ranging procedure during location update. Figure 4 shows the ranging procedure during network reentry via paging ID allocation. Since target BS does not have any information about MS, it shall retrieve corresponding MS information from paging controller when it receives a RNG-REQ containing the paging ID from MS. This can be done via the exchange of MS Info Request/Response between target BS and paging controller. The BS will then know of the MS MAC address corresponding to the received paging ID. Hence, the MS MAC address is not revealed and this meets the requirements of IEEE 802.16m SRD with regard to location privacy.

# References

[1] IEEE 802.16e Rev2/D2, Part 16: Air interface for Broadband Wireless Access Systems, December 2007

[2] IEEE 802.16m-07/002r2150, Draft IEEE 802.16m Requirements, 2007-06-08

[3] “A Framework for Location Privacy in Wireless Networks”, Yin-Chun Hu, Helen J. Wang, ACM SIGCOMM Asia Workshop 2005, April 12-14, 2005

[4] “A Study on the value of Location Privacy”, Dan Cvrcek, Marek Kumpost, WPES’06, October 30, 2006

[5] “How Much is Location Privacy Worth?”, George Danezis, Stephen Lewis and Ross Anderson, Workshop on the Economics of Information Security, June 1-3, 2005

[6] “Towards Mobile Internet : Location Privacy Threats and Granular Computation Challenges”, Ling-Lu, Granular Computing 2007, IEEE International Conference on, Nov 2-4, 2007

Text Proposal for IEEE 802.16m SDD

============================= Start of Proposed Text =============================

# 10.x.y ID Management

In order to achieve the protection and confidentiality of user-generated and user-related data (e.g., location privacy and user identity), the use of temporary or paging identifiers shall be considered in IEEE 802.16m specification. That is, a temporary identifier shall be used instead of MS MAC address for initial network entry. Besides, a Paging ID shall be used instead of MS MAC address for location update and network reentry from idle mode. In the legacy IEEE 802.16 system, location privacy is not supported and thus mobile users can be tracked by malicious attackers. This indicates that MS MAC address is clearly exchanged between MS and BS during the procedure of initial ranging, location update and network reentry. Thus, the use of temporary and paging identifiers shall be incorporated in IEEE 802.16m specification to support location privacy.

============================= End of Proposed Text =============================