| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation Using Precoded Multi-Cell MIMO in the Downlink |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Jaewan Kim Sunam Kim Dongguk Lim Bin-Chul Ihm HanGyu Cho LG Electronics LG R&D Complex, 533 Hogye-1 dong Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-1883 E-mail: lgjohnkim@lge.com snkim816@lge.com dongguk@lge.com bcihm@lge.com hgcho@lge.com |
| Re: | IEEE 802.16m-08/024 “Call for Comments and Contributions on Project 802.16m System Description Document (SDD)” in response to the following topic – Interference Mitigation |  |
| Abstract | Propose high level concept on IEEE 802.16m interference mitigation section |  |
| Purpose | For IEEE 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Mitigation Using Precoded Multi-Cell MIMO in the Downlink

Jaewan Kim, Sunam Kim, Dongguk Lim, Bin-Chul Ihm, and HanGyu Cho

LG Electronics

# Introduction

System requirement- for the cell-edge user performance is defined in the document as a baseline. In order to improve the cell-edge user throughput, we can consider several interference mitigation techniques. Among them, multi-cell MIMO can be one of the prominent techniques providing good cell-edge performance[1]. Here, we propose interference mitigation using the codebook-based precoding multi-cell MIMO in the downlink.

# Closed-loop multi-cell MIMO

Closed-loop multi-cell MIMO can be a good solution for cell-edge users whose moving velocity is low or medium since multi-cell BSs can exploit the downlink channel information through the uplink feedback signal in order to improve the performance. Closed-loop multi-cell MIMO is suitable for getting the beamforming gain as well as AMC gain. Particularly, precoding MIMO scheme can be applied for multi-cell MIMO. But, there is possibility that PMI feedback information for target multi-cell BS is not useful for multi-cell MIMO if backbone network latency is longer than the coherence time of a transmitted signal. So, closed-loop multi-cell MIMO scheme is necessary not to make performance degradation due to the backbone network latency.

# Proposed schemes

We propose that codebook shall be used for small and large antenna spacing multi-cell MIMO  follows.

1) Small antenna spacing : Codebook based BF

2) Large antenna spacing : Codebook based precoding MIMO

Basically, CL multi-cell MIMO scheme which exploits long term CSI(Channel Status Information) like AoD (Angle of Departure) information is desirable as the candidate for CL multi-cell MIMO if backbone network is used for transferring CSI to target BS. We have shown that DFT based codebook shows the robust performance in the spatially correlated channel [2]. Such a DFT based codebook can be also used for the multi-cell precoding MIMO or beamforming.

Equation (1) is the generalized form of the basic DFT matrix

(1)

where, Nt is the number of transmit antenna.

And, phase shift matrix including AoD information is represented by using equation (2).

(2)

where, for the case of m extension bits,                              .

The final codebook element is              where C is created by the combination of column vector elements in    .

# Coexistence with other interference mitigation schemes

PMI restriction (or beam avoidance) or FFR can be applied with CL multi-cell MIMO in order to enhance cell-edge user throughput. CL multi-cell MIMO request may be rejected if the requiring PMI is the same as that for PMI restriction of interfering BS because the PMI is requested by many cell-edge users for mitigating inter-cell interference[3]. Its impact is FFS.

# Message type and information for CL multi-cell MIMO

We can categorize the messages into three types, largely.

- Service control message : This message is related to establish the signaling between MS, serving BS and target BS.

- Scheduling information : BS ID, band, CQI, etc

- MIMO related information : CSI, MIMO type and antenna configuration, etc.

# Method for transferring the channel status information

There are three feedback methods for transferring the channel status information to be considered:

- BS to BS backbone network communication between the serving BS and the collaborative BSs.

In this case, MIMO schemes robust to backbone network latency is desirable for multi-cell MIMO in order to overcome the issue.

- Direct feedback to the collaborative BSs, that is, air to air from the MS side.

In this case, MIMO schemes applied to single cell MIMO is reusable but its impact on the conventional uplink signaling structure should be considered.

3)  Feedback via relay station located at the cell-edge area of the collaborative BSs. Relay station is considered as a good solution to enhance the cell-edge user throughput.

-

# Reference

[1] C80216m-08_783, ”Mitigation using FFR and Multi-cell MIMO in Downlink”

[2] C80216m-08_431, “Codebook based pre-coding MIMO”

[3] C80216m-08_784, “Interference Mitigation Using PMI Restriction in the multi-cell environments”

# Proposed Texts

---------------------------------------------------  Text Start  ---------------------------------------------------

11.x.x. Interference Management

11.x.x.x. Downlink

11.x.x.x.x. Multi-cell MIMO

Based on the codebook, the closed-loop multi-cell MIMO shall be supported in downlink.

---------------------------------------------------  Text End  ---------------------------------------------------