| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MCS for IEEE 802.16m CTC |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Woosuk Kwon, Seunghyun Kang, Sukwoo Lee LG Electronics, Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-1869 E-mail: kytloze@lge.com, sh_kang@lge.com, sugoo@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/024 - Call for Contributions on Link Adaptation Schemes |  |
| Abstract | This contribution describes the considerations on MCS table in IEEE 802.16e reference system and provides our view of new MCS design for better link adaptation. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MCS Table Generation for IEEE 802.16m CTC

Woosuk Kwon, Seunghyun Kang, Sukwoo Lee

LG Electronics

- Introduction

In this contribution, we discuss the MCS table in the legacy system, and propose the necessity of new MCS with several criteria. A MCS table is designed from these criteria and show the system level simulation result to compare legacy MCS and new-designed MCS.

- MCS table in IEEE 802.16e

In the legacy system, there are 8 MCS indexes and they are shown in Table 1. Colored cells indicate usable NEP corresponding to MCS index and NSCH (RU). Since CTC interleaver sizes are limited to below 480 bits, all available MCS indexes are not used for most of NSCH (RU) and it is not sufficient to reflect various channel conditions using those limited MCS.

Figure 1 shows the required SNR at 10% BLER in terms of MCS index and spectral efficiency including repetition. In this figure, there are irregular spaces of required SNR between each MCS indexes, which is 4dB in the worst case. With the large gap of required SNR between each MCS indexes, it is difficult to reflect the channel condition exactly, and it causes a poor AMC gain.

Table 1 – Legacy MCS Table and NEP set

| Mod | Code Rate | Repetition | Spectral Efficiency | NSCH (RU) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 9 | 10 |
| 2 | 1/2 | 1 | 1.0 | 48 | 96 | 144 | 192 | 240 | 288 | 384 | 432 | 480 |
| 2 | 3/4 | 1 | 1.5 | 72 | 144 | 216 | 288 | 360 | 432 | 576 | 648 | 720 |
| 4 | 1/2 | 1 | 2.0 | 96 | 192 | 288 | 384 | 480 | 576 | 768 | 864 | 960 |
| 4 | 3/4 | 1 | 3.0 | 144 | 288 | 432 | 576 | 720 | 864 | 1152 | 1296 | 1440 |
| 6 | 1/2 | 1 | 3.0 | 144 | 288 | 432 | 576 | 720 | 864 | 1152 | 1296 | 1440 |
| 6 | 2/3 | 1 | 4.0 | 192 | 384 | 576 | 768 | 960 | 1152 | 1536 | 1728 | 1920 |
| 6 | 3/4 | 1 | 4.5 | 216 | 432 | 648 | 864 | 1080 | 1296 | 1728 | 1944 | 2160 |
| 6 | 5/6 | 1 | 5.0 | 240 | 480 | 720 | 960 | 1200 | 1440 | 1920 | 2160 | 2400 |

Figure 1 - Required SNR of each MCS indexes in IEEE 802.16e

In IEEE 802.16m system, it is need to have an equal spacing of required SNR between each MCS indexes and a denser MCS to reflect more exact channel condition. It is possible to set equal required SNR spacing by changing of code rate in the MCS, and generate denser MCS using all these information to indicate MCS (4 bits).

- MCS Table Generation Procedure

  - Assumptions

In IEEE 802.16m system, various frame configurations are used and each configuration has a different number of data sub-carriers. In order to design a new MCS table, it is needed to select basic frame configuration. Table 2 shows the number of data sub-carriers for each frame structures in IEEE 802.16m system, and 85 data sub-carriers per one sub-frame are selected for basic frame configuration.

Table 2 – Number of data sub-carriers for each frame structure in IEEE 802.16m

|  | 18 × 5 |  |  |  | 18 × 6 |  |  |  | 18 × 7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 OFDM Control |  | 0 OFDM Control |  | 1 OFDM Control |  | 0 OFDM Control |  | 1 OFDM Control |  | 0 OFDM Control |  |
| Tx Antenna | Pilot | Data sub-carrier | Pilot | Data sub-carrier | Pilot | Data sub-carrier | Pilot | Data sub-carrier | Pilot | Data sub-carrier | Pilot | Data sub-carrier |
| 1 Tx. | 4 | 68 | 5 | 85 | 5 | 85 | 6 | 102 | 6 | 102 | 7 | 119 |
| 2 Tx. | 8 | 64 | 10 | 80 | 10 | 80 | 12 | 96 | 12 | 96 | 14 | 112 |
| 4 Tx. | 16 | 56 | 20 | 70 | 20 | 70 | 24 | 84 | 24 | 84 | 28 | 98 |

Based on this frame structure, additional assumptions are expressed in table 3.

Table 3 – Additional assumptions for IEEE 802.16m MCS design

| Parameter | Assumption |
| --- | --- |
| Frame structure | 18 sub-carrier × 6 OFDM symbols |
| Control | 1 OFDM symbol |
| Tx antenna | 1 Tx |
| Cyclic prefix | 1/8 |
| NEP Range | NEP_min = 40 bits, NEP_max = 4800 bits |
| Modulation | 2(QPSK), 4(16-QAM), 6(64-QAM) |
| MCS level | 16 |
| Target Code Rate | 0.1 ~ 0.9 |
| Required SNR | -5 ~ 20 dB |
| Supporting Spectral Efficiency | ~ 5.5 |

  - Design Procedure for New MCS

The MCS table is designed with the following procedure:

- Select target code rate range about 0.1~0.9 and required SNR about -5~20 dB

- Mapping modulation order to target code rate range.

- Select NEP to support the target code rate and modulation order.

- BLER simulation for target code rate, modulation, and selected NEP. Check Required SNR in terms of spectral efficiency.

- Select 16 MCS indexes from the candidate set, to have uniform required SNR space.

16 MCS indexes are selected as shown in table 4, and the resulting required SNR at 10% BLER is shown in figure 2. Figure 2 shows that the required SNR space between each MCS indexes are equal and it is about 1.4dB even with a different number of RU sizes. It is further clearly compared in figure 3 that new-designed MCS has dense and linear aspects of required SNR compared to legacy MCS.

Table 4 – Modulation and Coding Set Table for CTC

| MCS Index | Target Code Rate | Modulation | Spectral Efficiency |
| --- | --- | --- | --- |
| 0 | 0.1504 | 2 | 0.3008 |
| 1 | 0.2168 | 2 | 0.4336 |
| 2 | 0.3203 | 2 | 0.6406 |
| 3 | 0.4326 | 2 | 0.8652 |
| 4 | 0.5645 | 2 | 1.1289 |
| 5 | 0.3105 | 4 | 1.2422 |
| 6 | 0.4141 | 4 | 1.6563 |
| 7 | 0.5176 | 4 | 2.0703 |
| 8 | 0.6025 | 4 | 2.4102 |
| 9 | 0.7148 | 4 | 2.8594 |
| 10 | 0.5146 | 6 | 3.0879 |
| 11 | 0.5898 | 6 | 3.5391 |
| 12 | 0.6904 | 6 | 4.1426 |
| 13 | 0.7656 | 6 | 4.5938 |
| 14 | 0.8281 | 6 | 4.9688 |
| 15 | 0.9033 | 6 | 5.4199 |

Figure 2 –Required SNR vs. MCS index with a new-designed MCS

Figure 3 – Required SNR comparison with legacy and new-designed MCS Index

  - SLS Performance Evaluation

In order to show the performance gain of the new-designed MCS compared to legacy MCS, system level simulation is performed, and its assumptions and scenario are added in appendix A.

Figure 4 – Comparison of Gap between Selected SIR and Received SIR

When MCS is selected, normally the biggest MCS lower than the received SIR is selected, and there will be a gap between the received SIR and the selected SIR corresponding to a selected MCS. Figure 4 shows the comparison result of the gap between selected SIR and received SIR. In the legacy system, because of large required SNR spacing between MCS indexes in most regions, the gap between selected SIR and received SIR is large. This gap affects the throughput performance, and it is shown in figure 5. When the new-designed MCS is used, the performance gain is about 5%, and 6% compared to the legacy MCS in the aspect of average sector throughput, and cell edge throughput, respectively without further increased complexity.

Figure 5 – Throughput Comparison

Table 5 –Throughput result from system level simulation

| Metric | Legacy MCS | New-Designed MCS | Gain |
| --- | --- | --- | --- |
| Average Sector Throughput | 1.864 Mbps | 1.960 Mbps | 5.15% |
| Cell Edge Throughput | 730 kbps | 776 kbps | 6.30% |

- Conclusions

In the legacy system, there are several restrictions on MCS, and there is a room for performance improvement to make up for these restrictions. Therefore, we recommend a new MCS should be designed.

Text Proposal for the 802.16m SDD

============================== Start of Proposed Text ================================

# 11.x Channel Coding

# 11.x.1 Channel Coding for data channel

# 11.x.1.x Convolutional Turbo Codes

MCS should be selected with equal space of required SNR.

MCS should be dense enough to facilitate more efficient link adaptation.

Table 11.x.x.x gives the code rates, modulation, and spectral efficiency.

Table 11.x.x.x – Modulation and Coding Set Table for CTC

| MCS Index | Target Code Rate | Modulation | Spectral Efficiency |
| --- | --- | --- | --- |
| 0 | 0.1504 | 2 | 0.3008 |
| 1 | 0.2168 | 2 | 0.4336 |
| 2 | 0.3203 | 2 | 0.6406 |
| 3 | 0.4326 | 2 | 0.8652 |
| 4 | 0.5645 | 2 | 1.1289 |
| 5 | 0.3105 | 4 | 1.2422 |
| 6 | 0.4141 | 4 | 1.6563 |
| 7 | 0.5176 | 4 | 2.0703 |
| 8 | 0.6025 | 4 | 2.4102 |
| 9 | 0.7148 | 4 | 2.8594 |
| 10 | 0.5146 | 6 | 3.0879 |
| 11 | 0.5898 | 6 | 3.5391 |
| 12 | 0.6904 | 6 | 4.1426 |
| 13 | 0.7656 | 6 | 4.5938 |
| 14 | 0.8281 | 6 | 4.9688 |
| 15 | 0.9033 | 6 | 5.4199 |

=============================== End of Text Proposal ===============================

Appendix A. System Level Simulation Assumption

Table A.1 – Simulation Assumptions

| Topic | Description | Baseline Simulation Assumptions | Proposal Specific Assumptions |
| --- | --- | --- | --- |
| Basic modulation | Modulation schemes for data and control | QPSK, 16QAM, 64QAM | QPSK, 16QAM, 64QAM |
| Duplexing scheme | TDD, HD-FDD or FD-FDD | TDD | FDD |
| Subchannelization | Subcarrier permutation | PUSC | Band-AMC |
| Resource Allocation Granularity | Smallest unit of resource allocation | PUSC: Non-: 1 slot, : 2 slots (1 slot = 1 subchannel x 2 OFDMA symbols) | Band-AMC (18 subcarriers x 6 OFDM symbols) |
| Downlink Pilot Structure | Pilot structure, density etc. | Specific to PUSC subchannelization scheme | Band-AMC |
| Multi-antenna Transmission Format | Multi-antenna configuration and transmission scheme | MIMO 2x2 (Adaptive MIMO Switching Matrix A & Matrix B) Beamforming (2x2) | MIMO 2x2 (Adaptive MIMO Switching Matrix A & Matrix B) Codebook based precoding(16e 3bit codebook) |
| Receiver Structure | MMSE/ML/MRC/ Interference Cancellation | MMSE (Matrix B data zone) MRC (MAP, Matrix A data zone) | MMSE (Rank 2) MRC (Rank1) |
| Data Channel Coding | Channel coding schemes | Convolutional Turbo Coding (CTC) | Convolutional Turbo Coding (CTC) |
| Control Channel Coding | Channel coding schemes and block sizes | Convolutional Turbo Coding, Convolutional Coding (CC) for FCH only | - |
| Scheduling | Demonstrate performance / fairness criteria in accordance to traffic mix | Proportional fairness for full buffer data only *, 10 active users per sector, fixed control overhead of 6 symbols, 22 symbols for data, 5 partitions of 66 slots each, latency timescale 1.5s | Proportional fairness for full buffer data only *, 10 active users per sector, fixed control overhead of 0 symbols, 6 symbols for data, 6 partitions of 16 slots each, latency timescale 1.5s |
| Link Adaptation | Modulation and Coding Schemes (MCS), CQI feedback delay / error | QPSK(1/2) with repetition 1/2/4/6, QPSK(3/4), 16QAM(1/2), 16QAM(3/4), 64QAM(1/2), 64QAM(2/3), 64QAM(3/4) 64QAM(5/6), CQI feedback delay of 3 frames, error free CQI feedback ** | WiMAX MCS & LGE MCS CQI feedback delay of 3 sub-frames, error free CQI feedback ** |
| Link to System Mapping | EESM/MI | MI (RBIR) *** | RBIR |
| HARQ | Chase combining/ incremental redundancy, synchronous/asynchronous, adaptive/non-adaptive ACK/NACK delay, Maximum number of retransmissions, retransmission delay | Chase combining asynchronous, non-adaptive, 1 frame ACK/NACK delay, ACK/NACK error, maximum 4 HARQ retransmissions, minimum retransmission delay 2 frames**** | Chase combining asynchronous, non-adaptive, 3 subframes ACK/NACK delay, ACK/NACK error, maximum 4 HARQ retransmissions, minimum retransmission delay 8 sub-frames |
| Power Control | Subcarrier power allocation | Equal power per subcarrier | Equal power per subcarrier |
| Interference Model | Co-channel interference model, fading model for interferers, number of major interferers, threshold, receiver interference awareness | Average interference on used tones in PHY abstraction (Refer to Section ‎4.4.8) | Average interference on used tones in PHY abstraction (Refer to Section ‎4.4.8) |
| Frequency Reuse | Frequency reuse pattern | 3 Sectors with frequency reuse of 1 ***** | - |

Table A.2 – Test Scenarios

| Scenario/ Parameters | Baseline Configuration (Calibration & SRD) TDD and FDD | Specific Assumption |
| --- | --- | --- |
| Requirement | Mandatory |  |
| Site-to-Site Distance | 1.5 km | 1.5 km |
| Carrier Frequency | 2.5 GHz | 2.5 GHz |
| Operating Bandwidth | 10 MHz for TDD / 10 MHz per UL and DL for FDD | 10 MHz per UL and DL for FDD |
| BS Height | 32 m | 32 m |
| BS Tx Power per sector | 46 dBm | 46 dBm |
| MS Tx Power | 23 dBm | 23 dBm |
| MS Height | 1.5 m | 1.5 m |
| Penetration Loss | 10 dB | 10 dB |
| Path Loss Model | Loss (dB) = 130.19+37.6log10(R) (R in km) ** | Loss (dB) = 130.19+37.6log10(R) (R in km) |
| Lognormal Shadowing Standard Deviation | 8 dB | 8 dB |
| Correlation Distance for Shadowing | 50m | 50m |
| Mobility | 0-120 km/hr | 3 km/hr |
| Channel Mix | ITU Ped B 3 km/hr – 60% ITU Veh A 30 km/hr – 30% ITU Veh A 120 km/hr – 10% | ITU Ped B 3 km/hr |
| Spatial Channel Model | ITU with spatial correlation (Refer to Section ‎3.2.9 ***) | ITU with spatial correlation |
| Error Vector Magnitude (EVM) | 30 dB | 30 dB |