| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink Power Control Design - Considerations and Mechanism |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Dong-Cheol Kim Jin Sam Kawk Young-Hyoun Kwon Sungho Moon Wookbong Lee LG Electronics LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice : +82-31-450-1916 E-mail: dongcheol.kim@lge.com samji@lge.com WishWill@lge.com msungho@lge.com wbong@lge.com |
| Re: | IEEE 802.16m-08/024 "Call for Comments and Contributions on Project 802.16m SDD" Specific Topic: Power Control |  |
| Abstract | This contribution describes several consideration points in the uplink power control design and proposes a high-level concept for IEEE 802.16m uplink power control mechanism. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink Power Control Design - Considerations and Mechanism

Dong-Cheol Kim, Jin Sam Kwak, Young-Hyoun Kwon,

Sungho Moon, and Wookbong Lee

LG Electronics

# Introduction

In the reference system [3], the uplink power control (UL PC) consists of two different modes with closed- and open-loop operations as mandatory and optional features, respectively. The MSs shall then be operated by the legacy UL PC mode change between CL-PC and OL-PC according to their channel quality. However, the legacy power control mechanism in uplink is not efficient for enhancing the system throughput by considering the trade-off between the overall spectral efficiency and the cell edge performance. In this contribution, in order to design the uplink power control for IEEE 802.16m, we address some discussion points and propose a high-level UL PC mechanism by considering several key aspects and their impacts on the 802.16m system performance in terms of interference-level control, pathloss compensation, multi-user MIMO operation, control/data transmission, interference management including FFR operation, etc.

# Consideration Points on Uplink Power Control design

In this section, the generic description on the key aspects, considerations, and features for 802.16m uplink power control design including interference control extending MIMO operation, full/fractional pathloss compensation, the concept of separate power control between data and control channels, the combined OL- and CL-PC mechanism, and the FFR operation support, etc.

# Intra/Inter-Cell Power Control

In a multi-cell system using frequency reuse factor, however, if MSs are transmitting at maximum power level, then a significant amount of interference is generated at neighboring cells/sectors that limits both cell-edge throughput as well as system capacity. To some extent, therefore, the inter-cell interference is quite essential to be controlled additionally with the intra-cell power control method by adjusting the parameter sent from serving or non-serving BS in the 802.16m. Table I shows the simulation assumption in order to compare the system performance using the intra- or inter-cell power control method as shown in Table II. The details of the parameters are given in the Annex. As summarized in the Table II, in order to get the comparable cell-edge user throughput to the requirements in the 802.16m SRD [1], the inter-cell power control considering inter-cell interference is necessary without significant degradation of the system throughput.

Table I. intra-cell vs. inter-cell power control simulation assumptions

| Frequency reuse | 1 |
| --- | --- |
| Cell deployment | 3 sectors/cell and 19 cell wrap-around |
| The number of User | 10/sector |
| Power control | Open-loop power control + Offset_ICI |
| The number of strong interference | 10 |
| Channel model | ITU Ped B 3km/h |
| Permutation mode | Distributed mode |
| Site-to-Site distance | 1500m |

Table II. Intra vs. Inter cell power control

|  |  | Intra-cell | Inter-cell | Gain |
| --- | --- | --- | --- | --- |
| Sector throughput |  | 2.30 Mbps (0.623bps/Hz) | 2.272Mbps (0.614bps/Hz) | -1.59% |
| 5% tile user throughput |  | 64.21 kbps (0.0173 bps/Hz) | 75.6kbps (0.0204bps/Hz) | 17.74% |
| IoT | Mean | 5.51 | 5.09 | -7.62% |
|  | Std. | 1.53 | 0.95 | -37.90% |

As described in [3], on the other hand, the legacy power control mechanism has been based on the single user transmission per allocated resource units. In order to extend the UL PC operation into the multi-user support, it is required to consider the interference level control at the allocated resource units in the multi-user transmission. Table III shows the impact of inter-cell interference between single- and multi-user transmission on the system and cell-edge user throughput. In the case of CSM, we assumed that two users are transmitted in same allocated resource(s) at the same time. It is shown that the increased IoT level in the CSM causes the significant decrease of the cell-edge user throughput. This implies that the single-user UL PC should be modified and updated the multi-user operation, e.g., simply power scaling factor, in order to effectively control the inter-cell interference level.

Table III. Throughput/IoT Level Comparison Between SU-MIMO and MU-MIMO

|  |  | SIMO | CSM w/o power scaling | CSM with power scaling |
| --- | --- | --- | --- | --- |
| System throughput |  | 2.5752 Mbps (0.70 bps/Hz) | 2.24 Mbps (0.602 bps/Hz) | 2.9231 Mbps (0.79 bps/Hz) |
| 5% tile user throughput |  | 57.79 kbps (0.0156 bps/Hz) | 51.71 kbps (0.0140 bps/Hz) | 64.43kbps (0.0174 bps/Hz) |
| IoT level | Mean | 5.959 | 6.3417 | 5.9195 |
|  | Std. | 0.6836 | 0.7505 | 0.5401 |

# Full or fractional path loss compensation

According the IEEE 802.16m SRD [1], the enhanced cell-edge user throughput shall be supported in the 802.16m while maintaining the comparable/enhanced system throughput. In order to maximize the spectral efficiency even in the cell-edge users, compared with full pathloss compensation in the reference system, the fractional pathloss compensation is useful to support the affordable IoT level as well as to increase the cell-edge user throughput. Figure 1 shows the CDFs of the average SINR distribution, IoT level, and throughput with various pathloss compensation factors. Table IV summarizes the average system throughput and cell-edge user throughput with full (α=1) and fractional (α=0.8) pathloss compensation. In the simulation results, based on the 802.16m evaluation methodology [2], the simple open-loop power control with target SINR and BS specific offset from BS in the frequency reuse-1 environment is assumed. As shown in the figure 1 and table IV, the fractional pathloss compensation is beneficial to decrease the IoT level of the neighboring cells and increase the cell-edge user throughput with comparable the overall system throughput.

(a) SINR Distribution

(b) IoT Level

(c) System Throughput

Figure 1. CDFs of received SINR distribution, IoT level, throughput with full/fractional pathloss compensation

Table IV. Throughput/IoT level comparison between fractional vs. full pathloss compensation

| Pathloss Compensation |  | Full (α=1) | Fractional (α=0.8) |
| --- | --- | --- | --- |
| System throughput |  | 0.674 bps/Hz | 0.589 bps/Hz |
| 5% tile user throughput |  | 6.776kbps (0.0018 bps/Hz) | 19.8kpbs (0.0053 bps/Hz) |
| IoT level | Mean | 15.4466 | 6.9727 |
|  | Std | 3.2831 | 1.5032 |

With FFR mode operation, however, the fractional factor for pathloss compensation may not be necessary for the cell-edge users due to the frequency reuse in the neighbor cells/sectors. For example, the FFR zone with frequency reuse factor of 3 can be allowed to use full pathloss compensation for maximizing the cell-edge user throughput. This implies that since the different FFR zones may suffer the inherently different level of inter-cell interference, using the common fractional factor could not be effective to increase the cell-edge user throughput with the interference management scheme. Therefore, it is desirable that the full fractional pathloss compensation is adopted in the uplink power control in the FFR operation and the fractional/full pathloss compensation is FFS in the 802.16m.

# Separate power control between control and data channels

In the uplink, MS should transmit control information via dedicated control channels such as CQI, ACK/NACK, PMI, and DL measurements for feedback information, etc. The reliability of the control channels directly affects the entire system performance and the adjustment of the uplink power level for control channels is important to meet the strictly required QoS target and coverage since the UL control data does not get the benefits of HARQ operation and link adaptation. Therefore, the uplink power control between data and control channels should be separately operated with different power correction parameters while considering the maximum allowable UL transmit power for transmitting the data and control at the same time. In addition, since data and control resource can be allocated in the different FFR zones, it is necessary to apply the UL power control with the different ICI control between data and control channels.

# Open-loop or Closed-loop power control

The open-loop power control can compensate the long-term channel variations in an effective way, for instance, without the history of the transmit power, but it typically suffers from errors in path-loss measurement and transmit power setting. This implies that significant power margins could be necessary to be applied in order to meet the strict QoS requirements in UL data and control transmission. Large power margins can be translated as the extra transmit power, which is detrimental to both terminal battery life as well as system performance in the form of generating excess other cell interference. However, the merit of OLPC is no need for explicit DL signaling.

On the other hand, a closed-loop power control mechanism can tightly control the received SINR, which implies much smaller power margins are necessary, with increased overhead. Table V summarizes the pros and cons between CL- and OL-power controls. In the 802.16m uplink power control, therefore, it is desirable to support the tight power control with the reasonable power margin and overhead, e.g., the uplink power control can be based on the OL-PC for compensating the long-term channel variations with partial closed-loop operation with small power margin and signaling overhead. The details are described in Section 3.

Table V. Pros and Cons of CL- and OL-Power Control Operation

|  | OL-PC | CL-PC |
| --- | --- | --- |
| Pros | ∙ No need for explicit DL signaling, ∙ Effective way without the history of the uplink transmit power | ∙ Tight control of received SINR ∙ Minimized transmit power for desired QoS |
| Cons | ∙ Necessity of significant power margins | ∙ Necessity of explicit DL signaling per MS ∙ Increased signal overhead |

# Power control with FFR mode

According to the IEEE 802.16m SRD [1], the fractional frequency reuse method for interference management should be considered. In general, there are a couple of FFR modes, i.e., hard or soft FFR with static, semi-static, and dynamic operation. The details on the interference management and FFR concept for the 802.16m are shown in [4] and reference therein. Figure 2 illustrates the simple FFR concept with different maximum PSDs in the separate zones. It is well-known that the hard FFR mode can effectively decrease the interference level in the frequency reuse zones, whereas the overall spectral efficiency may be decreased due to the inactive (un-utilized) region in the frequency reuse zone. On the other hand, the soft FFR mode can use/utilize the inactive region for enhancing the resource utilization and throughput, but the inactive region in the freq. reuse zone should be restricted with a tight control of the interference in order not to affect the active region in the FFR zone at the neighboring cells or vice versa. To this end, it may be necessary to control the transmission power level for users in the inactive region by limiting the maximum PSD per the allocated resource with a tight ICI control.

In addition, since the active or inactive FFR zones in the physical channels including data and control channels suffers the different ICI level and user multiplexing rate per basic resource unit, the transmission power level for these channels should be determined by using separate parameters in terms of the FFR zone-specific offsets or interference measurements, etc.

In order to get a high spectral efficiency and use the FFR scheme effectively, active and inactive FFR resource within FFR regions should be used the different power control methods. Especially, spectral efficiency of the entire system can be achieved by using the inactive part of FFR region effectively.

Figure 2. An illustration of the FFR concept with transmit power level.

# Summary

Table VI shows a summary of the key aspects in the uplink power control design and also addresses the design criteria/high-level concepts for the IEEE 802.16m uplink power control method. In the following section, the uplink power control scheme for the 802.16m is proposed by reflecting the overall concepts described in Table IV.

Table VI. The summary of the uplink power control design for IEEE 802.16m

|  | Reference System [3] | IEEE 802.16m | Notes |
| --- | --- | --- | --- |
| Path-loss Compensation | Full | Full (default) /Fractional (FFS) | • Further consideration with interference management methods |
| Data/Control Channels | Same | Unified formula with separate parameters | • Support of the different targets and operating points between data and control information |
| Open/Closed-loop Mechanism | CLPC(mandatory) OLPC(optional) | Combined with OL and CL | • Tight/effective power control with reduced power margin and overhead |
| IoT control | Not Support (Intra-cell PC) | Support | • Flexible/effective intra- and inter-cell interference control |
| FFR Mode Support | No consideration | Consideration | • Support of the various channel characteristics in FFR operations |

# Power Control Mechanism for IEEE 802.16m

In this section, we describe the high-level concept for the 802.16m uplink power control design. As addressed in section 2, the following aspects should be incorporated into the power control formula for the 802.16m to meet the target requirements in the 802.16m SRD [1]:

- Unified power control formula supporting various channel, traffic, interference environment

- Path-loss compensation: Full or/and fractional

- Combined with OL and CL for overhead reduction with effective power adjustment

- IoT control : Intra or inter cell interference control, multi user support

- Separate power control for data and control

- Efficient FFR mode support

The generic mechanism and parameters for the 802.16m power control can be embodied as follows.

# Details on the parameters related to the 802.16m uplink power control

According to the discussion on the 802.16m power control key aspects, the generic parameters can be incorporated into the unified power control formula as

PTx = α∙PL + NI + SINRTarget + ΔPowerAdjust + δPowerScaling + OffsetICI [dBm]     (1)

where

PTx : Transmit power level at MS for achieving a given SINRTarget with a desired MCS

α : Path-loss compensation factor with a range of 0 < α ≤ 1 (Default: α = 1)

PL : Downlink/Uplink pathloss estimate calculated at BS/MS

NI : Noise and interference level per subcarrier at BS

SINRTarget : Target SINR threshold for each MCS level

ΔPowerAdjust : User specific power correction value from BS

δPowerScaling : Power scaling factor according to the single- or multi-user transmission power level

OffsetICI : User-/cell-/group-specific [FFS] power adjustment value related to the inter-cell power

control

In (1), PTx is the user transmit power level per subcarrier/resource unit, i.e., power spectral density, which is a function of the user power class and the assigned transmission bandwidth. The total of user transmit power can not exceed the predefined user transmit power class in the 802.16m system. Basically, the power control scheme in (1) supports the transmit power control by compensating the full/fraction of the pathloss, i.e., 0 < α ≤ 1 for the flexibility in balancing spectral efficiency and cell edge performance as described in Section 2.2. Furthermore, the intra cell power control is defined by combining with open loop and closed loop operation. Additionally, it can easily extend to inter-cell power control using Offset_ICI, for controlling the inter-cell interference level. The noise and interference level measured at BS can be broadcasted to all users in corresponding serving cell for achieving the target SINR for a given MCS. Power correction parameter can be also used to adjust the transmit power level by compensating the estimation errors or inaccuracy in the fast channel varying, practical measurements, open-loop related components, etc.

# Signaling and procedure

In order to determine the uplink transmit power level at a certain MS, the parameters for the uplink power control should be obtained from the serving BS. The power control messages from the serving BS can be categorized into a couple of types, i.e., common and user-specific messages as shown in Figure 3. The common messages include the cell-specific or common values to be used by the MSs in the serving cell, e.g., the pathloss compensation α, noise and interference level NI, etc. The user-specific messages includes the dedicated parameters for the MS, e.g., target SINR, SINRTarget, power correction term ΔPowerAdjust, and power scaling factor δPowerScaling . OffsetICI is FFS.

Figure 3. Categorization of power control signaling: common and user-specific message types

# Conclusion

In this contribution, we have addressed the key aspects of the power control in the 802.16m and proposed the high-level concept and mechanism for the uplink power control design. The uplink transmit power level at an MS should be determined by considering the following features and functions for optimizing the intra- and inter-cell power control:

- Unified power control method combined with open- and closed-loop PC parameters

- Fractional pathloss compensation can be useful to reduce the IoT level from the neighboring cells as well as to increase the cell-edge user throughput without significantly sacrificing the overall system throughput.

- Simple inter-cell interference control by adding a parameter into the intra-cell power control operation

- Support of the IoT control by depending on the single- or multi-user transmission in the same resource units

- Applicable to the FFR modes for supporting the zone-specific interference and channel characteristic

- Efficient uplink power control for data and control transmission

As a result, the proposed uplink power control can be effective to achieve the 802.16m uplink target spectral efficiency by considering both system throughput and cell edge user performance. In addition, the proposed concept is generic enough to readily adopt new enhanced techniques and features for the 802.16m system.

# Reference

[1] IEEE 80216m-07/002r4, “IEEE 802.16m System Requirements”

[2] IEEE 80216m-08/004r1, “Project 802.16m Evaluation Methodology Document (EMD)”

[3] IEEEP802.16Rev2/D5, “DRAFT Standard for Local and metropolitan area networks-Part 16: Air Interface for Broadband Wireless Access Systems” June2008

[4] IEEE 80216m-08/788, “Interference Mitigation for IEEE 802.16m,” LG Electronics

Text Proposal for the 802.16m SDD

============================== Start of Proposed Text =================================

# x.y. Uplink Power Control

The 802.16m power control shall support the following aspects in the uplink.

- Uplink power control supporting open- and closed-loop operation with reduced overhead and effective power adjustment

- Path-loss compensation: Full or/and fractional

- IoT control : Intra- or inter-cell interference control, multi-user support

- Separate power control operation for data and control

- Effective FFR mode support

The generic mechanism and parameters for the 802.16m power control can be embodied as follows:

PTx = α∙PL + NI + SINRTarget + ΔPowerAdjust + δPowerScaling + OffsetICI [dBm]     (1)

where

PTx : Transmit power level at MS for achieving a given SINRTarget with a desired MCS

α : Path-loss compensation factor with a range of 0 < α ≤ 1 (Default: α = 1)

PL : Downlink/Uplink pathloss estimate calculated at BS/MS

NI : Noise and interference level per subcarrier at BS

SINRTarget : Target SINR threshold for each MCS level

ΔPowerAdjust : User specific power correction value from BS

δPowerScaling : Power scaling factor according to the single- or multi-user transmission power level

OffsetICI : User-/cell-/group-specific [FFS] power adjustment value related to the inter-cell power

control

PTx is the user transmit power level per subcarrier/resource unit, i.e., power spectral density, which is a function of the user power class and the assigned transmission bandwidth. The total of user transmit power can not exceed the predefined user transmit power class in the 802.16m system. Basically, the power control scheme in (1) supports the transmit power control by compensating the full/fraction of the pathloss, i.e., 0 < α ≤ 1 for the flexibility in balancing spectral efficiency and cell edge performance. Furthermore, the intra cell power control is defined by combining with open loop and closed loop operation. Additionally, it can easily extend to inter-cell power control using Offset_ICI, for controlling the inter-cell interference level. The noise and interference level measured at BS can be broadcasted to all users in corresponding serving cell for achieving the target SINR for a given MCS. Power correction parameter can be also used to adjust the transmit power level by compensating the estimation errors or inaccuracy in the fast channel varying, practical measurements, open-loop related components, etc.

=============================== End of Text Proposal ===============================

# ANNEX: Simulation Assumptions

| Topic | Baseline System Assumptions |
| --- | --- |
| Basic modulation | QPSK, 16QAM |
| Duplexing scheme | TDD |
| Subchannelization | PUSC |
| Resource Allocation Granularity | PUSC: 1 slot (1 slot = 1 subchannel x 3 OFDMA symbols) |
| Downlink pilot structure | Specific to subchannelization scheme PUSC |
| Multi-antenna Transmission format | SIMO (1x2) CSM |
| Receiver Structure | MRC for SIMO, MMSE for CSM |
| Data Channel coding | Convolutional Turbo Coding (CTC) |
| Control Channel Coding | CDMA Codes (PUSC 2 symbols) for Initial Ranging and Handover, CDMA Codes (PUSC 1 symbol) for Periodic Ranging and Bandwidth Request, CQICH (6 bits) |
| Scheduling | Proportional Fairness for full buffer data only |
| Link adaptation | QPSK(1/2) with repetition 1/2/4/6, QPSK(3/4), 16QAM(1/2), 16QAM(3/4) |
| Link to System Mapping | MMIB |
| H-ARQ | Chase Combining Synchronous, Nonadaptive, 1 frame ACK/NACK delay, Maximum 4 HARQ Retransmissions, initial target PER of 20% |
| Power Control | Simulation scenario specific |
| Interference Model | Frequency selective interference model for PUSC/AMC, no interference awareness at receiver |
| Frequency Reuse | 3 Sectors with Frequency Reuse of 1 |
| Control signaling | Initial Ranging, Periodic Ranging, Handover Ranging, Bandwidth Request, Fast Feedback/CQI Channel, Sounding |

Table 1 System-level simulation assumptions for the uplink

| Scenario/ Parameters | Baseline Configuration (Calibration & SRD) | Urban Macrocell |
| --- | --- | --- |
| Requirement | Mandatory | Mandatory |
| Site-to-Site distance | 1.5 km | 0.5 km 1.5 km |
| Carrier Frequency | 2.5 GHz | 2.5 GHz |
| Operating Bandwidth | 10 MHz for TDD | 10 MHz for TDD |
| BS Height | 32 m | 32 m |
| MS Tx Power | 23 dBm | 23 dBm |
| MS Height | 1.5 m | 1.5 m |
| Penetration Loss | 10 dB | 10 dB |
| Path Loss Model | Loss (dB) = 130.62+37.6log10(R) (R in km) | Loss (dB) = 35.2+35log10(R)+ 26log10(f[GHz]/2) (R in m) |
| Lognormal Shadowing Standard Deviation | 8 dB | 8 dB |
| Correlation distance for shadowing | 50m | TBD [50m] |
| Mobility | 0-120 km/hr | 0-120 kmph |
| Channel Mix | ITU Ped B * 3 km/hr – 60% ITU Veh A 30 km/hr – 30% ITU Veh A 120 km/hr – 10% | Mix or specific |
| Spatial Channel Model | ITU with spatial correlation a) Uncorrelated b) Correlated (BS Correlation Coefficient = 0.5) | [Correlation based implementation 3.2.7] |

Table 2 Test Scenarios