| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Non-orthogonal Superposition for UL MIMO |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Joerg Schaepperle Andreas Rueegg Juergen Otterbach Bozo Cesar Alcatel-Lucent | Voice: +49 711 821 32266 E-mail: Joerg.Schaepperle@alcatel-lucent.de Andreas.Rueegg@alcatel-lucent.de Juergen.Otterbach@alcatel-lucent.de Bozo.Cesar@alcatel-lucent.de *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Comments and Contributions (IEEE 802.16m-08/024) on Project 802.16m, System Description Document (SDD) Specific topic: Uplink MIMO schemes |  |
| Abstract | Non-orthogonal superposition in uplink for single and multiple antenna modes |  |
| Purpose | To be presented and discussed in TGm for adoption of the specific text in the SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Non-orthogonal Superposition for UL MIMO

Joerg Schaepperle, Andreas Rueegg, Juergen Otterbach, Bozo Cesar

Alcatel-Lucent

# Introduction

According to the System Requirements Document (SRD) [1], IEEE 802.16m shall support multi-user MIMO techniques.

Superposition of signals (layers) that are neither orthogonal in time or frequency nor in space and can only be separated by joint detection or successive interference cancellation is a multi-user technique for single and multiple antenna systems that can enhance the aggregate sector throughput significantly. It allows using more streams than the number of receive antennas at the BS. Comparable aggregate throughput cannot be achieved by orthogonal signaling under fairness constraints. Only long-term channel statistics are required at the transmitter.

Figure 1 shows the CCDF for the spectral efficiency within a sector per time-frequency radio resource with and without non-orthogonal superposition under the following assumptions:

- Max. path loss difference in the cell: 60dB.

- Cell edge SNR: 0dB

- Path loss exponent: 3.5.

- Lowest MCS: QPSK 1/2 with repetition coding 4.

- 30 active users in the cell

The spectral efficiencies are:

- Cell spectral efficiency with SU access: 0.766 bps/Hz

- Cell spectral efficiency with MU access: 1.580 bps/Hz

- Cell spectral efficiency gain by non-orthogonal superposition: 106%

Figure 1: Spectral efficiency per time-frequency resource with and without non-orthogonal superposition

To support non-orthogonal signaling, IEEE 802.16m must be able to

- assign multiple terminals/basic CIDs (layers) to a common time-frequency resource (the number of terminals may be higher than the number of transmit antennas at the BS)

- signal in the DL map for each layer in each time-frequency allocation:

  - the modultion and coding scheme (MCS) or burst profile

  - the terminal address/basic CID

  - the transmit power offsets relative to an uplink signal without superposition

- adjust different receive power per data tone at the BS for different layers

- cause a terminal to use different transmit power per data tone in the uplink for bursts with and without superposition even if the modulation and coding scheme (MCS) is the same.

- cause a terminal to adjust pilot boosting for the case that the same pilot patterns are used in different layers.

- It may allow for hopping pattern assignment to the supported layers in conjunction with restrictions imposed by Fractional Frequency Reuse (FFR).

# Proposed Text for the SDD:

-------------------------------------------------------------Start of the Text-------------------------------------------------------

10 Medium Access Control Sub-Layer

10.x UL MIMO

IEEE 802.16m supports non-orthogonal superposition of uplink signals (layers) from different terminals with the same or different receive power levels in the uplink, where the number of layers may be higher than the number of receive antennas at the base station. It provides mechanisms to control the transmit power of a MS differently for different layers and allows the transmit power per data tone of an uplink transmit burst using superposition to be different from other uplink bursts without superposition.

The IEEE 802.16m MAC allows the allocation of time frequency resources in uplink to be used for non-orthogonal superposition of signals from different MSs in single and multiple antenna modes.

It provides control structures for DL signaling of transmit power offsets to be used by the terminals for non-orthogonal superposition in UL.

10.x Power Control

10.x.x UL Power Control

IEEE 802.16m provides a mechanism and control structures for uplink power control on a per terminal basis to provide additional transmit power offsets required for adjusting the transmit powers for different layers in non-orthogonal superposition mode.

11 Physical Layer

11.6 DL Control Structure

...

11.6.2.3.1.2 User-specific control information

The user specific control information includes additional user specific transmit power offsets to be used by the terminals only when transmitting in UL allocations reserved for non-orthogonal superposition. It may also include frequency hopping pattern configurations.

11.x UL MIMO

IEEE 802.16m MSs are able to adjust their transmit power on time-frequency resources reserved for non-orthogonal superposition in single and multiple antenna configurations according to transmit power offsets signaled by the BS specifically for that purpose.

--------------------------------------------------------------End of the Text-------------------------------------------------------

# References

[1] IEEE 802.16m System Requirements Document (SRD), IEEE 802.16m-07/002r4