| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on the DL MIMO SDD Text |  |
| Date Submitted | 2008-07-11 |  |
| Source(s) | Yinggang Du, Hongjie Si, Qitao Song, Martin Liu, Mingyang Sun, Xin Chang, Yi Lu, Yinwei Zhao, Qi Li, Yang Tang, Young Hoon Kwon, Jianmin Lu Huawei | Voice: + 86 755 28970190 E-mail: duyinggang@huawei.com |
| Re: | IEEE C802.16m-08/657r2 “Final Draft: SDD Text on Downlink MIMO Schemes” |  |
| Abstract | Text proposal for the SDD on Downlink MIMO. |  |
| Purpose | To incorporate the proposals into the DL MIMO RG report. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on the DL MIMO SDD Text

Yinggang Du, Hongjie Si, Qitao Song, Martin Liu, Mingyang Sun, Xin Chang, Yi Lu

Yinwei Zhao, Qi Li, Yang Tang, Young Hoon Kwon, Jianmin Lu

Huawei

# Introduction

The final draft of SDD text on DL MIMO was released by the DL MIMO RG group [1], 4 Tx antennas open loop SU-MIMO schemes were proposed to support rate 1 to rate 4 transmission. However, the STC encoding is only adopted for rate 1 with 4 Tx antennas, to achieve more diversity gain and improve the downlink performance, we proposed to adopt the STC encoding for rate 2 and rate 3 with 4 Tx antennas as well as rate 1 with 4 Tx antennas.

# Text Proposal 1 to C80216m-08/657r2

------------------------------------Start text proposal------------------------------------

[Adopt the following modification on Table 1 in the section 11.x.2.1.1 of C80216m-08/657r2]

# Open-loop SU-MIMO

Table 1. Matrix dimensions for open-loop SU-MIMO modes

| NT | Rate | M | NF |
| --- | --- | --- | --- |
| 2 | 1 | 2 | 2 |
| 4 | 1 | 2 | 2 |
| 8 | 1 | 2 | 2 |
| 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 1 |
| 4 | 2 | 4 | 2 |
| 8 | 2 | 2 | 1 |
| 4 | 3 | 3 | 1 |
| 8 | 3 | 3 | 1 |
| 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 1 |

[Adopt the following text modification in the section 11.x.2.1.1.2 of C80216m-08/657r2]

# Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM

  - 4Tx rate-2: rate 2 SM with precoding

  - 4Tx rate-2: rate 2 D-STTD

  - 8Tx rate-2: rate 2 SM with precoding

[Insert the following text at line 9 in the section 11.x.2.1.1.2 of C80216m-08/657r2]

For 4Tx antennas, rate 2 D-STTD, the input to the MIMO encoder is represented as a 4  1 vector

x=s1s2s3s4.

The output of the MIMO encoder is a 4  2 matrix zi(i=1,2,⋯,6)

z1=s1−s2s2s1s3−s4s4s3,z2=s1−s2s2s1s3s4s4−s3,z3=s1−s3s2−s4s3s1s4s2,z4=s1−s3s2s4s3s1s4−s2,z5=s1−s4s2−s3s3s2s4s1,z6=s1−s4s2s3s3−s2s4s1.

The output of the precoder is a 4  2 matrix

y=zi(i=1,2,⋯,6),

------------------------------------End text proposal------------------------------------

# Text Proposal 2 to C80216m-08/657r2

------------------------------------Start text proposal------------------------------------

[Adopt the following modification on Table 1 in the section 11.x.2.1.1 of C80216m-08/657r2]

# Open-loop SU-MIMO

Table 2. Matrix dimensions for open-loop SU-MIMO modes

| NT | Rate | M | NF |
| --- | --- | --- | --- |
| 2 | 1 | 2 | 2 |
| 4 | 1 | 2 | 2 |
| 8 | 1 | 2 | 2 |
| 2 | 2 | 2 | 1 |
| 4 | 2 | 2 | 1 |
| 8 | 2 | 2 | 1 |
| 4 | 3 | 3 | 1 |
| 4 | 3 | 12 | 4 |
| 8 | 3 | 3 | 1 |
| 4 | 4 | 4 | 1 |
| 8 | 4 | 4 | 1 |

[Adopt the following text modification in the section 11.x.2.1.1.2 of C80216m-08/657r2]

# Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-user MIMO:

- Rate-2 spatial multiplexing modes:

  - 2Tx rate-2: rate 2 SM

  - 4Tx rate-2: rate 2 SM with precoding

  - 8Tx rate-2: rate 2 SM with precoding

- Rate-3 spatial multiplexing modes:

  - 4Tx rate-3: rate 3 SM with precoding

  - 4Tx rate-3: rate 3 hybrid SM and STBC

  - 8Tx rate-3: rate 3 SM with precoding

[Insert the following text at line 22 in the section 11.x.2.1.1.2 of C80216m-08/657r2]

For 4Tx antennas, rate 3 hybrid SM and STBC, the input to the MIMO encoder is represented as a 12  1 vector

x=s1s2s3s4s5s6s7s8s9s10s11s12T.

The output of the MIMO encoder is a 4  4 matrix zi(i=1,2,3)

z1=s1−s2s7s11s2s1s8s12s3s5s9−s10s4s6s10s9,z2=s1−s3s7s11s2s5s8−s10s3s1s9s12s4s6s10s8,z3=s1−s4s7s11s2s5s8−s9s3s6s9s8s4s1s10s12

These matrices are permutated one by one to achieve the diversity both in spatial and time (frequency) domain. Moreover, the computational complexity of the SIC (successive interference cancellation) receiver for the above matrix Cs can be similar to that of the SIC receiver for 3×3 MIMO.

The output of the precoder is a 4  4 matrix

y=zi(i=1,2,3)

------------------------------------End text proposal------------------------------------